3GPP TSG-SA WG6 Meeting #48-e	S6-220763

e-meeting, 5th – 14th April 2022	(revision of S6-220615)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0052 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements related to 5G ProSe Layer-3 relaying via N3IWF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5GProSe |  |  |  |  |  | Date: |  |  | 2022-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The introduced CR adds the requirements to be considered in order to relay traffic of a remote MC service UE via a 5G ProSe Layer-3 UE-to-network relay via the support of N3IWF, as described in 3GPP TS 23.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Identify the requirements to enable 5G ProSe Layer-3 UE-to-network relay via the support of N3IWF, as described in 3GPP TS 23.304. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe Layer-3 UE-to-network relay via the support of N3IWF is not included in 3GPP TS 23.289. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8, 4.8.1, 4.8.2, and 7.6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 4.8 Use of 5G ProSe UE-to-network relay

### 4.8.1 General

The MC service shall support the capabilities for 5G ProSe UE-to-network relay. For this matter, 5G ProSe Layer-2 and 5G ProSe Layer-3 UE-to-network relaying techniques can be utilized, as described in 3GPP TS 23.304 [17].  The 5G ProSe Layer-3 UE-to-Network relaying technique may be done with or without the support of N3IWF, as described in 3GPP TS 23.304 [17].

A 5G ProSe UE-to-network relay supporting MC service UE provides means of connectivity and relaying of MC traffic to remote MC service UE(s). For this matter, the 5G ProSe UE-to-network Relay Discovery service allows the MC service remote UE to discover a potential UE-to-network relay UE supporting MC service in its proximity as described in 3GPP TS 23.304 [17]. Upon its discovery, the 5G ProSe Direct UE-to-network Relay Communication functionality is utilized to achieve communication to provide the MC service remote UE access to 5GS, and relay MC traffic via the UE-to-network relay UE over the NR PC5 reference point.

### 4.8.2 5G ProSe UE-to-network relay service requirements

In order to enable 5G ProSe UE-to-network relaying capabilities – whether based on Layer-3 or Layer-2 UE-to-network relaying techniques, the MC service system provides the appropriate parameters and configurations to the MC service UE(s).

As defined in 3GPP TS 32.304 [17]3GPP TS 23.304 [17], among these parameters are: Relay Service Code(s) (RSCs) which can be associated to a certain MC service group, User Info, ProSe Layer-2 Group ID and ProSe Group IP multicast address. Moreover, the MC service group ID is resolved to the ProSe Layer-2 Group ID and ProSe Group IP multicast address, which are utilized within the 5G ProSe Relay Discovery and 5G ProSe Direct Communication procedures, as described in 3GPP TS 23.304 [17]. Furthermore, the RSCs are utilized to restrict the necessary UE-to-network relay service and related procedures within members of a certain MC service group.

Moreover, in case of 5G ProSe Layer-3 UE-to-network relay with the support of N3IWF, the UE-to-network relay is provisioned with policies and parameters, among others suitable RSC(s), in order to support N3IWF access, as defined in 3GPP TS 23.304 [17].

# * * * * Second change * * * *

#### 7.6.3.1	5G ProSe UE-to-network relay service authorization

The MC service shall support the capability for 5G ProSe UE-to-network relay to restrict the relayed group communication on a per group basis by using the relay service codes corresponding to the group.

5G ProSe (as specified in 3GPP TS 23.304 [17]) supports layer 2 UE-to-network relay and layer 3 UE-to-network relay. The procedure defined in clause 10.5 of 3GPP TS 23.280  [3] applies with the following differences:

-	5G ProSe UE-to-network relay is provisioned that each relay service code is offering layer 2 or layer 3 UE-to-Network Relay service.

-	5G ProSe remote UE is provisioned that each relay service code is offering layer 2 or layer 3 UE-to-Network Relay service.

-	5G ProSe Layer-3 UE-to-network relay is provisioned with suitable relay service codes to support access to N3IWF for the case of 5G ProSe Layer-3 UE-to-network relay with the support of N3IWF.

# * * * * End of changes * * * *