3GPP TSG-SA WG6 Meeting #47-e	S6-220831

e-meeting, 5th – 14nd April 2022	(revision of S6-220572)

Source:	China Mobile

Title:	update Solution #8

Spec:	3GPP TR 23.700-98

Agenda item:	9.8

Document for:	Approval

Contact:	Xiaohui Shi, shixiaohui@chinamobile.com

1. Introduction

This paper proposes update for solution#8.

2. Reason for Change

Supplement the API exposing and invoking methods of EAS

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98.

* * * First Change * * * *

Solution #8: EAS Service API enablement using CAPIF

7.8.1	Architecture enhancements

None.

7.8.2	Solution description

7.8.2.1	General

This solution addresses the Key issue #2: Enablement of Service APIs exposed by EAS as specified in the clause 4.2 by supporting for an EAS to expose its Service APIs towards the other EASs.

As specified in TS 23.558 (Rel-17), the Edge Enabler Layer exposes Service APIs towards the EASs. The exposed Service APIs include the capabilities provided by EES (clause 8.6 of TS 23.558); the capabilities provided by the 3GPP core network (clause 8.7 of TS 23.558); and SEAL service APIs (clause A.4 of TS 23.558).

In this solution, the Edge Enabler Layer also supports for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs in order to fulfil the architectural requirements specified in the clause 5.x. This solution exploits CAPIF specified in 3GPP TS 23.222 [16] to support publication/discovery, and change subscription of EAS Service APIs as studied in Sol#15 of TR 23.758 (Rel-17) with the following architectural assumptions within the CAPIF architecture:

-	An EAS may act as an API provider by implementing API provider domain functions (i.e., API exposing function, API publishing function, and API management function)

-	An EAS may act as an API invoker

-	An EES may act as a CAPIF provider by implementing CAPIF core function (CCF)

Based on the architectural assumption above, the essential operations regarding EAS Service APIs complying with CAPIF are as follows:

-	An EAS (acting as API provider) may publish its EAS Service APIs to EES (acting as CAPIF provider)

-	An EAS (acting as API invoker) may discover EAS Service APIs from EES (acting as CAPIF provider)

-	An EAS (acting as API invoker) may subscribe to be notified of dynamic information or availability of EAS Service APIs from EES (acting as CAPIF provider)

-	An EAS (acting as API invoker) can discover and invoke service APIs registered and published on other EESs

-	As EAS1(acting as API invoker) discovers EAS2(acting as API provider) from EES, it can invoke the Service APIs directly from EAS2

* * * Next Change * * * *