3GPP TSG-SA WG6 Meeting #48-e	S6-220671Rev2

e-meeting, 5th – 14th April 2022	(revision of S6-22xxxx)

Source:	CATT

Title:	Pseudo-CR on solution for supporting Location QoS

Spec:	3GPP TR 23.700-96 v0.4.0

Agenda item:	9.7

Document for:	Approval

Contact:	Chunshan Xiong, chunshan.xiong@cictmobile.com

1. Introduction

This contribution proposes solution for key issue #2: Support of LCS QoS.

2. Reason for Change

Depending on requested location QoS, the Fused Location Server will select different access type and postioning methods and different location delivery message to retrieve the location information of the target UE.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-96 v0.4.0.

* * * First Change * * * *

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[4]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[7]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[8]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP; Stage 1".

[9]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[10]	Open Mobile Alliance, OMA AD SUPL: "Secure User Plane Location Architecture", (http://www.openmobilealliance.org).

[11]	Open Mobile Alliance, OMA AD MLS: "Mobile Location Service Architecture", (http://www.openmobilealliance.org).

[12]	3GPP TS 23.502: " Procedures for the 5G System (5GS); Stage 2".

[13]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[x]	3GPP TS 29.572: "Location Management Services; Stage 3".

* * * Second Change * * * *

## 7. x	Solution #x: Location QoS based location sources and positioning methods Selection.

### 7. x.1	Solution description

This solution addresses key issue #2: Support of LCS QoS.

The Fused Location Fuction firstly needs to produce the fused location data from multiple sources based on the the rquested location QoS (e.g. the requirements of the positionting accuracy, reliability and latency). Based on the requested location QoS, the FLF needs to select one or more access types, one or more location methods (as described in TS 29.572 [x] ) and related CP/UP(SUPL) methods based on the requested location QoS (not exhausted):

- 2G/3G/4G/5G/NR satellite access

- Non-3GPP access connected to 5GC

- GNSS (e.g. GPS, Galilieo, BeiDou etc.)

- Barometric Pressure

- WLAN

- Bluetooth

- Terrestrial Beacon System (TBS) positioning based on MBS signals

- Motion Sensor

- RFID

- Radio finger-print

- Celll ID

- ECID

- OTDOA

- DL_TDOA

- DL_AOD

- Multi-RTT

- NR_ECID

- UL_TDOA

- UL_AOA

#### 7. X.1.1	Procedure of location QoS based location sources and positioning methods selection

Figure 7.x.1.1-1: location sources and positioning methods selection

- The application specific server sends a location request to the Fused Location Server to request the location information of the target Fused Location Client (UE Identity, location QoS). The location QoS can includes the location accuracy, reliability and latency etc. as described in clause 4.1b of TS23.273[4].

- The Fused Location Server queries the UE location context (e.g. in the internal database) with the location QoS to retrieve the available the access type, positioning methods as described in TS 29.572 [x] for the target Fused Location Client.

- The Fused Location Server selects the available access type (i.e. different location sources) and position methods to gets the UE location informations from these available access types.

- The Fused Location Server fuses the UE location information from different sources to get a fused UE location that meets the location QoS requirements.

- The FLS provides the fused UE location that meets the location QoS requirements to the application specific server.

### 7. X.2	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

This solution is based on the architecture proposed in Solution #1. There is no similar precedure defined in the TS 23.434[13]. Different location source can provide location information with difference location QoS, the FLS takes the different advantages of these difference location sources and decides to select which location sources based on the required location QoS and fuses the location information from the selected location sources to produce the final location information to meet the location QoS.