3GPP TSG-SA WG6 Meeting #48	S6-220547 rev1

e-meeting, 5th – 14th April 2022	(revision of S6-22xxxx)

Source:	Convida Wireless LLC

Title:	AC Association Aware EEC Registration

Spec:	3GPP TR 23.700-98 v.0.5.1

Agenda item:	9.8

Document for:	Approval

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

1. Introduction

This pCR provides a solution in support KI #17 for use cases requiring multiple ACs to use services from a single common EAS (e.g., to meet strict latency requirements).

The solution defines enhancements to the EEC registration procedure that enables ACs/EECs of different UEs to share AC association information with an EES. This AC association information can then be used by the EES during the processing of other procedures (e.g., EAS discovery, service continuity, etc.) such that awareness of the AC associations can be factored into these procedures.

AC association information can be provided as part of the AC Profiles.

Rev 1 merges in content from S6-220548. It also clarifies the relationships between registration and discovery (covered in the initial tdocs versions) and other procedures used to enable EECs to receive edge services when AC associations are identified, and the EAS services for the associated ACs are optimized on a common EAS is selected for the associated ACs.

2. Reason for Change

Address Key Issue #17 discovery of a common EAS

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98

* * * First Change * * * *

## 7.X	Solution #X: Enabling AC Association Aware services by selecting common EASs

### 7.X.1	Architecture enhancements

None.

### 7.X.2	Solution description

#### 7.X.2.1	General

This solution addresses key issue #17 on discovery of a common EAS in clause 4.17.

The solution descrfibnes enhancements to several edge  the EEC registration procedures that enables ACs/EECs of different UEs to share AC association information with the EELan EES. This AC association is formed such that information can then be used by the services forEES during the associated ACs are optimized by selecting aprocessing of other procedures (e.g., common EAS.

NOTE: In  aware discovery, common EAS aware service continuity, etc.) such that awareness of the following descriptions within this solution, the term “Associated ACs” is used to denote a set of ACs on different UEs for which the association described above has been configured via an AC association Profile . How the AC association Profile is determined is out of scope of the current specificationassociations can be factored into these procedures.

- 7.X.2.2	New Information Elements

The AC association Profile is proposed as a new IE s in TS 23.558:

Table 8.2.x: AC association Profile

| Information element | Status | Description |
| --- | --- | --- |
| Association ID | M | Identity of the Association |
| AC association type | M | Choice of multi-use or multi-session |
| List of associated UEs characteristics | O | Information for determining the UEs with associated ACs |
| > Associated UE group ID | O | Group ID pre-provisioned (i.e. as External Group ID) to the UEs with ACs in the associated group. If this IE is not present, |
| > Associated UEs service area | O | Service area for determining other UEs in the association. The UE location is described in clause 7.3.2. The optional additional EAS selection criteria describe criteria for the EAS selection (e.g., “same latency for all” or “lowest latency for the own UE location”). |
| List of associated ACs characteristics (NOTE 1) | M | Information for determining the ACs on the associated UEs |
| > common ACID | O | If provided, all ACs in the association have the same AC ID as indicated |
| > common ACs Type | O | If provided, all ACs in the association have the same AC Type as indicated |
| > common ACs Schedule | O | If provided, all ACs in the association have the same AC Schedule as indicated |
| List of common EAS characteristics (NOTE 1) | M | Information for determining the common EAS |
| >Common EAS discovery filter (NOTE 2) | O | Describes the characteristics of a common EASs using the EAS discovery filter described in Table 8.5.3.2-2. |
| > Common EAS aggregate Service KPIs | O | Service characteristics provided by the common EAS, detailed in Table 8.2.5-1. The characteristics are described to meet the requirements for the AC association. |
| NOTE 1: Either "List of associated AC characteristics" or "List of common EAS characteristics" shall be present. NOTE 2: Only the "List of EAS characteristics" IE from Table 8.5.3.2-2 is included. The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

- 7.x.2.3	Enhancements to Common Information Elements and Information Flows

The following enhancements (highlighted in yellow) are proposed to tables in TS 23.558:

Table 8.4.2.3.2-1: EEC registration request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| UE Identifier | O | The identifier of the hosting UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| AC Profile(s) (NOTE 1) | O | Profiles of ACs for which the EEC provides edge enabling services. AC Profiles are further described in Table 8.2.2-1. |
| AC association profile(s) (NOTE 1) | O | Describes an association between ACs and an association between ACs and a common EAS. |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. |
| Proposed expiration time | O | Proposed expiration time for the registration. |
| EEC context ID (NOTE 2) | O | Identifier of the EEC context obtained from a previous registration. |
| Source EESID (NOTE 2) | O | Identifier of the EES that provided EEC context ID. |
| Source EES Endpoint (NOTE 2) | O | The endpoint address (e.g. URI, IP address) of the EES that provided EEC context ID. |
| NOTE 1: Either "AC Profile" or "AC association profile" shall be present for an AC to be registered. NOTE 2: This IE shall not be present when EEC registration is performed as part of ACR. |  |  |

Table 8.5.3.2-2: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of AC characteristics (NOTE 1) | O | Describes the ACs for which a matching EAS is needed. |
| > AC profile (NOTE 2) | M | AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1. |
| List of EAS characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required EASs. |
| > EASID | O | Identifier of the required EAS. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g. V2X) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g. time windows) |
| > EAS Geographical Service Area | O | Location(s) (e.g. geographical area, route) where the EAS service should be available. |
| > EAS Topological Service Area | O | Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > Service permission level | O | Required level of service permissions e.g. trial, gold-class |
| > Service feature(s) | O | Required service features e.g. single vs. multi-player gaming service |
| AC association profile | O | Describes an association between ACs and an association between ACs and a common EAS. |
| NOTE 1: Either "List of AC characteristics" ,"List of EAS characteristics" or AC association profile shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

- 7.X.2.4	Determining common EES Procedure

Editor’s Note. The procedure for determining common EES  is FFS.

#### 7.X.2.5	Registration at common EES Procedure

In this procedure, an EEC sends EEC registration request including AC association Profile. The EES stores the AC association Profile information as part of the EEC context such that it can be used by the EES during the processing of other procedures (e.g., common EAS aware discovery, common EAS aware service continuity, etc.).

Pre-conditions:

- AC association Profile information has been provided at the  EEC.

Figure 7x.2.5-1 UE registration using AC association Profile

- An EEC sends EEC registration request(s) to the EES. The request includes  an AC association Profile indicating that the use of a common EAS is required.

- Upon receiving the registration request(s) from the  EEC the EES processes the request(s) per the procedure defined in clause 8.4.2 of 3GPP TS 23.558 [2] v17.0.0. As part of the EEC context the AC association Profile is also included and maintained by the EES such that it can be used by the EES during the processing of other procedures (e.g., EAS discovery, service continuity, etc.).

- If the processing of the request was successful, the EES responds to the EECwith an EEC registration response.

- 7.X.2.6	Discovery of common EAS Procedure

In this procedure, the EEC sends an EAS discovery request to an EES including AC association Profile. Based the AC association Profile, the EES determines an  EAS discovery result that includes a common EAS for the AC association.

Pre-conditions:

1.An AC association Profile  has been provided to the EEC.

Figure 7x.2.6-1 Common EAS discovery AC association Profile

- The  EEC sends an EAS discovery request to the EES. The EAS discovery request includes an AC association Profiles. Included in the AC association Profile  is information regarding an association between AC(s) on this UE  and other ACs  indicating that the use of a common EAS is required.

- Upon receiving the request, the EES determines whether a common EAS is available to provide services to the associated ACs that meet the criteria specified in the AC association Profile

- If the processing of the request was successful, the EES responds to the EEC with an EAS discovery response which includes information about the discovered common EAS able to provide services to the associated ACs.

If the EES is unable to determine a common EAS for the associated ACs, the EES responds with an appropriate failure cause.

- 7.X.2.7	ACR with common EAS Procedure

Editor’s Note. The ACR with common EAS procedure is FFS.

7.X.3	Solution evaluation

This solution addresses KI#17. The solution defines enhancements to the EEC registration procedure that enables ACs/EECs of different UEs to share AC association Profile information with an EES. This AC association Profile information can then be used by the EES during the processing of other procedures (e.g., common EAS aware discovery, common EAS aware service continuity, etc.) such that awareness of the AC associations can be factored into these procedures