3GPP TSG-SA WG6 Meeting #49-bis-e	S6-221631

e-meeting, 22nd June – 1st July 2022

Title:	LS on FS_eEDGEAPP Solution for Support of NAT deployed within the edge data network

Response to:

Release:	Rel-18

Work Item:	FS_eEDGEAPP

Source:	SA6

To:	SA2, SA3

Cc:

Contact Person:

Name:	Shaowen Zheng

E-mail Address:	zhengshaowen at chinamobile dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

As part of SA6’s Rel-18 FS_eEDGEAPP study, SA6 has agreed solutions for KI#16 of Supporting of NAT deployed within the edge data network.

In Sol#23, UE provides either the CN network assigned IP address (i.e., the private IP address of the UE) or its UE ID (if it already has one) to the EES to obtain the UE identifier (Edge UE ID) from the EES. If the UE ID is not provided, EES uses the private IP address of the UE provided by the EEC to invoke the CN capability to translate UE's IP address to its UE ID (e.g., UE's External Identifier), as defined in TS23.502 clause 4.15.10.

In Sol#X, a new solution introduced in this meeting, the EES can get the UE public IP address (port number if needed) and IP domain from EAS. And then the EES(AF) is supposed to invoke the 5GS service to get the UE private IP address. After that, the EES uses the private IP address of the UE provided by the EEC to invoke the CN capability to translate UE's IP address to its UE ID (e.g., UE's External Identifier), as defined in TS23.502 clause 4.15.10.

There are remaining issues for Sol#23 and Sol#x:

For Sol#23:

1) If the UE has overlapped private IP address(es), the cooperation between SA6 and SA2 may be needed to solve this problem.

2) The validity for UE providing UE ID and/or UE private IP address should be checked with SA3.

For Sol#x:

1) Whether the SA2 can support the translation between UE public IP address and UE private IP address is FFS.

SA6 would like to invite SA2 to check the first issue(es) for Sol#23 and for Sol#x and comment on whether the above issues can be solved by SA2 in Rel-18.

SA6 would like to invite SA3 to check the second issue for Sol#23.

2. Actions:

To SA3

ACTION:

SA6 asks SA2 to consider the above information and invites SA2 to provide comments on the first issue(es) for Sol#23 and Sol#x.

SA6 asks SA3 to consider the above information and invites SA3 to provide comments on the second issue for Sol#23.

3. Date of Next TSG SA WG2 Meetings:

SA6 #50-e	August 22nd - 31st 	e-meeting

SA6 #51-e	October 10th - 19th 	e-meeting