3GPP TSG-SA WG6 Meeting #49-bis-e	S6-221969

e-meeting, 22nd June – 1st July 2022	(revision of S6-221884_Rev1, S6-221685_Rev1)

Source:	Samsung

Title:	Pseudo-CR on consistency

Spec:	3GPP TR 23.700-98 V1.0.1

Agenda item:	9.8

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samusung.com

1. Introduction

The contribution proposes to move text to the right clauses in a concistent manner.

2. Reason for Change

To improve the readablity, the changes are to move the clauses for consistency sake. All the green text is moved from one clause to another without any chages. Red text is some updates as per the template.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98 v1.0.1.

* * * First Change * * * *

## 5.4	Alignment of EDGEAPP and ETSI MEC

### 5.4.1	General

This clause specifies the architecture requirements for alignment of EDGEAPP and ETSI MEC to address the key issue #4.

### 5.4.2	Requirements

The following requirements will serve as the guiding principles for the alignment of EDGEAPP and ETSI MEC architectures.

1.	The scope of 3GPP alignment efforts between EDGEAPP and ETSI MEC shall be limited to architecture enhancements that apply only to EDGEAPP.

NOTE: 3GPP will liaise with ETSI ISG MEC for architecture recommendation (if any).

2.	The architecture enhancements to support alignment shall ensure backwards compatibility with the existing EDGEAPP architecture.

3.	The architecture enhancements shall not consider to align features exclusive to the each of the EDGEAPP and ETSI MEC architectures, i.e. the architecture enhancements shall only focus on the overlapping aspects between the EDGEAPP and ETSI MEC architectures.  The alignment aspects in the present release of this specification includes the following:

a.	alignment of EAS profile (EDGEAPP) and appInfo (ETSI MEC),

b.	alignment of EDGE-3/Mp1 reference points

c.	alignment of EDGE-9/Mp3 reference points

d.	usage of CAPIF between the two architectures

NOTE: The term alignment does not imply that the reference points will be exact equivalents.

Editor's note:	Any additional aspects to be considered for alignment is FFS.

4.	The architecture enhancements shall ensure that EDGEAPP architecture can remain as a standalone or a complete system i.e. EDGEAPP can be deployed independent of the ETSI MEC architecture.

5.	The architecture enhancements shall analyse impacts to other working groups e.g. SA2, SA3 and SA5.

* * * Next Change * * * *

## 6.7	Option #7: Architecture for Common EAS selection with central binding server

### 6.7.1	Architecture enhancements

Figure 6.7.1-1: EDGEAPP architecture enhanced with binding server

The EDGE-X reference point is introduced to support EES interaction with binding server. The EES can store, update and remove the binding information via EDGE-X reference point.

The binding server is deployed in a central cloud and it is a single server maintaining binding information.

Editor's note:	Whether binding server can be deployed in EDN is FFS.

NOTE:	CBS can be co-located with an ECS in a deployment.

### 6.7.2	Identities

None.

### 6.7.3	Cardinality rules

None.

* * * Next Change * * * *

## 6.8	Option #8: Architecture for ACR update in service continuity planning

### 6.8.1	Architecture enhancements

Enhancement of the service continuity planning capability is expected to support update of ACR. As can be seen at the Figure 6.8.1-1, this solution proposes the ACR update capabilities as enhancements after the ACR launch to deal with UE behavior changes. This includes a Detection entity, a Decision Update entity and an ACR update execution entity. These entities can be different based on the scenarios identified in TS 23.558, clause 8.8.2.

Figure 6.8.1-1: high level illustration of proposed service continuity planning enhancements

* * * Next Change * * * *

## 7.6	Solution #6: ACR update in service continuity planning

### 7.6.1	Architecture enhancements

Option #8 in the clause 6.8 is the basis for this solution. Enhancement of the service continuity planning capability is expected to support update of ACR. As can be seen at the Figure 7.6.1-1, this solution proposes the ACR update capabilities as enhancements after the ACR launch to deal with UE behavior changes. This includes a Detection entity, a Decision Update entity and an ACR update execution entity. These entities can be different based on the scenarios identified in TS 23.558, clause 8.8.2.

Figure 7.6.1-1: high level illustration of proposed service continuity planning enhancements

* * * Next Change * * * *

### 7.11.3	Solution evaluation

This solution provides a deployment option for alignment with ETSI MEC using CAPIF.

This solution addresses KI#5: Alignment of EDGEAPP and ETSI MEC as specified in the clause 5.5; and KI#2: Enablement of Service APIs exposed by EAS as specified in the clause 5.2.

This solution relies on the EDGEAPP architecture as specified in TS 23.558 [2] with extended roles of EAS and EES as CAPIF entities and extended roles of ETSI MEC entities like MEP as CAPIF entities to support the alignment of discovery and invocation of EES|EAS Service APIs and MEC Services by ETSI MEC and EDGEAPP entities, respectively.

This solution also relies on the CAPIF as specified in TS 23.222 [16].

## 7.12	Solution #12: Service continuity planning permission

### 7.12.1	Architecture enhancements

This solution provides a deployment option for alignment with ETSI MEC using CAPIF.

This solution addresses KI#5: Alignment of EDGEAPP and ETSI MEC as specified in the clause 5.5; and KI#2: Enablement of Service APIs exposed by EAS as specified in the clause 5.2.

This solution relies on the EDGEAPP architecture as specified in TS 23.558 [2] with extended roles of EAS and EES as CAPIF entities and extended roles of ETSI MEC entities like MEP as CAPIF entities to support the alignment of discovery and invocation of EES|EAS Service APIs and MEC Services by ETSI MEC and EDGEAPP entities, respectively.

This solution also relies on the CAPIF as specified in TS 23.222 [16].

None.

* * * Next Change * * * *

## 7.30	Solution #30: Common EAS selection

### 7.30.1	Architecture enhancements

Figure 7.30.1-1: EDGEAPP architecture enhanced with binding server

The EDGE-X reference point is introduced to support EES interaction with binding server. The EES can store, update and remove the binding information via EDGE-X reference point.

The binding server is deployed in a central cloud and it is a single server maintaining binding information.

Editor's note:	Whether binding server can be deployed in EDN is FFS.

NOTE:	CBS can be co-located with an ECS in a deployment.

Architecture enhancements in clause 6.7 is the basis for this solution.

* * * Next Change * * * *

## 7.36	Solution #36: Alignment of EDGEAPP and ETSI MEC

### 7.36.1	Architecture enhancements

Architecture requirements in clause 5.4 will serve as the guiding principles for the alignment of EDGEAPP and ETSI MEC architectures.

The following requirements will serve as the guiding principles for the alignment of EDGEAPP and ETSI MEC architectures.

1.	The scope of 3GPP alignment efforts between EDGEAPP and ETSI MEC shall be limited to architecture enhancements that apply only to EDGEAPP.

NOTE: 3GPP will liaise with ETSI ISG MEC for architecture recommendation (if any).

2.	The architecture enhancements to support alignment shall ensure backwards compatibility with the existing EDGEAPP architecture.

3.	The architecture enhancements shall not consider to align features exclusive to the each of the EDGEAPP and ETSI MEC architectures, i.e. the architecture enhancements shall only focus on the overlapping aspects between the EDGEAPP and ETSI MEC architectures.  The alignment aspects in the present release of this specification includes the following:

a.	alignment of EAS profile (EDGEAPP) and appInfo (ETSI MEC),

b.	alignment of EDGE-3/Mp1 reference points

c.	alignment of EDGE-9/Mp3 reference points

d.	usage of CAPIF between the two architectures

NOTE: The term alignment does not imply that the reference points will be exact equivalents.

Editor's note:	Any additional aspects to be considered for alignment is FFS.

4.	The architecture enhancements shall ensure that EDGEAPP architecture can remain as a standalone or a complete system i.e. EDGEAPP can be deployed independent of the ETSI MEC architecture.

5.	The architecture enhancements shall analyse impacts to other working groups e.g. SA2, SA3 and SA5.

* * * End of Changes * * * *