3GPP TSG-SA WG6 Meeting #49-bis-e	S6-221572

e-meeting, 22nd June – 1st July 2022	(revision of S6-22xxxx)

Source:	Samsung

Title:	Pseudo-CR on updates to the PIN profile

Spec:	3GPP TR 23.700-78 V0.3.0

Agenda item:	9.13

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

PIN profile information maintained at the PIN server, PEMC , PEGC and PINEs are of 2 types

- Static PIN profile information

- Dynamic PIN profile information

Information into Static PIN profile can be configured by the PIN owner or any authorized user and these information are not changed/updated frequently. These information persists across the multiple activation and deactivation of PIN and can be deleted once the PIN is deleted.

Information contained in the Dynamic PIN profile is maintaned by PIN server, PEMC and PEGC dynamically and these information contains the current status of PIN. E.g., Which PIN elements are PEMC currently, which PIN elements are PEGC currently, list of PIN elements joined the PIN, current PIN capabilities etc.,  This dynamic information needs to be maintained and updated by these entities and when required share with other PINEs. For e.g., during PEGC/PEMC role change the PIN dynamic information need to shared wit the new PINE taking the role.

Also this pCR covers the information present as part of the PIN client profile which needs to be shared by the PIN client when joining the PIN.

2. Reason for Change

This pCR intends to capture the information which are to be present in static and dynamic PIN profile.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-78 V0.3.0.

* * * First Change * * * *

### 7.7.2	Solution description

#### 7.7.2.1	General

This solution addresses aspects of Key Issue #1.

For an established PIN, the PIN should have the profile or configuration information in PIN server, PEMC and PEGC. Also the PIN server, PEMC and PEGC shall maintain the dynamic profile information related to the PIN which needs to be updated based on the events (e.g. PIN element joining or leaving, PIN element capability is changed) occuring in the PIN.

#### 7.7.2.2	PIN Profile in a PIN

Table 7.7.2.2.1-1: PIN Profile in PIN server, PEMC, PEGC and PINE

| Parameter Name | Parameter Description | PIN Server | PEMC | PEGC | PINE |
| --- | --- | --- | --- | --- | --- |
| PIN ID | The identifier of the PIN | Y | Y | Y | Y |
| PIN Description | Human-readable description of the PIN, for example, the company name, location or the type of service. | Y | Y | Y | Y |
| Duration | Specifies the time period of how long the PIN can be active | Y | Y | Y | Y |
| Maximum number of PIN elements | Maximum number of PIN elements allowed to join the PIN | Y | Y | N | N |
| PEMC list | The list of identifiers of the PIN elements which can be allowed to take the role as PEMC (e.g.: PIN client ID, UE GPSI etc.,) | Y | Y | Y | Y |
| PEGC list | The list of identifiers of the PIN elements which can be allowed to take the role as PEGC (e.g.: PIN client ID, UE GPSI etc.,) | Y | Y | Y | Y |
| PIN Server ID | The identifier of the PIN server that serves the PIN | N | Y | Y | Y |
| PIN server Endpoint | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the PIN server. | N | Y | Y | Y |
| PIN Elements List | List of PIN elements which can be allowed to join the PIN > PIN element ID | Y | Y | Y | N |

Editor's note: Other parameters will be defined of updated in PIN in future.

#### 7.7.2.3	Dynamic profile information of a PIN

Table 7.7.2.3-1 describes the list of parameters that are classified as dynamic profile information and which are maintained at the PIN server, PEMC and PEGC. Dynamic profile information maintained at these entities are updated based on the following events occurring in the PIN:

-	PINE joins or leaves the PIN;

-	Role of PEMC or PEGC changes;

-	When the services offered by the PIN changes;

-	When a PINE updates the services it offers;

-	When a PINE joins or leaves the PIN;

Table 7.7.2.3-1: Dynamic profile information of a PIN

| Parameter Name | Parameter Description | PIN Server | PEMC | PEGC |
| --- | --- | --- | --- | --- |
| PIN ID | The identifier of the PIN | Y | Y | Y |
| PIN Description | Human-readable description of the PIN, for example, the company name, location or the type of service. | Y | Y | Y |
| Services offered | List of services offered by the PIN | Y | Y | N |
| Duration | Time period specifying how long the PIN can be active |  |  |  |
| PEMC list | The list of identifiers of the PIN elements which are currently serving as PEMC (e.g.: PIN client ID, UE GPSI etc.,) | Y | Y | Y |
| > PEMC Endpoint | Endpoint information of each PEMC (e.g. URI, FQDN, IP address) used to communicate with the PEMC. |  |  |  |
| > Duration | Time period of being PEMC |  |  |  |
| PEGC list | The list of identifiers of the PIN elements which are currently serving as PEGC (e.g.: PIN client ID, UE GPSI etc.,) | Y | Y | Y |
| > PEGC Endpoint | Endpoint information of each PEGC (e.g. URI, FQDN, IP address) used to communicate with the PEGC. |  |  |  |
| > Duration | Time period of being PEGC |  |  |  |
| > PIN Elements details | List of PIN elements being served by PEGC and their connectivity information |  |  |  |
| PIN Server ID | The identifier of the PIN server that serves the PIN | N | Y | Y |
| PIN server Endpoint | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the PIN server. | N | Y | Y |
| PIN Elements List | List of PIN elements currently registered/joined the PIN | Y | Y | N |
| > PIN Element ID | Identifty of the PIN element |  |  |  |
| > Services offered | Services offered by the the PIN element |  |  |  |
| > Reachability information | Reachability information of the PIN element |  |  |  |

Editor's note: Relationship between the PIN profile and dynamic profile information of PIN is FFS.

#### 7.7.2.4	PIN client profile

Table 7.7.2.4-1 describes the list of parameters of the PIN element profile which needs to be shared by the PIN client while joining or registering to the PIN.

Table 7.7.2.4-1: PIN Client Profile

| Parameter Name | Status | Parameter Description |
| --- | --- | --- |
| PIN ID | M | The identifier of the PIN where the PIN client is joined |
| UE identifier | O | PIN Element or UE identifier |
| PIN client ID | M | The unique identity of the PIN client within PIN |
| Name of the device | O | Human-readable name of the device (i.e. door sensor, watch, smart TV, etc) along with manufacturer details |
| Application List | O | List of application identities |
| capabilities | M | capabilities of the PIN client like supports relay capability for other PIN elements |
| Visibility | M | Determines whether this PIN element is discoverable by other PIN elements within PIN, discoverable by other UEs outside the PIN etc., |
| access type | M | Access type supported for the communication |
| Layer-2 details | O | Layer-2 address of the PIN element |

* * * End Change * * * *