3GPP TSG-SA WG6 Meeting #49-e	S6-220996

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0103 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS monitoring clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2022-04-29 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For Unicast QoS monitoring, it is unclear how to handle the subscription once the reporting termination conditions are met. And for the threshold based reporting, it is unclear in which direction (e.g. ascending) the reporting should be performed. The default setting for reporting termination is set to threshold based event trigger, the VAL server is still required to give detailed threshold settings. To ease VAL server’s handling, the explicit termination should be used as default setting. Last, it is also unclear how to handle threshold-based termination if multiple measurement indexes are provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe subscription termination if reporting termination conditions are met. Add reaching direction for threshold based reporting. Change default setting for reporting termination. add termination condition for threshold-based termination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear procedure for stage 3 to proceed protocol realization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.20, 14.3.3.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 14.3.2.20	Unicast QoS monitoring subscription request

Table 14.3.2.20-1 describes the information flow from the VAL server to the network resource management server for unicast QoS monitoring subscription request.

Table 14.3.2.20-1: Unicast QoS monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL UE IDs | O (see NOTE 1) | List of VAL UEs whose QoS monitoring data is requested. |
| VAL group ID | O (see NOTE 1) | The group ID used for the VAL group for which QoS monitoring data is requested. |
| List of VAL stream IDs | O (see NOTE 1) | List of VAL streams for which QoS monitoring data is requested. |
| Measurement requirements | O | It describes the requirements on the QoS measurements. |
| > type of data | M | At least one of the following measurement index shall be provided. |
| >> packet delay | O | UL/DL/RT packet delay. For VAL stream, the packet delay means measurement from a source UE to a target UE. |
| >> packet loss rate | O | Average packet loss rate. |
| >> data rate | O | Average data rate and/or maximum data rate. |
| >> traffic volume | O | Average traffic volume for UL and/or DL |
| > measurement time period | O | The measurement time period indicates a current time period. If absent, current time and 5 minutes duration are used as default setting. |
| > aggregation granularity window | O | Indicates how long the QoS monitoring is performed to compute the average value and gather the maximum value. If absent, 1 minute is used as default setting. |
| Reporting requirements | O | It describes the requirements for QoS monitoring reporting. |
| > Frequency of reporting | O | It indicates the requested frequency of reporting. The reporting frequency may be periodic or event triggered (i.e. threshold reached). If absent in the request, default event triggered reporting is used. |
| > Reporting threshold | O | If the Frequency of reporting is event triggered (threshold reached), the reporting threshold corresponding to the measurement index shall be provided. |
| > Threshold reaching direction | O | It indicates the reaching direction (e.g. ascending, descending or both) for the reporting threshold. It shall be provided if reporting threshold is present. |
| > Reporting periodicity | O | If the Frequency of reporting is periodic, the reporting periodicity shall be provided. |
| > Termination of reporting | O | It indicates when the reporting of QoS monitoring shall stop. It can be event triggered (i.e. number of reports reached or threshold reached), when reporting duration expires or when explicit termination is requested. If absent in the request, it defaults to explicit termination. Once the termination condition is met, the monitoring subscription ceases to exist. |
| > Max number of reports | O | It indicates the maximum number of reports, it shall be provided when Termination of reporting is set to event triggered (number of reports reached). |
| > Termination threshold | O (see NOTE 2) | It indicates the reporting termination threshold corresponding to the measurement index. Each measurement index has termination threshold which is set to the worst case (e.g. lowest data rate a VAL service can tolerate). |
| > Termination condition for threshold | O (see NOTE 2) | It indicates threshold-based termination condition (e.g. all reached or any of them reached) if multiple measurement indexes are provided. |
| > Reporting duration | O | It indicates the reporting duration, it shall be provided when Termination of reporting is set to duration expiration. |
| NOTE 1: Only one of these information elements shall be present. NOTE 2: It shall be provided when Termination of reporting is set to event triggered (threshold reached). |  |  |

* * * Next Change * * * *

###### 14.3.3.4.2.2	Procedure

Figure 14.3.3.4.2.2-1 illustrates the high level procedure of unicast QoS monitoring notification event.

Pre-conditions:

-	The VAL server has an active unicast QoS monitoring subscription with the NRM server

Figure 14.3.3.4.2.2-1: Unicast QoS monitoring notification procedure

1.	The NRM server receives QoS monitoring data by means of notifications provided by the NEF. The NRM server coordinates and combines the information from the NEF notifications and determines whether to send a notification to the VAL server based on the VAL server subscription's frequency of reporting. For a VAL group or a list of VAL UEs, the NRM server aggregates QoS monitoring data for each UE belonging to the group or the list; for a VAL stream, the NRM server aggregates the QoS monitoring data for the stream. The NRM server stores the QoS monitoring data as needed for later retrieval.

2.	The NRM server sends a unicast QoS monitoring notification including the measured QoS data to the VAL server. If the reporting termination criteria is met (e.g. number of reports reached, threshold reached), NRM server shall terminate the subscription.

* * * End of Changes * * * *