3GPP TSG-SA WG6 Meeting #49-e	S6-221000

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0295 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Decoupling signalling and media for MCData service capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC, Nokia, Nokia Shanghai Bel, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMCPTT4, IRAIL |  |  |  |  |  | Date: |  |  | 2022-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The functional models as currently specified require that signalling and media is processed together by the MC server. For rail, which does not have this requirement and is more geared towards the automation of rail traffic, there is a requirement that data applications have very low latency requirements (<100msec, <10msec). Thus, the media transition is to be placed in the immediate vicinity of the radio, which is not necessary for the signaling. The requirements with regard to latency and packet reliability are detailed in 3GPP TS 22.289 and 3GPP TR 23.783 has taken up this fact and discussed it sufficiently. In order to meet such requirements, alternatively, verticals need to be able to manage signalling separately from the media. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR describes the minimum adjustments that are necessary to enable signalling and media to be managed separately. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements regarding very low latency application support cannot be fullfiled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1 (23.282), 5.6.1 (23.289) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******************************************1st Change******************************************

### 6.4.1	On-network functional model

Figure 6.4.1-1 shows the generic application plane functional model.

Figure 6.4.1-1: Generic application plane functional model

In the model shown in figure 6.4.1-1, capability functions (SDS, FD, DS, IPcon) of the MCData client and the MCData server along with their reference points (MCData-cap-1 to MCData-cap-n) are described in the respective functional models for each capability.

NOTE:	The security aspects of new network components (MCData message store, Message store client and MCData notification server) and the associated new reference points are the responsibility of SA3 and thus outside the scope of the present document.

An MCData server controls the signaling reference points and the corresponding media allowing different paths for media and signalling.

NOTE:	Any implications regarding MC user data logging are left to implementation.

******************************************End of Changes *****************************************