3GPP TSG-SA WG6 Meeting #49-e	S6-221001

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0063 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Decoupling signalling and media for MCData service capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC, Nokia, Nokia Shanghai Bel, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMCPTT4 |  |  |  |  |  | Date: |  |  | 2022-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The functional models as currently specified require that signalling and media is processed together by the MC server. For rail, which does not have this requirement and is more geared towards the automation of rail traffic, there is a requirement that data applications have very low latency requirements (<100msec, <10msec). Thus, the media transition is to be placed in the immediate vicinity of the radio, which is not necessary for the signaling. The requirements with regard to latency and packet reliability are detailed in 3GPP TS 22.289 and 3GPP TR 23.783 has taken up this fact and discussed it sufficiently. In order to meet such requirements, alternatively, verticals need to be able to manage signalling separately from the media. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR describes the minimum adjustments that are necessary to enable signalling and media to be managed separately. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements regarding very low latency application support cannot be fullfiled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******************************************1st Change******************************************

### 5.6.1	On-network functional model

Figure 5.6.1-1 shows the generic functional model for the application plane for an MCData system using the 5GS.

Figure 5.6.1-1: Generic MCData functional model for application plane

In the functional model shown in figure 5.6.1-1, the following is considered:

-	The description of the corresponding functional entities and reference points in 3GPP TS 23.282 [5] applies.

-	The description of the MCData-5 reference point in 3GPP TS 23.282 [6] applies considering that it exists between the MCData server and the 5GS. It is used for resource management of MCData sessions, e.g. QoS control, and utilizes the N5 reference point or the Rx reference point or the N33 reference point as defined in 3GPP TS 23.501 [7], 3GPP TS 23.502 [10] and 3GPP TS 23.503 [9].

-	MCData-5, utilizing Rx reference point or N5 reference point, may be used when the MCData service provider directly interacts with operator's relevant 5GS network function for QoS control for both unicast PDU sessions and MBS sessions (if dynamic PCC is applicable).

-	MCData-5, utilizing N33 reference point, may be used when the MCData service provider is limited by the operational agreement, i.e., indirect interaction with operator's 5GS network functions for QoS control.

-	The respective functional models supporting MCData capabilities (e.g., SDS, FD, DS, IPcon) over unicast transmissions along with the corresponding reference points (i.e., MCData-cap-1 to MCData-cap-n) described in 3GPP TS 23.282 [5] also apply when the 5G system is used.

-	The MCData-6 reference point, which exists between the MCData server and the 5GS, is used to create an MBS session obtaining multicast or broadcast resources for MCData application usage.

-	The MCData-6 reference point utilizes Nmb13 reference point when the MCData service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCData service provider domain.

-	The MCData-6 reference point utilizes Nmb10 reference point when MBSF is used.

-	The MCData-6 reference point utilizes N33 reference point when the MCData service provider is limited by the operational agreement for QoS control, i.e. indirect interaction with operator's 5GS network functions for QoS control is only allowed.

-	The MCData-6 reference point utilizes Nmb6 reference point when MCData service provider interacts with the Joint BM-SC, MBSF and MBSTF entity to facilitate interworking with LTE.

-	The MCData-6 reference point utilizes the Nmb8 reference point according to 3GPP TS 23.247 [15] when MBSTF is used.

-	For interworking with LTE via Joint BM-SC, MBSF and MBSTF entity, the MCData-6 reference point utilizes the Nmb4 reference point according to 3GPP TS 23.247 [15].

-	An MCData server controls the signaling reference points and the corresponding media allowing different paths for media and signalling.

NOTE:	Any implications regarding MC user data logging are left to implementation.

-	The respective functional models supporting MCData capabilities (e.g., SDS, FD, DS, IPcon) over MBMS transmissions along with the corresponding reference points (i.e., MCData-cap-1 to MCData-cap-n) described in 3GPP TS 23.282 [5] also apply when the 5G MBS is used.

*****************************************End of Changes****************************************