3GPP TSG-SA WG6 Meeting #49-e	S6-221024

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0066 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on network slicing for MC services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5GS |  |  |  |  |  | Date: |  |  | 2022-05-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CT1 has sent an LS asking for clarifications on the configuration and use of slicing by MC services. The intention of stage-2 specs is to enable the MC service provider to control which slice(s) should be requested at initial registration and used from MC services in all deployment scenarios described in clause 6 of TS 23.280, and in particular to specify that - An MC service provider shall be able to set one or more slices as required/default via the MCS UE initial configuration. - The UE shall always include any such S-NSSAIs to the requested NSSAI. This can be achieved either by using the MCS UE initial configuration as part of the Configured NSSAI or of the Default Configured NSSAI. Here, to enable per PLMN flexibility, the MC-related slices to be required used are indicated as part of Configured NSSAI in Initial MC service UE configuration, and with an extra indication of "required" that slices that are crucial for MC operation. To avoid any misconfiguration issues that could limit the use of other applications on an MC UE, it is clarified that the slices set as required in MC configuration should be limited, e.g., up to 2. Similarly, the NSAA credentials can be controlled by MC service provider. The main underlying design principle is that the MC service provider should be able to select the slices requested and used by an MC UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Clarify NS requirements that the MCS UE initial configuration indicates the subset of the configured NSSAI which are for MC services and should be used as per TS. 23.501. 2) Update MCS UE initial configuration 3) Clarify that the MCS UE initial configuration can carry a "required" indication to avoid that the MC slice is not requested, and the MC service runs as per the default URSP rule (see match-all). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear stage-2 NS requirements that cannot be implemented in stage-3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.2, Annex A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

### 4.4.2	Requirements

For the use of network slicing in the MC service context, the following minimum requirements in accordance with 3GPP TS 23.501 [7] shall be considered:

One network slice shall be assigned per PDU session and may benefit from a dedicated transmission resource allocation.

The Initial MC service UE configuration shall contain at least one network slice identity (S-NSSAI) on a per HPLMN and optionally also per VPLMN basis. Those S-NSSAIs shall be considered as part of the Configured NSSAI and handled as specified in 3GPP TS 23.501 [7]. This configuration shall be used by an MC service UE over 5GS to form the Requested NSSAI at registration.

Any S-NSSAI in the Initial MC service UE configuration may be marked as required. If an S-NSSAI exists in the Initial MC service UE configuration, is marked as required and has not been rejected by the network, the MC service UE shall include it in the Requested NSSAI.

NOTE:	It is recommended to avoid indicating more than two slices as required, since the total number of S-NSSAI values in a Requested NSSAI cannot exceed eight.

The use of network slices corresponding to non-standardized NSSAIs across PLMN boundaries requires harmonisation to guarantee their availability.

* * * * * * * NEXT CHANGE * * * * * * *

# A.2	Initial MC service UE configuration data

The configuration data defined in 3GPP TS 23.280 [3] in Annex A.6 apply, with the following exceptions:

-	DNN and the corresponding DN credentials instead of the PDN credentials shall be used,

-	MC network slice identification information may be provided

i)	to be included in the Configured NSSAI, and to form the Requested NSSAI at registration as specified in 3GPP TS 23.501 [7]), along with the corresponding network slice credentials, and a "required" indication; and

NOTE:	It is up to UE implementation, whether an S-NSSAI value not marked as required is included in the Requested NSSAI. If not included, the corresponding MC traffic will be handled according to the URSP rule with the "match all" Traffic descriptor.

ii) 	to be used as UE local configuration to determine the slice per MC service, as defined in clause 6.1.2.2.1 of 3GPP TS 23.503 [9].

* * * * * * * END OF CHANGES * * * * * * *