3GPP TSG-SA6 Meeting #49-e	S6-221032

# Online, , 16th May 2022 - 25th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0083 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to API invoker onboarding/offboarding in TS 23.222 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, Uangel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAPIF |  |  |  |  |  | Date: |  |  | 2022-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 23.222, CAPIF supports onboarding and offboarding of API invokers in the CAPIF core function via CAPIF-1 or CAPIF-1e. However, 1) the API invoker onboarding/offboarding is missing in the capabilities of API invokers and the supporting operations of CAPIF-1; and 2) the current text provides the same functionality in the API management function via CAPIF-5, which is misleading. While some additional features (e.g., granting or provisioning) for API invoker onboarding are addressed in the CAPIF-5 description, they are not implemented in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed API invoker onboarding/offboarding from the capabilities of the API management function - Removed API invoker onboarding/offboarding from the supporting operations of CAPIF-5 - Added API invoker onboarding/offboarding to the capabilities of API invokers and the supporting operations of CAPIF-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may cause misleading in implementation of the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.3.6, 6.4.2, 6.4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

6.3.2	API invoker

The API invoker is typically provided by a 3rd party application provider who has service agreement with PLMN operator. The API invoker may reside within the same trust domain as the PLMN operator network.

The API invoker supports the following capabilities:

-	Triggering API invoker onboarding/offboarding;

-	Supporting the authentication by providing the API invoker identity and other information required for authentication of the API invoker;

-	Supporting mutual authentication with CAPIF;

-	Obtaining the authorization prior to accessing the service API;

-	Discovering service APIs information; and

-	Invoking the service APIs.

NOTE:	The details of the specific service APIs are out of scope of the present document.

NEXT CHANGE

6.3.6	API management function

The API management function enables the API provider to perform administration of the service APIs. The API management function consists of the following capabilities:

-	Auditing the service API invocation logs received from the CAPIF core function;

-	Monitoring the events reported by the CAPIF core function;

-	Configuring the API provider policies to the CAPIF core function;

-	Monitoring the status of the service APIs; and

-	Registering and maintaining registration information of the API provider domain functions on the CAPIF core function.

NEXT CHANGE

6.4.2	Reference point CAPIF-1 (between the API invoker and the CAPIF core function)

The CAPIF-1 reference point, which exists between the API invoker and the CAPIF core function, is used for the API invoker within the PLMN trust domain to discover service APIs, to authenticate and to get authorization.

The CAPIF-1 reference point supports:

-	Onboarding the new API invokers and offboarding API invokers;

-	Authenticating the API invoker based on the identity and credentials of the API invoker;

-	Mutual authentication between the API invoker and the CAPIF core function;

-	Providing authorization for the API invoker prior to accessing the service API; and

-	Discovering the service APIs information.

NOTE:	The security aspects of CAPIF-1 are specified in subclause 6.2 of 3GPP TS 33.122 [12].

NEXT CHANGE

6.4.8	Reference point CAPIF-5 (between the API management function and the CAPIF core function)

The CAPIF-5 reference point, which exists between the API management function and the CAPIF core function, is used for management of service API, API invoker and API provider domain function information.

The CAPIF-5 reference point supports:

-	Accessing the service API invocation logs by the API management function;

-	Enabling the API management function to monitor the events reported due to the service APIs invocations;

-	Enabling the API management function to configure policies at the CAPIF core function e.g. service API invocation throttling, blocking API invocation for certain duration;

-	Enabling the API provider to monitor the status of service APIs (e.g. pilot or live status, start or stop status of service API);

-	Registering API provider domain functions on the CAPIF core function; and

-	Update of the registration information of API provider domain functions on the CAPIF core function.

NOTE:	The security aspects of CAPIF-5 are specified in subclause 6.6 of 3GPP TS 33.122 [12].

END OF CHANGES