3GPP TSG-SA WG6 Meeting #49-e	S6-221062

e-meeting, 16th – 25nd May 2022	(revision of S6-22xxxx)

Source:	BDBOS

Title:	pCR on architectural requirements for secure exchange between MC systems

Spec:	3GPP TR 23.700-38 v0.2.0

Agenda item:	9.2

Document for:	Approval

Contact:	Dania.Azem@bdbos.bmi.bund.de

1. Introduction

This pCR provides the architectural requirements for secure administrative configuration exchange between MC systems.

2. Reason for Change

In 3GPP TS 22.280, clause 5.16.4 defines requirements on sharing administrative configurations, specifying the main functionalities to be supported and provided by MC systems.

This pCR derives SA6 requirements based on stage 1 requirements, on how the interconnected MC systems expected to interact and work together, in respect to topology hiding and no compromises on the security of either MC system.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-38 v0.2.0.

* * * First Change * * * *

# 6	Architecture requirements

## 6.1	General

This clause provides a general description of enhancements to the architecture requirements.

## 6.2	Requirements

### 6.2.1	General

#### 6.2.1.1	Description

This clause specifies the general requirements for functional architecture.

#### 6.2.1.2 	Requirements

[AR-6.2.1.2-01] An authorized user of an MC system shall be able to exchange administrative configuration parameters and device information with one or more partner MC system(s) at the same time.

[AR-6.2.1.2-02] An authorized user of an MC system shall have the capability to address partner MC system(s) by a worldwide unique identity.

[AR-6.2.1.2-03] An authorized user of an MC system shall have appropriate access to setup, initiate, authorize, verify and review interconnection and migration requests between the primary MC system and the partner MC system(s).

[AR-6.2.1.2-04] The MC system receiving administrative configuration and information requests shall have the capability to react to the requests.

[AR-6.2.1.2-05] MC system receiving a particular request shall be able to notify the requesting MC system, including a result for this request.

[AR-6.2.1.2-06] An authorized user of an MC system shall be able to request information from partner MC system about its available functionalities and offered MC services.

[AR-6.2.1.2-07] MC system shall be able to provide information about its available functionalities and offered MC services.

[AR-6.2.1.2-08] Exchange of information between MC systems shall not compromise the security of either MC system.

[AR-6.2.1.2-09] Exchange of information shall not expose the internal structure or configuration of either MC system.

* * * End of Change * * * *