3GPP TSG-SA WG6 Meeting #49-e	S6-221064

e-meeting, 16th – 25th May 2022	(revision of S6-xxx)

Source:	China Mobile

Title:	update Solution #8

Spec:	3GPP TR 23.700-98

Agenda item:	9.8

Document for:	Approval

Contact:	Xiaohui Shi, shixiaohui@chinamobile.com

1. Introduction

This paper proposes update for solution#8.

2. Reason for Change

Supplement the service KPI definition, the definition in 23.558 should be updated

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98.

* * * First Change * * * *

Solution #8: EAS Service API enablement using CAPIF

7.8.1	Architecture enhancements

None.

7.8.2	Solution description

7.8.2.3	Service KPIs in CAPIF for EAS Service APIs

In TS 23.558, "Service KPI" IEs are specified to provide information about service characteristics provided by EASs; or required by ACs. This is used for discovery or provisioning of EASs which meet the Service KPIs required by ACs.

In the similar manner, Service KPIs of EASs (as API Providers) need to be specified in CAPIF to be used for discovery or provisioning of EAS Service APIs which meet the Service KPIs required by EASs (as API Invokers).

The proposed IEs in CAPIF as specified in 3GPP TS 23.222 [16] related to Service KPIs can be summarized as follows:

1)	Service KPIs provided by EAS as API Provider

a.	Service API publish request

i.	Service API information

-	Service KPI (new IE): information about service characteristics provided by the Service API; can be mapped to EAS Service KPIs in EAS Profile [TS 23.558] of the EAS providing the Service API

2)	Service KPIs required by EAS as API Invoker

a.	Onboard API invoker response

i	Service API information

-	Service KPI per API (new IE): information about service characteristics provided by the Service API which is allowed to access

b.	Service API discover request

i	Query information

-	Service KPI (new IE): information about service characteristics as a criterion for discovering matching Service APIs required by the API invoker

c.	Service API discover response

i	Service API information

-	Service KPI per API (new IE): information about service characteristics provided by the Service API corresponding to the discovery request

3)	Service KPIs for CAPIF interconnection between EESs

a.	Interconnection API publish request

i.	Service API information

-	Service KPI (new IE): information about service characteristics provided by the Service API which is published across CCFs (i.e., implemented in EESs)

b.	Interconnection service API discover request

i	Query information

-	Service KPI (new IE): information about service characteristics as a criterion for discovering matching Service APIs across CCFs (i.e., implemented in EESs)

The relevant information elements of CAPIF are listed as follows. The highlighted with bold text is proposed to add for supporting the Service KPIs.

The EAS Service KPIs is defined as below

# Table 8.2.5-1: EAS Service KPIs

| Information element | Status | Description |
| --- | --- | --- |
| Maximum Request rate | O | Maximum request rate from the API Invoker supported by the server. |
| Maximum Response time | O | The maximum response time advertised for the API Invoker 's service requests. |
| Availability | O | Advertised percentage of time the server is available for the API Invoker 's use. |
| Available Compute | O | The maximum compute resource available for the API Invoker. |
| Available Graphical Compute | O | The maximum graphical compute resource available for the API Invoker. |
| Available Memory | O | The maximum memory resource available for the API Invoker. |
| Available Storage | O | The maximum storage resource available for the API Invoker. |
| Connection Bandwidth | O | The connection bandwidth in Kbit/s advertised for the AC's use. |
| NOTE: The maximum response time includes the round-trip time of the request and response packet, the processing time at the server and the time required by the server to consume 3GPP Core Network capabilities, if any. |  |  |

* * * Next Change * * * *