3GPP TSG-SA WG6 Meeting #49-e	S6-221071

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

Source:	China Mobile

Title:	solution for KI#x Support for load control for VAL applications

Spec:	3GPP TR 23.700-34

Agenda item:	9.10

Document for:	Approval

Contact:	Xiaohui Shi, shixiaohui@chinamobile.com

1. Introduction

This paper proposes a solution for new KI#x support for load control for VAL applications.

2. Reason for Change

In EDN scenario, most VAL applications are multiple instances deployed, and different instances have different private network addresses, also different VAL application instance may have different load limit, or the VAL app has a total access bandwidth limit for a specific user. Based on the above requirements, the following coordination aspects can be studied for this Key Issue:

-	Whether and how to enable the load control for a VAL application?

-	What APIs should SEALDD enabler provide to VAL clients/servers to implement bandwidth management?

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98.

* * * First Change * * * *

7.X	Solution #X: Support for Load control for VAL applications

7.X.1	Architecture enhancements

None.

7.X.2	Solution description

7.X.2.1	General

The following solution corresponds to the new key issue on support for load control for VAL applications.

In most EDN scenarios, VAL applications are multiple instances deployed:

- Each instance has its own public network address, UE access to the VAL application through the domain name of VAL app

- App instances have different private network addresses, and VAL app presents load balanced virtual addresses to UE

SEALDD can offer load balance for VAL app.

For the first scenario, SEALDD needs to implement DNS-based load balancing: SEALDD can return IP address polling to UE through DNS response, to make sure that the VAL app IP that different users access will change one by one

For the second scenario, UE can only see the virtual IP of load balancing, SEALDD masks the internal topology of VAL app with multiple instances

7.X.2.2	Procedure

7.X.3	Solution evaluation

This clause provides an evaluation of the solution.