3GPP TSG-SA WG6 Meeting #49-e	S6-221072

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

Source:	China Mobile

Title:	solution for KI#x Support for load control for VAL applications

Spec:	3GPP TR 23.700-34

Agenda item:	9.10

Document for:	Approval

Contact:	Xiaohui Shi, shixiaohui@chinamobile.com

1. Introduction

This paper proposes a solution for new KI#x support for load control for VAL applications.

2. Reason for Change

In EDN scenario, most VAL applications are multiple instances deployed, and different instances have different private network addresses, also different VAL application instance may have different load limit, or the VAL app has a total access bandwidth limit for a specific user. Based on the above requirements, the following coordination aspects can be studied for this Key Issue:

-	Whether and how to enable the load control for a VAL application?

-	What APIs should SEALDD enabler provide to VAL clients/servers to implement bandwidth management?

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98.

* * * First Change * * * *

7.X	Solution #X: Support for Load control for VAL applications

7.X.1	Architecture enhancements

None.

7.X.2	Solution description

7.X.2.1	General

The following solution corresponds to the new key issue Support for load control for VAL applications

In some scenarios, VAL applications need to restrict the bandwidth of users accessing it:

- The VAL application needs to limit the total bandwidth of all the users accessing it

- The VAL application needs to limit the bandwidth of specific users accessing it

7.X.2.2	Procedure

SEALDD can offer bandwidth management for VAL applications as API

In this solution, when VAL applications needs bandwidth management service for the total users accessing it，it will create bandwidth management request to SEALDD, carrying such key information as VAL application info, bandwidth allocation, direction(uplink/downlink/both) etc

When VAL application needs bandwidth management service for specific users, besides the key information above, bandwidth management request will also need to contain UEID that uniquely identifies a user (e.g. GPSI/SUPI/source IP address)

Figure 7.X.2.2 VAL app request bandwidth management

- VAL app create a bandwidth management request，

- SEALDD response to VAL app

- SEALDD performs bandwidth management according to the request initiated by VAL app，when the bandwidth of specific user or total users that accessing the VAL application exceeds the limit，SEALDD may drop the flow，or returns an access denied response to users

7.X.3	Solution evaluation

This clause provides an evaluation of the solution.