3GPP TSG-SA WG6 Meeting #49-e	S6-221079

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0329 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Allow the user to restrict the dissemination of the location information. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2022-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Stage 1 has the following requirement and which is not yet realized in Stage 2. “[R-6.12-003] The MCX Service shall provide a means for an authorized MCX User to restrict the dissemination of his Location information” The user location received at the server from network or reported by user can be disseminated further to users. Currently there is no procedure available for an authorized user to restrict the dissemination of his/her location information. The CR proposes the new information flows and the procedure to allow the user to restrict the dissemination of the location information or not. The location information restriction is not applicable for emergency procedurs and a dispatcher user. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 10.9.2.17 & 10.9.2.18: Added new information flows with corresponding information elements. In 10.9.3.10: New procedure is defined using the newly added information elements for restrict the dissemination of the location information or not. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The authorized user will not be able to control the dissemination of his/her Location information as expected from stage 1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New (10.9.2.17, 10.9.2.18 & 10.9.3.10) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.9.2.17	Restrict location information dissemination request

Table 10.9.2.17-1 describes the information flow from the location management client to the location management server for restricting the location information dissemination further.

Table 10.9.2.17-1: Restrict location information dissemination request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the requesting MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Restrict Location Information Dissemination | M | Indicates to enable or disable the location information dissemination |
| List of users (see NOTE) | O | Indicates that the restriction of the location information dissemination is applicable for the list of users based on the role or any of the listed users. |
| NOTE: If not present, restricting of the location information dissemination is applicable to any users. |  |  |

#### 10.9.2.18	Restrict location information dissemination response

Table 10.9.2.18-1 describes the information flow from the location management server to the location management client for response to restricting the location information dissemination request.

Table 10.9.2.18-1: Restrict location information dissemination response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the requesting MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Restrict Location Information Dissemination status | M | Indicates the result of restricting the location information dissemination |

* * * Next Change * * * *

#### 10.9.3.10	Restrict location information dissemination procedure

The location management client can control the location information dissemination at any time by sending a restrict location information dissemination request to the location management server, which will indicate to the location management server whether the location information can disseminated further or not.

Figure 10.9.3.10-1 illustrates the high level procedure whether to enable/disable the restriction to distribute the location information further or not.

Figure 10.9.3.10-1: Restrict the location information dissemination procedure

1.	The location management client initiates a request for restricting the location information dissemination, containing an indication to enable the restriction or disable the restriction. By default, the location management server can disable the restriction, i.e. location information of user can be distributed further. The request may provide the list of MC service IDs of MC service users (e.g. based on the role or selected users or any). The list could be pre-defined or dynamic or based on any other criteria.

2.	The location management server checks if the location management client is authorized to restrict the location information dissemination.

3.	The location management server updates restriction information of the reporting location management client. If the location management server does not have the restriction record for the location information dissemination of the requesting location management client before, then just store the restriction record for that location management client.

NOTE:	The restriction of the location information is not applicable for emergency services (e.g. Emergecny Call, Emergency alert etc) and an authorized user (e.g. dispatcher, supervisor).

4.	The location management server sends the restrict location information dissemination response to the location management client to confirm the status of the restrict location information dissemination request.

* * * End of Changes * * * *