3GPP TSG-SA WG6 Meeting #49-e	S6-221124

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0044 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Message delivery based on Messaging Topic for different PLMNs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2022-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR is proposed to add the procedures for Message delivery based on Messaging Topic for UEs/Application Servers reside in different MSGin5G Servers and different PLMNs. It is included in the first objective of MSGIn5G_Ph2 WID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds the procedures for Message delivery based on Messaging Topic for UEs/Application Servers reside in different MSGin5G Servers and different PLMNs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The related feature can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.5.1, 8.7.5.2 and 8.7.5.x (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*****************Change 1************************

8.7.5	Message delivery between different PLMNs

8.7.5.1	General

MSGin5G message may be delivered between different PLMNs.

The procedure specified in clause 8.7.5.2 apply to Point-to-Point message and Group message, and may also apply to Application-to-Point message and Point-to-Application message delivery if agreed by the business agreement between the PLMNs. The procedure specified in clause 8.7.5.x apply to Message delivery based on Messaging Topic.

8.7.5.2	Inter-PLMN message exchange procedure

Pre-condition:

1.	The Message Sender (e.g. MSGin5G Client 1 in MSGin5G UE 1) is registered to the MSGin5G Server 1 in one PLMN.

2.	The Message Receiver (e.g. MSGin5G Client 2 in MSGin5G UE 2) is registered to the MSGin5G Server 2 in another PLMN.

3.	MSGin5G Server 1 and MSGin5G Server 2 have established a secured connection.

Figure 8.7.5.2-1 shows message delivery between MSGin5G endpoints in different PLMNs, where Message Sender is registered in MSGin5G Server 1 and Message Receiver is registered in MSGin5G Server 2.

Figure 8.7.5.2-1: Message delivery between MSGin5G UEs in different PLMNs

1.	The Message Sender sends an MSGin5G message request to MSGin5G Server 1 in PLMN 1 as specified in clause 8.3.2.

2.	The MSGin5G Server 1 analyses the target UE Service ID and determines that the message is targeted to the Message Receiver in PLMN 2, authenticates that the Message Sender is allowed to send a message to the Message Receiver, and then the MSGin5G Server 1 forwards the MSGin5G message request to MSGin5G Server 2 in PLMN 2.

3.	MSGin5G Server 2 forwards the MSGin5G message request to the Message Receiver as specified in clause 8.3.3.

4-6.	If the message delivery status report is requested, the Message Receiver sends a message delivery status report to Message Sender as per procedure specified in clause 8.3.4 and 8.3.5.

*****************Change 2************************

8.7.5.x	Inter-PLMN message exchange procedure based on Messaging Topic

Figure 8.7.5.x-1 shows the inter-PLMN Message delivery to a subscribing service endpoint based on Messaging Topic.

Pre-condition:

1.	The Message Sender (e.g. an MSGin5G UE, a Message Gateway on behalf of a Non-MSGin5G UE or an Application Server) is registered to the MSGin5G Server 1 in PLMN 1.

2.	The Message Receiver (e.g. an MSGin5G UE, a Message Gateway on behalf of a Non-MSGin5G UE or an Application Server) is registered to the MSGin5G Server 2 in PLMN 2.

3.	MSGin5G Server 1 and MSGin5G Server 2 have established a secured connection.

4.	MSGin5G Server 2 subscribed to a Messaging Topic with the MSGin5G Server 1 as specified in clause 8.8.x. A Messaging Topic with the MSGin5G Server 2 address of the subscriber has been created on MSGin5G Server 1.

5.	The MSGin5G Client or Application Server served by MSGin5G Server 2 subscribed to the Messaging Topic with the MSGin5G Server 2. A Messaging Topic with the UE Service ID/AS Service ID of the subscriber has been created on MSGin5G Server 2.

Figure 8.7.5.x-1 shows the inter-PLMN Message delivery based on Messaging Topic when MSGin5G Server 2 forwards the Messaging Topic subscription request from the MSGin5G UE/Application Server served by it to MSGin5G Server 1.

Figure 8.7.5.x-1: Message delivery between MSGin5G UEs in different PLMNs

1.	The Message Sender sends an MSGin5G message request (if the sender is an MSGin5G UE) or API request (if the sender is an Application Server or a Message Gateway on behalf of a Non-MSGin5G UE) to MSGin5G Server 1 in PLMN 1 as specified in clause 8.3.2. A Messaging Topic is included in this request.

2.	The MSGin5G Server 1 checks the subscription the Messaging Topic included in the inbound request and obtains the UE Service ID/AS Service ID of the Message Receiver. The MSGin5G Server 1 determines that the message with ths Messaging Topic is needed to be delivered to an MSGin5G UE/Application Server served by Message Server 2 in PLMN 2 based on the UE Service ID/AS Service ID of the Message Receiver. The MSGin5G Server 1 then sends the MSGin5G message to MSGin5G Server 2.

3	The MSGin5G Server 2 delivers the MSGin5G message request to Messager Receiver based on the UE Service ID/AS Service ID in the inbound message as specified in clause 8.3.3.

4-6.	If the message delivery status report is requested, the Message Receiver sends message delivery status report to Message Sender as per procedure specified in clause 8.3.4 and 8.3.5.

Figure 8.7.5.x-2 shows the inter-PLMN Message delivery to subscribing service endpoints based on Messaging Topic when MSGin5G Server 2 subscribe the Messaging Topic on behalf of all MSGin5G UEs/Application Servers served by it.

Figure 8.7.5.x-2: Message delivery between MSGin5G UEs in different PLMNs

1.	The Message Sender sends an MSGin5G message request (if the sender is an MSGin5G UE) or API request (if the sender is an Application Server or a Message Gateway) to MSGin5G Server 1 in PLMN 1 as specified in clause 8.3.2. A Messaging Topic is included in this request.

2.	The MSGin5G Server 1 checks the subscription the Messaging Topic included in the inbound request and obtains the MSGin5G Server 2 address. The MSGin5G Server 1 determines that the message with ths Messaging Topic is needed to be delivered to the Message Server 2 in PLMN 2 and sends the MSGin5G message to MSGin5G Server 2.

3a	The MSGin5G Server 2 checks the subscription the Messaging Topic included in the inbound request and obtains the UE Service ID/AS Service ID of the Message Receiver 1. The MSGin5G Server 2 delivers the MSGin5G message request to Messager Receiver 1 based on its UE Service ID/AS Service ID as specified in clause 8.8.2.

3b	The MSGin5G Server 2 also delivers the MSGin5G message request to Messager Receiver 2 which subscribed this Messaging Topic as specified in clause 8.8.2.

4a-6a.	If the message delivery status report is requested, the Message Receiver 1 sends message delivery status report to Message Sender as per procedure specified in clause 8.3.4 and 8.3.5.

4b-6b.	If the message delivery status report is requested, the Message Receiver 2 sends message delivery status report to Message Sender as per procedure specified in clause 8.3.4 and 8.3.5.