3GPP TSG-SA WG6 Meeting #49-e meeting	S6-221146

16th – 25th May 2022, Online

Source:	ETRI, Uangel

Title:	Update to Solution #11

Spec:	3GPP TR 23.700-98

Agenda item:	9.9

Document for:	Approval

Contact:	Seung-Ik Lee <seungiklee@etri.re.kr>

1	Introduction

The Solution #11 addresses "KI#5: Alignment of EDGEAPP and ETSI MEC" in alignment with ETSI MEC using CAPIF for exposing and invoking Service APIs as well as "KI#2: Enablement of Service APIs exposed by EAS" for exposing EAS Service APIs.

2	Discussion

This paper proposes to update the solution #11 for description, procedures, and evaluations as follows:

- This solution also covers KI#2

- The figure has a small change in API representation according to the discussions in SA6#48

- The current figure is for a deployment option for non-collocated platforms

- There is one reference from ETSI MEC for mapping ETSI MEC APIs and CAPIF

- As discussed in the previous SA6 meeting, the procedures are the same as specified in TS 23.222

- Solution evaluation with impact to the EDGEAPP architecture

3		Proposal

It is proposed to modify the text of TR 23.700-98 as follows.

START OF CHANGE

7.11	Solution #11: A deployment option for alignment with ETSI MEC using CAPIF

7.11.1	Architecture enhancements

None.

7.11.2	Solution description

7.11.2.1	General

This solution addresses "KI#5: Alignment of EDGEAPP and ETSI MEC" in alignment with ETSI MEC using CAPIF for exposing and invoking Service APIs as well as "KI#2: Enablement of Service APIs exposed by EAS" for exposing EAS Service APIs to ETSI MEC entities. It specifies a CAPIF deployment option for:

1.	allowing EASs to invoke MEC services, and

2.	exposing Service APIs provided by EES or EAS (as described in KI#2) to MEC Applications

The following architecture is proposed to use CAPIF for alignment with ETSI MEC in terms of exposing EAS|EES Service APIs and invoking MEC Services as a deployment option.

Based on the proposed deployment option, the alignment of EDGEAPP and ETSI MEC could be done by mapping or complementing their own APIs to CAPIF APIs, especially for invoking/exposing Service APIs and MEC Services.

Note that Figure 7.11.2.1-1 below is to illustrate what EDGEAPP and ETSI MEC could look like when they exploit CAPIF for invoking and exposing their APIs. There is no such architecture defined in EDGEAPP or ETSI MEC as shown in Figure 7.11.2.1-1.

Figure 7.11.2.1-1: A deployment option for alignment with ETSI MEC using CAPIF (Non-collocated platforms)

The proposed deployment option can be realized with the following implementations:

1)	3GPP EDGEAPP

a.	EES may implement CAPIF Core Function to manage the Service APIs exposed by EAS and EES;

b.	EES may implement CAPIF Core Function to interconnect with CCF of ETSI MEC via CAPIF-6

c.	EAS may implement CAPIF API Provider functions in order to expose EAS Service APIs to EDGEAPP EASs or ETSI MEC Applications

d.	EAS may implement CAPIF API Invoker function to invoke EAS Service APIs or ETSI MEC Services

e.	EAS may invoke EES Service APIs via EDGE-3

f.	EES may implement CAPIF API Provider functions in order to expose EES Service APIs (e.g. UE location API, AC information exposure API, etc.) to API invokers (e.g. EASs, ETSI MEC Applications)

-	i.e. EES Service APIs (as well as EAS Service APIs) need to be published to CAPIF Core Function

2)	ETSI MEC (out of the SA6 scope)

a.	ETSI MEC (e.g. MEP) may implement CAPIF Core Function to manage the MEC Services exposed by MEP and MEC App.

b.	ETSI MEC (e.g. MEP) may implement CAPIF Core Function to interconnect with CCF of EDGEAPP EES via CAPIF-6

c.	ETSI MEC (e.g. MEP, MEC App.) may implement CAPIF API Provider functions in order to enable EDGEAPP EASs to invoke MEC Services via CAPIF-2

d.	ETSI MEC (e.g. MEC App.) may implement CAPIF API Invoker function to invoke EDGEAPP EAS|EES Service APIs

NOTE 1: Refer to Annex B.2 of ETSI GS MEC 011 [14] for mapping MEC service management API to CAPIF APIs.

7.11.2.2	Procedure

The procedures for API publication, API discovery, API invocation, and CCF interconnection are as specified in TS 23.222 [16].

7.11.3	Solution evaluation

This solution provides a deployment option for alignment with ETSI MEC using CAPIF.

This solution addresses KI#5: Alignment of EDGEAPP and ETSI MEC as specified in the clause 5.5; and KI#2: Enablement of Service APIs exposed by EAS as specified in the clause 5.2.

This solution relies on the EDGEAPP architecture as specified in TS 23.558 [2] with extended roles of EAS and EES as CAPIF entities to support the alignment of discovery and invocation of EES|EAS Service APIs and MEC Services for ETSI MEC and EDGEAPP entities, respectively.

This solution also relies on the CAPIF as specified in TS 23.222 [16].

END OF CHANGES