3GPP TSG-SA WG6 Meeting #49-e	S6-221160

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0105 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEAL Notification Management service – Information Flows and Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Recently in 23.700-98 the following EN is added as part of the Edge Notification service architecture : Editor's Note: Whether ENS should be addressed as a generic function provided as part of SEAL is FFS. Many of the applications requires notification functionality and its becoming a more common functionality. The 3GPP document TS 23.434 has identified many set of common services (e.g., group management, configuration management, location management) which is used by many verticals and provided a functional architecture for SEAL and the procedures, information flows and APIs for each service within SEAL in order to support vertical applications over the 3GPP system. This CR aims to make the notification mechanism as SEAL service so that it can be used by verticals and thus offloading the complexity of maintaining subscription and notification logic from the enabler layer |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces notifiction management as seal service and in particular this CR introduces information flows and procedures for the notification management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Notification service needs to be implemented at every VAL layer which requires notification functionality which is redundant and waste of effort. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.3(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 17.3	Procedures and information flows for notification management

### 17.3.1	General

This sub-clause describes the procedure and information flows for notification management SEAL service. The notification management procedures apply to on-network VAL service only.

Editor’s Note: Whether to define the Information flows required between VAL client and notification management client is FFS

#### 17.3.2	Information flows for notification management

#### 17.3.2.1	Create notification channel request

Table 17.3.2.1-1 describes the information flow create notification channel request from the notification management client to the notification management server.

Table 17.3.2.1-1: Create notification channel request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the requesting notification management client |
| Channel type | M | Indicates PULL or PUSH method to be used for delivering the notification messages |
| PUSH channel details (see NOTE) | O | Carries the details of the type of PUSH delivery and its associated data |
| Validity Duration | M | How long the notification channel will last (i.e. channel lifetime) as requested by the notification management client. |
| NOTE : This IE shall be present of the channel type is PUSH |  |  |

#### 17.3.2.2	Create notification channel response

Table 17.3.2.2-1 describes the information flow create notification channel response from the notification management server to the notification management client.

Table 17.3.2.2-1: Create notification channel response

| Information element | Status | Description |
| --- | --- | --- |
| Identify | M | Identify of the requested notification management client |
| Notification URL | O | The URL to receive the notification message if a PULL method is requested. For some PUSH method implementation (such as WebSockets) this URL is used to start the PUSH notification service from the notification management server |
| Callback URL | O | The URL used by the notification management client to subscribe to receive notifications from notification management server |
| Validity duration | M | How long the notification channel will last (i.e. channel lifetime) as granted by the notification management server. |
| Channel ID | M | Unique identifier for the created notification channel |

#### 17.3.2.3	Open notification channel

Table 17.3.2.3-1 describes the information flow open notification channel from the notification management client to the notification management server.

Table 17.3.2.3-1: Open notification channel

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting notification management client |
| Notification URL | M | The URL to receive the notification message |
| Channel ID | M | Identifies the notification channel and the value shall be same as returned in Create notification channel response |

#### 17.3.2.4	Re-use notification channel request

Table 17.3.2.4-1 describes the information flow re-use notification channel request from the notification management client to the notification management server.

Table 17.3.2.4-1: Re-use notification channel request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting notification management client |
| Notification URL | M | The URL to receive the notification message |
| Channel ID | M | Identifies the notification channel and the value shall be same as returned in Create notification channel response |
| VAL UE Identity | M | Identity of the VAL UE or VAL user intending to receive notifications from the VAL server |

#### 17.3.2.5	Re-use notification channel response

Table 17.3.2.5-1 describes the information flow re-use notification channel response from the notification management client to the notification management server.

Table 17.3.2.5-1: Re-use notification channel response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting notification management client |
| Callback URL | M | The URL to be used while subscribing to the VAL servers for receiving notifications |
| Result | M | Indicates whether the notification channel can be re-used or not |
| Notification URL(see NOTE) | O | The URL to receive the notification message if a PULL method is requested. For some PUSH method implementation (such as WebSockets) this URL is used to start the PUSH notification service from the notification management server |
| Channel ID (see NOTE) | O | Unique identifier for the created notification channel |
| Validity duration (see NOTE) | O | How long the notification channel will last (i.e. channel lifetime) as granted by the notification management server |
| NOTE: These information elements shall be present if the notification management server wants the notification management client to use a new notification channel instead of reusing the existing channel. |  |  |

#### 17.3.2.6	Update notification channel request

Table 17.3.2.6-1 describes the information flow update notification channel request from the notification management client to the notification management server.

Table 17.3.2.6-1: Update notification channel request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting notification management client |
| Channel ID | M | Identifies the notification channel and the value shall be same as returned in Create notification channel response |
| Validity duration | M | How long the notification channel will last (i.e. channel lifetime) as requested by the notification management client |

#### 17.3.2.7	Update notification channel response

Table 17.3.2.7-1 describes the information flow update notification channel response from the notification management server to the notification management client.

Table 17.3.2.7-1: Update notification channel response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting notification management client |
| Validity duration | M | How long the notification channel will last (i.e. channel lifetime) as requested by the notification management client |
| Result | M | Indicates if the update is success or failure |

#### 17.3.2.8	Delete notification channel request

Table 17.3.2.8-1 describes the information flow delete notification channel request from the notification management client to the notification management server.

Table 17.3.2.8-1: Delete notification channel request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting notification management client |
| Channel ID | M | Identifies the notification channel to be deleted |

#### 17.3.2.9	Delete notification channel response

Table 17.3.2.9-1 describes the information flow delete notification channel response from the notification management server to the notification management client.

Table 17.3.2.9-1: Delete notification channel response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting notification management client |
| Result | M | Indicates if the deletion of the channel is success or failure |

#### 17.3.2.10	Notification message

Table 17.3.2.10-1 describes the information flow notification message from VAL server to the notification management server and from notification management server to the notification management client.

Table 17.3.2.10-1: Notification Message

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL UE or VAL user to whom this notification message to be sent |
| VAL Application ID | M | Identity of the VAL application residing in the VAL UE this notification message is targeted |
| VAL Service ID | M | Identity of the VAL service for which this notification is generated |
| Event-data | M | This information element carries the details of the notification data such as type of the event and the data corresponding to the event |

### 17.3.3	Procedure for creating notification channel to receive notifications

Figure 17.3.3-1 below illustrates the procedure for receiving notifications from the notification management server by the notification management client.

Pre-conditions:

1.	The notification management client does not have any notification channel open with the notification management server.

Figure 7.13.3-1: Receiving notifications from notification management server

1.	The VAL client requests the notification management client to subscribe to and receive notifications from the VAL server.

2.	The notification management client creates notification channels (i.e. endpoint URLs), if not created already, to be used by the VAL server to send notification messages. To create notification channel, the notification management client sends a create notification channel request to the notification management server. The desired validity duration for the channels to be used and the notification channel type (PUSH or PULL) are included in the request.

3.	The notification management server authenticates the notification management client and authorizes its request.

4.	The notification management server sends the notification management client the Create notification channel response with the endpoint URLs that will be used by the VAL server to send the notification messages and the notification management client to receive the notification messages. The notification management server also includes what is the valid duration for these endpoint URLs to be used and the channel ID to uniquely identify the channel created in the response.

5.	If the notification type is PULL method, the notification management client sends the Open notification channel to the notification management server to start receiving the notification message. For certain PUSH method notification type (such as WebSockets) the notification management client requests the notification management server to start the PUSH notification service with its specific protocol that is outside the scope of this specification.

NOTE: This step is not required if the notification client wants to receive the notification messages via PUSH enabler server.

6.	The notification management client indicates the VAL client the status of its request to receive notifications from the VAL server. It also includes the callback URL which needs to be used by the VAL client while subscribing to the VAL server for receiving notifications.

7.	The VAL client sends the subscribe for notification request to the VAL server for receiving notifications. This request includes the subscription data and the callback URL returned from the notification management server in step 4.

8.	The VAL server sends the subscribe for notification response to the VAL client.

9.	The VAL server detects the event occurred matching the criteria specified in the subscription data and generates the notification message.

10.	The VAL server sends the notification message to the notification management server containing the VAL UE or VAL user ID, VAL service ID, VAL application ID and the notification data.

11.	If the delivery method is PULL, the notification management server sends the notification message to the notification management client over the opened notification channel.

NOTE 2: If the delivery method is PUSH, the notification management server sends the notification message to the PUSH Enabler server (not shown in the figure and is outside the scope of this specification) to deliver to the notification management client.

Editor’s Note: Whether to capture the procedure involving delivery of notification messages using indirect channel via Push Enabler Server as normative or informative is for FFS

12.	The notification management client delivers the received notification message to the appropriate VAL client based on the details as received in the notification message from the notification management server.

### 17.3.4	Procedure for re-using notification channel

Figure 17.3.4-1 below illustrates the procedure for receiving notifications from the notification management server by the notification management client by reusing the existing notification channel.

Pre-conditions:

1.	The notification management client already has a notification channel opened with the notification management server based on the request from VAL client 1 existing on VAL UE for a VAL service 1.

2.	The notification management client decides to use the same channel for receiving notification messages for multiple VAL clients residing on the VAL UE.

3.  The notification management server decides to use a different callback URL for the re-use notification channel request

.

Figure 17.3.4-1: Re-using existing channel for receiving notifications from notification management server

1.	The VAL client 2 requests the notification client to subscribe to and receive notifications from the VAL server 2.

2.	The notification management client sends the re-use notification channel request to the notification management server to indicate that it wants to reuse the existing channel to receive the notifications. This request carries the notification URL, channel ID of the existing channel.

3.	The notification management server sends the re-use notification channel response to the notification management client indicating the status of whether the existing channel can be re-used or not. The response carries the call back URL which needs to be used by the VAL client 2 while subscribing to the VAL server 2 to receive notifications. If the notification management server decides not to use existing notification channel, then it sends the details of new notification channel that needs to be used by the notification management client such as notification URL, channel ID and validity duration of the new notification channel.

4.	If the request to re-use the existing notification channel is not approved by the notification management server, the notification management client sends the open channel request by providing the notification URL and channel ID as received in the re-use notification channel response.

5.	The notification management client indicates the VAL client 2 the status of its request to receive notifications from the VAL server. It also includes the callback URL which needs to be used by the VAL client 2 while subscribing to the VAL server 2 for receiving notifications.

6.	Subscription to receive notifications and delivery of notifications occurs as specified in the clause 17.3.3.

* * * End Change * * * *