3GPP TSG-SA WG6 Meeting #4X-e	S6-221169

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.434 | CR | 0106 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEAL Registrar service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.X00. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-X (Release X) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As observed in discussion paper S6-221168, it is required for SEAL service proivider to manage access to SEAL servers and also share VAL service specific configutation and policies to the SEAL servers. In order to provide better access and to manage SEAL services, this paper proposes to define SEAL registrar (server). |  |  |  |  |  |  |  |  |
| Summary of change: |  | On network function model for SEAL registrar service is defined along with functiona entity and referene point descriptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Access to SEAL servers remain scatterred and managing SEAL servers will become difficult as more SEAL services added to the SEAL layer |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X (New), X.1 (New), X.2 (New), X.2.1 (New), X.2.2 (New), X.2.3 (New), X.2.3.1 (New), X.2.3.2 (New), X.2.4 (New), X.2.4.1 (New), X.2.4.2 (New), X.2.4.3 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# X	SEAL Registrar

## X.1	General

The SEAL registrar is a SEAL service that offers the registration related capabilities to one or more vertical applications.

## X.2	Functional model for SEAL Registrar

### X.2.1	General

The functional model for the SEAL Registrar is based on the generic functional model specified in clause 6.

### X.2.2	On-network functional model description

Figure X.2.2-1 illustrates the generic on-network functional model for SEAL registrar.

Figure X.2.2-1: On-network functional model for SEAL registrar

The VAL server(s) communicate with the SEAL registrar over the SR-S interface. The SEAL registrar shares policies and configurations of a VAL server to other SEAL server(s) over SEAL-Xn interface.

* * * Next Change * * * *

### X.2.3	Functional entities description

#### X.2.3.1	General

The functional entities for SEAL registrar service are described in the following clauses.

#### X.2.3.2	SEAL Registrar

The SEAL registrar provides access to the SEAL services provided by a service provider. It provides following functionalities:

-	supports VAL server registration and deregistration to SEAL service layer over SR-S interface.

-	stores the information related to the registered VAL servers to use some (or all) of the SEAL servers provided by the SEAL service provider.

-	stores the VAL service provider specific policies.

-	maintains the VAL server registration and updates other SEAL servers when new VAL server is registered or deregistered.

-	updates other SEAL servers when the service level contract between VAL service provider and SEAL service provider is expired.

NOTE:	The SEAL registrar can act as the CAPIF core function as specified in 3GPP TS 23.222.

* * * Next Change * * * *

### X.2.4	Reference points description

#### X.2.4.1	General

The reference points for the functional model for SEAL registrar are described in the following clauses.

#### X.2.4.2	SR-S

The SR-S supports the interaction between VAL server and SEAL registrar. It supports procedure for VAL server to register or deregister with SEAL registrar, and sharing of VAL service provider specific policy information and configuration to SEAL registrar, and discovery information about SEAL servers to the VAL server.

#### X.2.4.3	SEAL-Xn

The SEAL registrar shares VAL service provider specific policies and configurations to other SEAL server(s) over SEAL-Xn interface. It enables or disables SEAL services based on configurations or service level agreements.

* * * End of Change * * * *