3GPP TSG-SA WG6 Meeting #49-e	S6-221207

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0106 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the functions of EES and EAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2022-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clauses 6.3.2 and 6.3.6 of current specification, the functionalities of EES and EAS are incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the missing functions for EES. - Add the misssing functions for EAS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are incomplete descriptions for the functionalities of EES and EAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

### 6.3.2	Edge Enabler Server (EES)

EES provides supporting functions needed for EASs and EEC.

Functionalities of EES are:

a)	provisioning of configuration information to EEC, enabling exchange of application data traffic with the EAS;

b)	providing API invoker and API exposing functions as specified in 3GPP TS 23.222 [6];

c)	interacting with 3GPP Core Network for accessing the capabilities of network functions either directly (e.g. via PCF) or indirectly (i.e. SCEF/NEF/SCEF+NEF);

d)	exposing events related to ACT;

e)	EEC context transfer between EESs;

f)	supporting external exposure of 3GPP network and service capabilities to the EAS(s) over EDGE-3;

g)	registration functions (i.e., registration, update, and de-registration) for the EEC(s) and the EAS(s);

h) triggering the EAS instantiation on demand;

i) processing ACR launching request (i.e., ACR initiation or ACR determination);

j) providing ACR information notification (i.e., ACR complete and/or the selected T-EAS/T-EES) to EEC based on subscription for required AC(s); and

k) executing EELManagedACR operation for S-EAS.

# * * * Change 2 * * * *

### 6.3.6	Edge Application Server (EAS)

EAS is the application server resident in the EDN, performing the server functions. The AC connects to the EAS in order to avail the services of the application with the benefits of Edge Computing.

It is possible that the server functions of an application are available only as an EAS.

However, it is also possible that certain server functions are available both at the edge and in the cloud, as an EAS and an Application Server resident in the cloud respectively. The server functions offered by an EAS and its cloud Application Server counterpart may be the same or may differ; if they differ, the Application Data Traffic exchanged with the AC may also be different.

The EAS can consume the 3GPP Core Network capabilities in the following ways, all of which are optional to support:

a)	invoking 3GPP Core Network capabilities via the edge enabler layer through the EES

b)	invoking 3GPP Core Network function (e.g. PCF) APIs directly, if it is an entity trusted by the 3GPP Core Network; and

c)	invoking the 3GPP Core Network capabilities through the capability exposure functions i.e. SCEF/NEF/SCEF+NEF.

The S-EAS can provide the selected target EAS information to S-EES, and S-EAS/T-EAS can send ACR status update message to the corresponding S-EES/T-EES.

# * * * End of the Change(s) * * * *