3GPP TSG-SA WG6 Meeting #49-e	S6-221210

e-meeting, 16th – 25th May 2022	(revision of S6-220706, 909)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0098 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sharing location information across VAL servers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Kyonggi University |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-03-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently the location request/report from the VAL server towards the location management client via the Location management server is per VAL service/VAL server. However, it is a common case that multiple VAL servers/services may request the location of same UE. With current mechanism, it will cause much extra signalling exchange between the UE and location management server. Currently, there is no testable way for improving the efficiency of signalling exchange between the UE and the LMS which also has impacts to the energy efficiency of the system when looked upon from a highly scaled operations perspective. It is required that the location reporting is allowed per VAL service (for applications whose granularity of location information is different than the other applications) and also it is allowed that VAL services which have common requirements can use common configuration to obtain the location information. So, this CR proposes an enhanced solution to optimize it where the location management server and client maintain a single subscription for multiple VAL service/server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a sharing policy information to allow multiple VAL server to share the same location from location management client and a single subscription between the LMS and LMC for multiple VAL services/server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Redundant signallings at the UE to report the same location for each VAL service/server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.5, 9.3.7, 9.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.3.5	Client-triggered or VAL server-triggered location reporting procedure

Figure 9.3.5-1 illustrates the high level procedure of client-triggered or VAL server-triggered location reporting.

Pre-conditions:

1.	The location sharing policy is available at the location management server if the UE location can be shared among multiple VAL services.

Figure 9.3.5-1: Client-triggered location reporting procedure

1.	Location management client 2 (authorized VAL user or VAL UE) or VAL server sends a location reporting trigger to the location management server to activate a location reporting procedure for obtaining the location information of location management client 1.

NOTE:	Step 1 can be performed when Location management client 2 or VAL server require to update the location reporting trigger.

2.	Location management server checks whether location management client 2 or VAL server is authorized to send a location reporting trigger. Depending on the information specified by the location reporting trigger, location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1.

If it is allowed to share the location information among multiple VAL services based on the location sharing policy, the location management server reuses the existing location reporting configuration towards the location management client and and updates the location reporting configuration to incorporate the triggers provided in step 1, if required (e.g. updating the minimum time between consecutive reports with the shorter interval).

3.	Once the location information of the location management client 1 is available in the location management server by the on-demand location reporting procedure, a location information report is sent to the location management client 2 or VAL server. If multiple VAL servers request location from the location management client, the location management server sends the location information available at the location management server to the VAL servers with whom the location information can be shared as per the location sharing policy and as per the triggers for the VAL servers.

* * * Next Change * * * *

### 9.3.7	Location information subscription procedure

Figure 9.3.7-1 illustrates the high level procedure of location information subscription request. The same procedure can be applied for location management client and other entities that would like to subscribe to VAL user or VAL UE location information. This procedure is also used for intiating tracking a UE's location.

Pre-conditions:

1.	The location sharing policy is available at the location management server if the UE location can be shared among multiple service.

Figure 9.3.7-1: Location information subscription request procedure

1.	The VAL server sends a location information subscription request to the location management server to subscribe location information of one or more VAL users or VAL UEs. The request may include an indication for supplementary location information.

2.	The location management server shall check if the VAL server is authorized to initiate the location information subscription request. Further, the location management server may initiate location reporting configuration with the location management client of the UE for immediate reporting.

If it is allowed to share the location information among multiple services based on the location sharing policy, the location management server reuses the existing location reporting configuration towards the location management client and and updates the location reporting configuration to incorporate the triggers provided in step 1, if required (e.g. updating the minimum time between consecutive reports with the shorter interval).

3.	The location management server may optionally subscribe for UE location information from 3GPP core network for the UE. If the indication for supplementary location information is included in step 1, then UE location information is obtained from the 3GPP core network.

4.	The location management server determines the UE location information of the UE as received in steps 3 and 4.

5.	The location management server replies with a location information subscription response indicating the subscription status and if immediate reporting was requested, the location information of the VAL UE(s).

### 9.3.8	Event-trigger location information notification procedure

Figure 9.3.8-1 illustrates the high level procedure of event-trigger usage of location information. The same procedure can be applied for location management client and other entities that would like to subscribe to location information of VAL user or VAL UE. This procedure is also used for obtaining latest UE's location for tracking purpose.

Pre-conditions:

1.	The location sharing policy is available at the location management server if the UE location can be shared among multiple service.

Figure 9.3.8-1: Event-trigger usage of location information procedure

1.	The location management server receives the latest location information of the UE as per the location report procedure described in clause 9.3.3.3.

2.	The location management server may optionally receive the location information of the UE from 3GPP core network. If the indication for supplementary location information is included in the subscription, then UE location information is obtained from the 3GPP core network.

3.	Based on the configurations, e.g., subscription, periodical location information timer, location management server is triggered to report the latest user location information to VAL server. The location management server determines the location information of UE as received in steps 1 and 2, including the supplementary location information (if indicated).

4.	The location management server sends the location information report including the latest location information of one or more VAL users or VAL UEs to the VAL server or to the location management client that has previously configured.

If the VAL server is allowed to receive the location information as per the location sharing policy information, then the location management server utilizes the location information from the location management client which commonly applies for the VAL servers as per the location sharing policy information.

5.	VAL server may further share this location information to a group or to another VAL user or VAL UE.

NOTE:	For other entities, the step 5 can be skipped if not needed.

* * * End of Change * * * *