3GPP TSG-SA WG6 Meeting #49-e	S6-221216

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0078 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to de-announcement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The MBS session de-announcement acknowledgement is defined and optional as the MBS session announcement ackknowledge which is instructed by the MC service server by providing the MBS service announcement acknowledgement indication. However, the acknowledgement indication is missing in the de-announcement message. Also the name ‘MBS service announcement ’ and ‘MBS session de-announcement’ are not aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Add the acknowledgement indication in the MBS session de-announcement information flow in clause 7.3.2.11 2) Change the ‘MBS session de-announcement’ to MBS service de-announcement 3) change the the ‘MBS session de-announcement acknowledge’ to MBS service de-announcement acknowledge |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The de-announcement message is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.11, 7.3.2.12, 7.3.3.3.1, 7.3.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.3.2.11	MBS service de-announcement

Table 7.3.2.11-1 describes the information flow of an MBS service de-announcement sent from the MC service server to the MC service clients.

Table 7.3.2.11-1: MBS service de-announcement.

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID(s) | M | The identity of the MBS session(s) to be de-announced. |
| MBS service de-announcement acknowledgement | O | Indicate if the MC service server requires an acknowledgement to the MBS service de-announcement |

#### 7.3.2.12	MBS service de-announcement acknowledgement

Table 7.3.2.12-1 describes the information flow of an MBS service de-announcement acknowledgement message sent from the MC service clients to the MBS service server, to acknowledge the reception of the MBS service de-announcement.

Table 7.3.2.12-1: MBS service de-announcement acknowledgement.

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service identity of the MC service client, whose group is no longer associated to the MBS session(s) |
| MBS session ID(s) | M | The identity of the MBS session(s) to be de-announced. |
| MBS service de-announcement status | M | The de-announcement status per MBS session ID. |

* * * Next Change * * * *

##### 7.3.3.3.1	General

The MC service server can decide to release a certain MBS session once it is no longer further utilized for the associated MC service group communication, e.g., the MC service group is no longer active, the MC media transmission is over and no further MC media to be delivered, group communication is terminated. The MBS session deletion procedure leads to releasing the network resources associated to that MBS session.

NOTE:	It is up to implementation of MC service server to decide whether to release the MBS session or re-use it for subsequent group operations.

To delete the MBS session, the MC service server sends an MBS session deletion request to the 5GS providing the corresponding MBS session ID. The MBS session deletion request is sent to the MB-SMF (directly or via NEF/MBSF) when PCC is not used. However, if dynamic PCC rule is utilized, a policy authorization deletion request is initially sent to the PCF. Further details are provided in 3GPP TS 23.247 [15].

MC service server further informs the MC service client with the MBS service de-announcement, so that the MC client UE stops monitoring the broadcast MBS session or leaves the multicast MBS session. This procedure is applied for both broadcast MBS session and multicast MBS session.

##### 7.3.3.3.2	Procedure

The procedure in figure 7.3.3.3.2-1 describes the MBS session deletion aspects for group communication.

Pre-conditions:

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same active MC service group.

-	An MBS session is configured to address the corresponding MC service group with certain service requirements and optionally with a certain broadcast/multicast service area. The session is announced and established for group communication purposes for the MC service group.

Figure 7.3.3.3.2-1: MBS session deletion procedure.

1.	The MC service server decides to delete the MBS session for the associated MC group communication, either multicast or broadcast session.

2.	The MC service server sends an MBS service de-announcement message with the MBS session ID towards the MC service client(s). Upon receiving the MBS service de-announcement message, either 3a or 3b is performed.

3a.	If the MBS session identified by MBS session ID is a broadcast MBS session, the MC service client(s) stops monitoring the broadcast MBS session and removes the broadcast MBS session related information.

3b.	If the MBS session identified by MBS session ID is a multicast MBS session, the joined MC service client(s) initiate an MBS session leave procedure to leave the indicated MBS session in order to release the respective network resources, as defined in 3GPP TS 23.247 [15].

4. Subsequently, the MC service clients may send an MBS service de-announcement acknowledgement message to the MC service server indicating the status of MBS session.

5.	The MC service server initiates the MBS session deletion procedure with the 5GC (either directly or through NEF/MBSF) in order to stop using the configured MBS session and release the corresponding network resources. The MC service server indicates within the MBS session release request the corresponding MBS session ID. The MBS session deletion procedure can either be with or without a dynamic PCC rule, as indicated in 3GPP TS 23.247 [15].

* * * End of Change * * * *