3GPP TSG-SA WG6 Meeting #49-e	S6-221219

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0333 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Migration during an ongoing private communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2022-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TR 23.700-90 describes a procedure, which allows a MCPTT user to migrate to another MCPTT system and to resume the private communication after migration to the new MCPTT system. This CR copies the procedure from the TR and makes it generic for applicability to all MC services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new generic procedure is added which allows to stop the private communications and initiate a new private call after migration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Migration during an ongoing private communication is not handled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.16.3 (new), 10.16.3.1 (new), 10.16.3.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

### 10.16.3	Migration during an ongoing private communication

#### 10.16.3.1	General

The capability described below is to allow an MC service user to migrate to another MC system during a private communication and to initiate a new private communication in the other MC system.

NOTE: This is only applicable for MCPTT and MCVideo service.

#### 10.16.3.2	Procedure

The procedure supports migration from the primary MC system to a partner MC system, migration between two partner MC systems and migration from a partner MC system towards the primary MC system.

Pre-conditions:

1.	The MC service client has one or more ongoing private calls.

2.	The MC service UE detects the need to change the MC system.

Figure 10.16.3.2-1: Migration to another MC system during an ongoing private call

1.	MC service client 1 releases the private call towards the MC service client 2 and puts the call into suspended state in MC service client.

2.	After migration to the other MC system, the configuration management client 1 triggers retrieval of the MC service user profile used within the partner MC system (TS 23.280 [5] clause 10.1.4.3.2).

NOTE 1:	User authentication, service authorisation and signalling plane procedures are not shown.

3.	MC service client 1 initiates a new private call toward MC service client 2 based on the call is marked as suspended in step 1.

NOTE 2:	Any local call restrictions are considered.

NOTE 3:	If another private call request is sent to MC service client 1 or MC service client 2 before the call has resumed, the actions of the receiving MC service client are outside the scope of the present document and could include rejecting this new private call request.

The MC service client may indicate the successful migration of private call communications to the MC service user.

* * * * * * * END OF CHANGE * * * * * * *