3GPP TSG-SA WG6 Meeting #49-e meeting	S6-221260

16th – 25th May 2022, Online	(revision of S6-221095)

Source:	ETRI, Uangel

Title:	Overall evaluations for Key issue #2

Spec:	3GPP TR 23.700-98

Agenda item:	9.8

Document for:	Approval

Contact:	Seung-Ik Lee <seungiklee@etri.re.kr>

1	Introduction

This paper provides an overall evaluation of the solutions to address Key issue #2: Enablement of Service APIs exposed by EAS.

2	Discussion

The Key issue #2 addresses the enablement of Service APIs exposed by EASs (i.e., EAS Service API) in the Edge Enabler Layer. In order to study the open issues of the Key issue #2, there are two solutions proposed: Solution #8 and #11.

3		Proposal

It is proposed to change the text of TR 23.700-98 as follows.

1st CHANGE

### 10.2.x	Key issue #2: Enablement of Service APIs exposed by EAS

The open issues studied in the key issue #2 are as follows:

1.	Identify any gaps in CAPIF to enable EAS Service APIs in the EDGEAPP architecture in terms of e.g., service-specific attributes for API publish/discovery, API availability subscription/notification across EES/CCF

2.	If any, whether and how to enhance CAPIF capabilities to address the gap identified as above

In order to address the open issues aforementioned, there are two solutions proposed as follows:

- Solution #8: EAS Service API enablement using CAPIF

  - It exploits CAPIF as specified in 3GPP TS 23.222 [16] to support publication/discovery, and change subscription of EAS Service APIs for API invoking by the other EASs

- Solution #11: A deployment option for alignment with ETSI MEC using CAPIF

  - It exploits CAPIF as specified in 3GPP TS 23.222 [16] as a deployment option for exposing EAS Service APIs to ETSI MEC entities (i.e., MEP and MEC applications)

The Solution #8 provides 1) a detailed role of EAS and EES as CAPIF entities in order to support publication/discovery, and change subscription of EAS Service APIs; and 2) a new IE, Service KPIs, for CAPIF APIs.

The Solution #11 provides a detailed role of EAS and EES as CAPIF entities in order to support discovery and invocation of EAS Service APIs for ETSI MEC entities.

EES supporting EAS's and other entity’s (e.g., ETSI MEP or MEC App.) access to service APIs exposed by other EAS, in the context of CAPIF, will be addressed in the normative phase. The newly identified IE for CAPF APIs has an impact to 3GPP TS 23.222 [16] to be addressed in the normative phase.

END OF CHANGES