3GPP TSG-SA WG6 Meeting #49-e	S6-221343

e-meeting, 16th – 25th May 2022	(revision of S6-221086)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0309 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Auto affiliate to MCPTT group for remotely initiated MCPTT call request procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2022-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per the Pre-conditions defined in the procedure “10.16.3.1 Remotely initiated MCPTT call request” of TS 23.379, the reciepient of ‘Remotely initiated MCPTT call request’ an affiliated MCPTT group member of the MCPTT group that is the target of the remotely initiated MCPTT call, Otherwise initiator of the ‘Remotely initiated MCPTT call request’ should use the existing procedures to affiliate the reciepient user to the targeted MCPTT group for remotely initiated MCPTT group call. The above precondition results into initiating user to perform an superfluous step to initiate affiliation procedure on behalf of the reciepient user. This CR proposes an automatic affiliation of the MCPTT group during ‘Remotely initiated MCPTT call request’ procedure and deaffiliate from MCPTT group when the resulting MCPTT group call is terminated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the pre-condition and normative text in step 3 to include the implicit affiliation of the reciepient user of the ‘Remotely initiated MCPTT call request’. Added a NOTE to provide an information on when to de-affiliate the implicitly affiliated reciepient user from the MCPTT group. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An additional step would be required to affiliate an user to an MCPTT group prior to using the ‘Remotely initiated MCPTT call request’ procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.16.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.16.3.1	Remotely initiated MCPTT call request

The remotely initiated MCPTT call request procedure includes the initial remotely initiated MCPTT call request from the MCPTT user to the remote UE and either the MCPTT private call procedures or the MCPTT group call procedures originating at the remote UE.

Procedures in figure 10.16.3.1-1 show the signalling control plane procedures for the MCPTT client initiating a remotely initiated MCPTT call request with the chosen MCPTT user.

Pre-conditions:

1.	If the MCPTT user on MCPTT client 1 wants the resulting remotely initiated MCPTT call to be:

a.	an MCPTT group call, then MCPTT user 2 on MCPTT client 2 is an affiliated MCPTT group member of the MCPTT group that is the target of the remotely initiated MCPTT call.

b.	an MPCTT private call, then the MCPTT user 2 on MCPTT client 2 is permitted to initiate an MCPTT private call to the identified MCPTT user.

Figure 10.16.3.1-1: Remotely initiated MCPTT call request

1.	MCPTT user on MCPTT client 1 initiates a remotely initiated MCPTT call request to the MCPTT user of MCPTT client 2.

2.	MCPTT client 1 sends a remotely initiated MCPTT call request towards the MCPTT server.

3.	MCPTT server checks whether the MCPTT user at MCPTT client 1 is authorized to initiate a remotely initiated MCPTT call request. If the resulting of this request is to initiate a group call, MCPTT client 1 is authorized to remotely initiate the MCPTT call request, and if MCPTT client 2 is a member of the group, the MCPTT server implicitly affiliates the MCPTT user 2 on MCPTT client 2 to the MCPTT group if the MCPTT client 2 is not already affiliated and notifies the MCPTT client 2 of this affiliation change.

4.	If authorized, MCPTT server sends the corresponding remotely initiated MCPTT call request towards the MCPTT client 2.

5.	Based on the received information the receiving MCPTT client 2 may notify the user of the remotely initiated MCPTT call request.

6.	Optionally the receiving MCPTT client 2 sends a remotely initiated MCPTT call response to the MCPTT server.

7.	After receiving the remotely initiated MCPTT call response from MCPTT client 2, the MCPTT server informs the MCPTT client 1 about successful remotely initiated MCPTT call request.

NOTE 1:	Step 6 and step 7 might not be sent, since it could be determined that the remotely initiated MCPTT call was successful by receiving the MCPTT call initiated by MCPTT client 2.

8.	Based on the received information the MCPTT client 2 initiates an MCPTT call (either an MCPTT group call or an MCPTT private call) using the normal MCPTT call establishment procedures (10.6.2.3.1.1.2 or 10.7.2.2) with implicit floor request.

NOTE 2:	Step 6 and step 7 are received in this order. However, step 6 or step 7 or both might occur before or after step 8.

9.	If the remotely initiated call is a group call, then when the ongoing MCPTT group call is terminated, the MCPTT server de-affiliates the MCPTT user 2 on MCPTT client 2 from the MCPTT group if MCPTT client 2 is implicitly affiliated as defined in the step 3 above (the de-affiliation is not shown in the figure for simplicity).

The result of these procedures is an on-going MCPTT (group or private) call which includes MCPTT client 1.

* * * End of Changes * * * *