3GPP TSG-SA WG6 Meeting #49-e	S6-221417

e-meeting, 16th – 25th May 2022	(revision of S6-221215)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0077 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE session join notification message name alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The MBS UE session join notification is defined in clause 7.3.2.2, but the message used in the procedure use a different name.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the MBS UE session join notification to UE session join notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency between the information flow and the procedures |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.2, 7.3.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.3.2.1	MBS service announcement

Table 7.3.2.1-1 provides the information elements during MBS service announcement, which are sent by the MC service server to the clients. The MBS service announcement includes information elements related to the announced MBS session. Optionally it includes eMBMS related information elements once needed.

Table 7.3.2.1-1: MBS service announcement for the use of MBS sessions

| Information element | Status | Description |
| --- | --- | --- |
| 5G MBS session information | M | Providing the MBS session related information if MCX server decides to use 5G MBS session to deliver MC service group communication data |
| >MBS session ID | M | The identity of the MBS session used to deliver MC service group communication data. It is either TMGI for broadcast MBS and multicast MBS sessions, or source specific IP multicast address for multicast MBS session |
| >MBS session mode | M | Indicate the service type of the MBS session, either a multicast MBS session or a broadcast MBS session |
| >MC service group ID | O | Indicate the MC service group ID associated to the MBS session |
| >MBS related SDP information | M | SDP related to application-level control signaling or media to be transmitted over the MBS session (e.g., codec, protocol ID, FEC information, IP address and ports) |
| >List of MBS Service Area information (see NOTE 5) | O | For the case of local MBS services, it indicates either multicast service area identifier(s) for multicast MBS session, or broadcast service area identifier(s) for broadcast MBS session |
| >MBS service announcement acknowledgement | O | Indicate if the MC service server requires an acknowledgement to the MBS service announcement |
| >Multicast MBS session related information (see NOTE 1) | O | Additional information to be used by the MC service client to join the multicast MBS session such as PLMN ID of the default PLMN service provider in case of source specific IP multicast address, DNN, and SNSSAI of the PDU session associated with the multicast MBS session |
| >UE session join notification (see NOTE 2) | O | Indicate if the MC service server requires a notification from the MC service client once it has joined the multicast MBS session |
| >Monitoring state (see NOTE 3) | O | Indicate if the MC service client is required to actively monitor the broadcast MBS session quality and report it to the MC service server |
| eMBMS bearer information | O | Providing the 4G eMBMS bearer related information if MCX server decides to use 4G eMBMS additionally with 5G MBS session to deliver MC service group communication data |
| >TMGI (see NOTE 4) | M | TMGI information |
| >Alternative TMGI | O | A list of additional alternative TMGI may be included and used in roaming scenarios |
| >QCI | O | QCI information used by the ProSe UE-Network Relay to determine the ProSe Per-Packet Priority value to be applied for the multicast packets relayed to Remote UE over PC5 |
| >List of service area identifier | M | A list of service area identifier for the applicable MBMS broadcast area |
| >Frequency | O | Identification of frequency if multi carrier support is provided |
| >Unicast bearer status | O | An indication that the listening status of the LTE unicast bearer is requested |
| >eMBMS related SDP information | M | SDP with media and floor control information applicable to groups that can use this eMBMS bearer (e.g., codec, protocol id, FEC information) |
| >ROHC information | O | Indicate the usage of ROHC over the eMBMS bearer and provide the parameters of the ROHC channel to signal to the ROHC decoder |
| NOTE 1: Such information may be pre-configured in the MC service UE, or provided in any other implementation specific way NOTE 2: It is applicable for multicast MBS session NOTE 3: It is applicable for broadcast MBS session NOTE 4: TMGI for 4G eMBMS bearer can be the same or different with 5G MBS session ID. NOTE 5: Details of MBS service area information is defined in 3GPP TS 23.247 [15]. |  |  |

#### 7.3.2.2	UE session join notification

Table 7.3.2.2-1 describes the information flow of UE session joining notification from the MC service client to the MC service server after successfully joining a certain multicast MBS session procedure as defined in 3GPP TS 23.247 [15].

Table 7.3.2.2-1: UE session join notification

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID(s) | M | The identity of the broadcast MBS session(s) being monitored. It is the TMGI type. |
| MC service ID | M | Identity of the MC service user who is reporting the session status |
| MBS multicast joining status | M | The multicast listening status can be joined if successfully joined MBS session. |

* * * End of Change * * * *