33GPP TSG-SA WG6 Meeting #49-e	S6-221453

e-meeting, 16th – 22nd May 2022	(revision of S6-221270)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0330 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Migration procedure during and ongoing private communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2022-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The introduced CR provide a generic procedure to be followed by an MC service client which has detected the need to migrate to another MC system during an ongoing private communication, e.g., MCPTT and MCVideo private calls. The procedure includes both scenarios whether the MC service client migrates to its primary MC system or a partner MC system. Via this procedure the ongoing private communication is released due to migration of one private communication participant, the other participant may be informed, and the communication is re-established once the migration is over. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A guidance of the behaviour of an MC service client in the form of procedure to be followed when a client has detected the need to migrate to another MC system during an ongoing private communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Private communication participant will not be aware of the reason the private communication has dropped. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.16.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 10.16.2.X Migration during an onging private communication

##### 10.16.2.X.1 General

This subclause provides a genric guidance on the behaviour an MC service client follows to perform migration during an ongoing private communication, e.g., MCPTT private call. Once the MC service client detects the need to migrate during an ongoing private communication, it may initate preparations and UE configuration which facilitate migration as mentioned in clause 10.1.1.2. The described procedure is applicable to the scenarios whether an MC service client is migrating into its primary MC system or a partner MC system.

##### 10.16.2.X.2 Procedure

Figure 10.16.2.X.2-1 represents a generic procedure to be followed when migration is needed to be done during an ongoing private communication, such as MCPTT private call, or MCVideo private call.

Figure 10.16.2.X.2-1: Migration during ongoing private communication.

- A private call takes place between MC service client 1 and MC service client 2, where the former is connected to MC system A and the latter to MC system B.

- The MC service client 1 detects the need to migration into MC system C and MC service server 1 is notified to be prepared for possible service interruption. Furthermore, upon detection, MC service client 1 performs initial steps to be considered prior migration, e.g., UE configuration, authorization, and selection of user profile that permits migration, as described in clause 10.1.1.2

- MC service client 1 releases the private communication towards the MC service client 2 as described in clause 10.7.2.2.3.1 of 3GPP TS 23.379 [16] for MCPTT private call, or in clause 7.2.2.3.3.1 of 3GPP TS 23.281 [12] for MCVideo private call. A call release reason IE “release due to migration” may be included so that MC service client 2 is informed.

- Migration takes place and MC system C retrieves MC service client 1 profile:

4a. if MC system C is the primary MC system, retrieval of MC service client 1 profile takes place according to clause 10.1.4.3.1.

4b. if MC system C is a partner MC system, retrieval of MC service client 1 profile takes place according to clause 10.1.4.3.2.

- Registration procedure takes place:

5a. If system C is the primary MC system, the procedure takes place according to clause 10.6.1.

5b. If system C is a partner MC system, the procedure takes place according to clause 10.1.6.2.

6. 	MC service client 1 initiates a new private communication toward MC service client 2 as described in clause 10.7.2.2.1 of 3GPP TS 23.379 [16] for MCPTT private call, or in clause 7.2.2.3.1 of 3GPP TS 23.281 [12] for MCVideo private call.

# * * * * End of changes * * * *