3GPP TSG-SA WG6 Meeting #50-e	S6-222128

e-meeting, 22nd – 31st August 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0319 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Decoupling of signalling and media paths for MCPTT service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC, Nokia, Nokia Shanghai Bell, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2022-06-16 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The functional models as currently specified require that signalling and media is processed together by the MC server. For rail, which does not have this requirement and is more geared towards the automation of rail traffic, there is a requirement that data applications have very low latency requirements (<100msec, <10msec). Thus, the media transition is to be placed in the immediate vicinity of the radio, which is not necessary for the signaling. The requirements with regard to latency and packet reliability are detailed in 3GPP TS 22.289 and 3GPP TR 23.783 has taken up this fact and discussed it sufficiently. In order to meet such requirements, alternatively, verticals need to be able to manage signalling separately from the media. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR describes the minimum adjustments that are necessary to enable signalling and media to be managed separately. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements regarding very low latency application support cannot be fullfiled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******************************************1st Change******************************************

### 7.3.1	On-network functional model

Figure 7.3.1-1 shows the functional model for the application plane of the MCPTT service.

Figure 7.3.1-1: Functional model for application plane of the MCPTT service

In the model shown in figure 7.3.1-1, the following apply:

-	The MCPTT server is an instantiation of a MC service server in accordance with 3GPP TS 23.280 [16].

-	The MCPTT server is an instantiation of a GCS AS in accordance with 3GPP TS 23.468 [9].

-	The MCPTT server controls the signalling reference points and the corresponding media allowing different paths for media and signalling. The MCPTT server determines if it needs to stay in the media path based on the feature(s) involved in the MCPTT communication.

NOTE:	If the MCPTT server is not in the MCPTT communication media path, any implications regarding MC user data logging are left to implementation.

-	MCPTT-9 carries multicast floor control signalling between the floor control server of the MCPTT server and the floor participant of the MCPTT UE.

-	MCPTT-4 carries unicast floor control signalling between the floor control server of the MCPTT server and the floor participant of the MCPTT UE.

-	MCPTT-7 carries unicast media between the media distribution function of the MCPTT server and the media mixer of the MCPTT UE.

-	MCPTT-8 carries multicast media from the media distribution function of the MCPTT server to the media mixer of the MCPTT UE.

Figure 7.3.1-2 shows the relationships between the reference points of the application plane and the signalling plane.

Figure 7.3.1-2: Relationships between reference points of MCPTT application and signalling control planes

*****************************************End of Changes******************************************