3GPP TSG-SA WG6 Meeting #50-e	S6-222131

E-meeting, 22th – 31th August 2022

Source:	CATT

Title:	Pseudo-CR on Solution#4 Update

Spec:	3GPP TR 23.700-96 v0.6.0

Agenda item:	9.7

Document for:	Approval

Contact:	Liping Wu, liping.wu@cictmobile.com

1. Introduction

This contribution updates the solution #4: Location service registration based on the new archicture of KI#1.

2. Reason for Change

According to the new architecture of KI#1, the Fused Location Client has been removed and replaced by Location Management Client, and the LM-Uu for non-3GPP access interface is defined to support interaction between the target UE and Fused Location Function within Location Management Server, so the location service registration procedure between them needs to be updated accordingly.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-96 v0.6.0.

* * * First Change * * * *

7.4	Solution #4: Location service registration

7.4.1	Solution description

This solution addresses key issue #5: Initialization and configuration for fused location service.

The procedure for location service registration is illustrated in figure 7.4.1-1. This procedure is based on the fused location architecture of KI#1. The purpose of this procedure is for the Location Management Client (LMC) to register to the location services available at the Fused Location Function (FLF) which is part of Location Management Server (LMS) while ensuring the privacy of the user.

If the Multi-USIM is supported by the LMC, the LMC performs the registration procedure for each PLMN of the Multi-USIM PLMNs.

Figure 7.4.1-1: Location service registration procedure

1.	The LMC of a target UE sends location service registration request to the FLF which is part of LMS via the LM-Uu for non-3GPP access reference point, carrying the identifier of the UE (e.g. GPSI,  UUID, etc.) and non-3GPP defined access location capabilities(e.g. the available non-3GPP defined access types, the location methods, the location accuracy and latency of the non-3GPP defined access types).

To access the FLS via the LM-Uu for non-3GPP access reference point, the LMC can use any available non-3GPP defined accesses to send the location service registration request.

The LMS with FLF can be pre-configured in the UE or be discovered via the DNS query.

2.	The FLF checks authorization for the UE's request. If the FLF supports privacy checking it also performs or assists with e.g. 5GC on privacy check.

3.	The FLF, upon successful authorization and privacy check (if any), responds to the LMC with registration result and stores the UE identifierinformation and non-3GPP defined access location capabilities into the UE location context.

If the Multi-USIM is supported by the LMC, the LMC performs steps 1 to 3 to register its identifier (e.g. MSISDN) associated with each PLMN of the Multi-USIM PLMNs.

7.4.2	Solution evaluation

This solution is based on the new architecture proposed in KI#1. The location management client can provide its UE IDs and the location capabilities of the available non-3GPP defined accesses to the fused location function which is part of location management server through LM-Uu for non-3GPP access reference point, and the fused location function can generate and store this information as the UE location context, and also can use these UE location contexts in the subsequent location procedures.