3GPP TSG-SA WG6 Meeting #50-e	S6-222134

E-meeting, 22th – 31th August 2022

Source:	CATT

Title:	Pseudo-CR on Solution#7 update

Spec:	3GPP TR 23.700-96 v0.6.0

Agenda item:	9.7

Document for:	Approval

Contact:	Liping Wu, liping.wu@cictmobile.com

1. Introduction

This contribution updates the solution #7: Location QoS based location sources and positioning methods selection.

2. Reason for Change

According to the new architecture of KI#1, the Fused Location Client has been removed and replaced by Location Management Client, and the LM-Uu for non-3GPP access interface is defined to support interaction between the target UE and Fused Location Function which is part of Location Management Server, so the procedure of location QoS based location sources and positioning methods needs to be updated accordingly.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-96 v0.6.0.

* * * First Change * * * *

7.7	Solution #7: Location QoS based location sources and positioning methods selection

7.7.1	Solution description

This solution addresses key issue #2: Support of LCS QoS.

The Fused Location Function firstly needs to produce the fused location data from multiple sources based on the  requested location QoS (e.g. the requirements of the positioning accuracy, reliability and latency). Based on the requested location QoS, the FLF needs to select one or more access types, one or more location methods (as described in TS 29.572 [14] ) and related CP/UP(SUPL) methods based on the requested location QoS (not exhausted):

-	2G/3G/4G/5G/NR satellite access

-	Non-3GPP access connected to 5GC

-	GNSS (e.g. GPS, Galilieo, BeiDou etc.)

-	Barometric Pressure

-	WLAN

-	Bluetooth

-	Terrestrial Beacon System (TBS) positioning based on MBS signals

-	Motion Sensor

-	RFID

-	Radio finger-print

-	Cell ID

-	ECID

-	OTDOA

-	DL_TDOA

-	DL_AOD

-	Multi-RTT

-	NR_ECID

-	UL_TDOA

-	UL_AOA

-	Ultra Wide Band (UWB)

-	Fingerprint

7.7.1.1	Procedure of location QoS based location sources and positioning methods selection

Figure 7.7.1.1-1: location sources and positioning methods selection

1.	The application-specific server sends a location request to the Fused Location Function(FLF) within the Location Management Server(LMS) to request the location information of the target Location Management Client(LMC) (e.g. UE Identity, location QoS, etc). The location QoS can include the location accuracy, reliability and latency,etc. as described in clause 4.1b of TS23.273[4].

2.	The FLF queries the UE location context (e.g. in the internal database) with the location QoS to retrieve the available access type, positioning methods as described in TS 29.572 [14] for the targetLMC.

3.	The FLF selects the available access type (i.e. different location sources) and positioning methods to get the UE location information from these available access types.

4.	The FLF fuses the UE location information from different sources to get a fused UE location that meets the location QoS requirements.

5.	The LMS enhanced by  FLF provides the fused UE location that meets the location QoS requirements to the application-specific server.

7.7.2	Solution evaluation

This solution is based on the architecture proposed in new Solution of KI#1. Compared to existing On-demand usage of location information in SEAL LM (TS 23.434[13], clause 9.3.9), the FLF which is part of LMS can aggregate/fuse location data from more data sources so that more accurate location can be reported. Different location source can provide location information with different location QoS, the FLF takes the different advantages of these different location sources and decides to select which location sources based on the required location QoS and fuses the location information from the selected location sources to produce the final location information to meet the location QoS.