3GPP TSG-SA WG6 Meeting #50-e	S6-222179

e-meeting, 22nd – 31st August 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0111 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Information flows and procedures to maintain notification channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-08-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | S6-221957 which was agreed during SA6-49bis-e introdcued the procedures and information flows for creating the notification channel between the SEAL notification management client and server. But the procedure to delete the notification channel is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces the information flows and procedures required to delete the notification channel. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete functionality in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.3.2.5(new), 17.3.2.6(new), 17.3.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 17.3.2.5	Delete notification channel request

Table 17.3.2.5-1 describes the information flow delete notification channel request from notification management client to the notification management server.

Table 17.3.2.5-1: Delete notification channel request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting notification management client |
| Channel ID | M | Identifies the notification channel to be deleted |

#### 17.3.2.6	Delete notification channel response

Table 17.3.2.6-1 describes the information flow delete notification channel response from notification management server to the notification management client.

Table 17.3.2.6-1: Delete notification channel response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting notification management client |
| Result | M | Indicates if the deletion of the channel is success or failure |

### 17.3.4	Procedure for deleting notification channel

Figure 17.3.4-1 below illustrates the procedure for deleting notification channel by the notification management client.

Pre-conditions:

1.	The notification management client has notification channel open with the notification management server.

Figure 17.3.4-1: Deletion of notification channel by the notification management client

1.	The VAL client decides to stop receiving notifications from the VAL server and requests the notification management client accordingly.

2.	The notification management client sends the delete notification channel request to the notification management server if the channel is currently being used only by this VAL client. If the notification channel is being used for other VAL clients the notification management client just indicates success as specified in step 4.

3.	The notification management server acknowledges the request and sends the delete notification channel response to the notification management client.

4.	The notification management client indicates to the VAL client about the status of its request to stop receiving the notification.

5.	The VAL client deletes the subscription with VAL server for receiving notifications.

* * * End Change * * * *