3GPP TSG-SA WG6 Meeting #50-e	S6-222191

e-meeting, 22nd – 31st August 2022	(revision of S6-22xxxx)

Source:	China Mobile

Title:	Update and evaluation of sol#17

Spec:	3GPP TR  23.700-99

Agenda item:	9.4

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This contribution proposes to update and add the evaluation of sol#17.

2. Reason for Change

The solution 17 was updated and the evaluation part was added.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR23.700-99.

* * * First Change * * * *

6.17	Solution 17: Multi-Network slice resource optimization

6.17.1	Solution description

6.17.1.1	General

This solution aims to address the issues identified in Key Issue#1.

This solution provides a possible procedure to illustrate the process of the VAL server requesting to manage network slice from multi-networks (e.g. the PNI-NPN network and PLMN network) through NSCE server. This solution enables the trusted third-party AF to manage its own PNI-NPN (public network integrated non-public network) and its private slice(s) in the PLMN in a combined manner as specified in clause 6.10.2 of 3GPP TS 22.261 [7]. In the case that communication services of a PNI-NPN is extended through a PLMN (e.g. the service is supported by a slice in the PNI-NPN and a slice in the PLMN) as described in clause 6.1.2.4 of 3GPP TS 22.261 [7], the NSCE server can monitor the slice usage status of multiple networks and make resource adjustment between different networks to realize optimized and efficient resource usage among slices of multiple networks.

6.17.1.2	Multi-Network slice management capability

Figure 6.17.1.2-1 illustrates the slice management process to address the key issue 1 described in clause 5.1.

Pre-conditions:

1.	The network slice enabler layer is capable to interact with both PNI-NPN and PLMN 5GC or OAM system.

2.	PNI-NPNs are deployed as network slices of the PLMN.

Figure 6.17.1.2-1 Network slice application resource optimization process

1.	The VAL server initiates network slice resource optimization request towards the NSCE server. The request includes VAL server ID, application ID, Network ID. The message also includes preferred optimization zone.

2.	Upon receiving the request from the VAL server to make the network slice resource optimization, the NSCE server makes authentication and authorization of the VAL server and if VAL server is not authorized, the NSCE server replies with failure response.

3.	The NSCE server makes mapping from application ID that received from VAL server to slice identities (S-NSSAIs allocated in multiple networks) and retrieves the network slice related status information from 5GC or OAM of PNI-NPN networks, such as NF(s) load in 5GC and network utilization in access network as defined in TS 28.535 [11].

4.	The NSCE server retrieves the network slice related status information from PLMN networks from 5GC or OAM of PLMN networks, such as NF(s) load in 5GC and network utilization in access network as defined in TS 28.535 [11]. The PLMN operator can choose to deploy NSCE-Server acting as single entry of PLMN capability exposure which can be optional.

5.	The NSCE server verifies and analyses status data of network slice instance that received from both PNI-NPN and PLMN networks as well as network slice QoS monitoring response (optional), then NSCE server makes resource optimization decision among different kinds of networks in specific location zone.

6-7.	The NSCE server determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s), such as modifyNsi request as specified in TS 28.531 [5] to PNI-NPN network. Based on decision made by NSCE server, the network slice management entity (such as NSMF) performs the corresponding operation(s).

8-9.	The NSCE server determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s), such as modifyNsi request as specified in TS 28.531 [5] to PLMN network. Based on decision made by NSCE server, the network slice management entity (such as NSMF) performs the corresponding operation(s). The PLMN operator can choose to deploy NSCE-S acting as single entry of PLMN capability exposure which can be optional.

10.	The NSCE server sends the network slice resource optimization response towards VAL server.

6.17.2	Solution evaluation

This solution addresses the KI #1 by providing a network slice resource optimization procedure to enable the NSCE server making multiple networks resource adjustment by utilizing the network slice related management services. This solution has no impact on the 5GS architecture, and it may require the interactions between NSCE servers to retrieves the network slice related status information.

The APIs over NSCE-S interface to be defined in normative phase:

1.	APIs of network slice resource optimization request and response.

* * * Next Change * * * *

<Proposed change in revision marks>