3GPP TSG-SA WG6 Meeting #50-e meeting	S6-222199

22nd – 31st August 2022, Online

Source:	ETRI, Uangel

Title:	Update to Solution #32 for resolving editor’s notes

Spec:	3GPP TR 23.700-98

Agenda item:	9.8

Document for:	Approval

Contact:	Seung-Ik Lee <seungiklee@etri.re.kr>

1	Introduction

The Solution #32 addresses "Key issue #9: Enhancement of dynamic EAS instantiation triggering" for efficient utilization of EDN resources for EAS deployment using 1) indication of a need for EAS instantiation to the ECSP management system; and 2) the EAS discovery notification for informing EEC of the result of the EAS instantiation with the EAS profile.

In the Solution #32, there are two editor’s notes as follows:

- Editor's note: Further considerations for the other EAS discovery events (e.g., T-EAS discovery by S-EES and S-EAS; or EAS Service API discovery as studied in KI#2) are FFS.

- Editor's Note: Whether a new MnS API is needed needs to be consulted with SA5.

This paper addresses the editor’s notes above.

2	Discussion

- Editor's note: Further considerations for the other EAS discovery events (e.g., T-EAS discovery by S-EES and S-EAS; or EAS Service API discovery as studied in KI#2) are FFS.

- Observations:

  - This solution relies on the EAS discovery subscribe-notify procedures. However, TS 23.558 does not provide any subscribe-notify operations in the T-EAS discovery procedures triggered by S-EES or S-EAS. Thus, such events would not be applied to this solution.

  - The Solution #8 of TR 23.700-98 addresses how to support for an EAS to expose its Service APIs towards the other EASs. In the Solution #8, CAPIF may be exploited to publish and discover EAS Service APIs. CAPIF also provides subscription and notification procedures for CAPIF events including new API available event so that the same approach of the Solution #32 can be applied to the EAS Service API discovery cases of the Solution #8.

- Proposals:

  - To delete the EN;

  - To provide some text for EAS Service API discovery to indicate that the same procedure can be applied to the EAS Service API discovery cases.

- Editor's Note: Whether a new MnS API is needed needs to be consulted with SA5.

- Observations:

  - As clarified in the S6-222015 (Reply LS on FS_eEDGEAPP, Solution for Dynamic EAS instantiation), “SA5 currently do not have mechanism for EES to trigger/request EAS instantiation yet. SA5 is working the definition of related solutions as part of eECM WI.”.

  - It’s clear that there is no such an MnS API provided by SA5 at this moment. While an MnS API for that purpose will be provided by SA5, there is no clear view to develop a new MnS API for that purpose, e.g., re-use the existing MnS API with a relevant mechanism.

- Proposals:

  - To delete the EN;

  - To modify the relevant text as: “This solution also relies on an MnS API for indication of a need for EAS instantiation, which shall be provided by the ECSP management system by consulting with SA5.”

3		Proposal

It is proposed to modify the text of TR 23.700-98 as follows.

START OF CHANGES

## 7.32	Solution #32: Dynamic EAS instantiation triggering and notification

### 7.32.1	Architecture enhancements

None.

### 7.32.2	Solution description

#### 7.32.2.1	General

This solution addresses the Key issue #9: Enhancement of dynamic EAS instantiation triggering for efficient utilization of EDN resources for EAS deployment.

As specified in TS 23.558 (Rel-17), EES may trigger the EAS instantiation dynamically if there is no instantiated EAS that matches the requesting service characteristics during EAS discovery.

By collecting one or more of these triggering input events, EES may determine if there is a need for EAS instantiation based on the pre-configured information about instantiable EASs with further considering the requesting service characteristics (e.g., location, latency) by EEC or service load/capacity (e.g., number of service sessions) of EAS. If such a need for EAS instantiation determined, EES may sends a report for a need of the EAS instantiation to the ECSP management system (which is specified in TS 28.538 [x]) to consider instantiating the target EAS that is determined to instantiate by invoking an MnS API of the ECSP management system.

When the target EAS has been instantiated, the EES may obtain the EAS profile based on the updated configuration information by the ECSP management system. Then EES may further notify the instantiation result of the target EAS with the EAS profile to the corresponding EECs in order to inform the EECs of the availability of the EAS instance.

In this solution, the EAS discovery subscribe-notify procedures are re-used as specified in the clause of 8.5.2.3 of TS 23.558 [2]. EEC may request EES for subscribing to EAS discovery. When the EES gets informed of the EAS instantiation, the EES notifies the EECs whose EAS discovery filters match with the EAS instance.

NOTE 1:	How and when a triggering is determined by EES is upon implementation and out the scope of this solution.

NOTE 2:	The pre-configured information about instantiable EASs may be provided by the ECSP management system but such a mechanism is out of the SA6 scope.

NOTE 3:	With the EAS instantiation request, EES just provides its indication for demanding instantiation of the target EAS to the ECSP management system. Any decision or further actions to the requests are up to the ECSP management system.

NOTE 4:	The MnS APIs for EAS instantiation request/notification should be provided by the ECSP management system and should be consulted with SA5.

The solution can be summarized as follows:

Dynamic EAS instantiation triggering and notification

a.	Triggering inputs events:

i.	No available EAS instances matched during EAS discovery

b.	Triggering determination:

i.	Up to EES implementation

ii.	Based on the triggering input events and pre-configured information about instantiable EASs

iii.	Further considerations:

-	EEC's requesting service characteristics (e.g., location, latency) as specified in EAS discovery filters

-	EAS's service load/capacity (e.g., number of service sessions) maintained by EES

c.	Triggering actions

i.	Invoking an MnS API of the ECSP management system for reporting a need of instantiation of a target EAS

d.	Post-triggering actions

i.	Receiving a notification for instantiation result from the ECSP management system by configuration update

ii.	Notifying the EAS instantiation to the corresponding EECs which have initiated the triggering input events for EAS availability change.

As studied in the KI#2, an EAS may expose its Service APIs towards the other EASs. In the Solution #8, CAPIF may be exploited to publish and discover EAS Service APIs. CAPIF also provides the subscription and notification procedures for CAPIF events including new API available event so that the same approach of this solution can be applied to the EAS Service API discovery cases of the Solution #8. For example, an EAS (API Invoker) may request EES (CCF) for subscribing to a new API available event for an EAS Service API. If such a need for EAS instantiation determined, the EES may sends a report for a need of the EAS instantiation to the ECSP management system. When the EES gets informed of instantiation of an EAS, the EES notifies EAS subscribers whose event criteria of new API availability match with the EAS instance.

#### 7.32.2.2	Dynamic EAS instantiation triggering and notification procedures

The Figure 7.32.2.2-1 depicts the essential operational steps for dynamic EAS instantiation triggering and notification procedures.

Pre-conditions:

1.	EES is pre-configured with the information about instantiable EASs which may be provided by the ECSP management system.

Figure 7.32.2.2-1: Dynamic EAS instantiation triggering and notification procedures

1a-1b.	EECs proceed EAS discovery subscription request with EES, which are triggering input events for dynamic EAS instantiations.

2a-2b.	EES sends an EAS discovery subscription response to the EEC.

3.	Based on the triggering input events, the EES determines if there is a need for EAS instantiation considering the requesting service characteristics provided by EAS discovery filters; or service load/capacity (e.g., number of service sessions) of EAS maintained by the EES.

4.	If such a need for EAS instantiation determined, EES sends a report for a need of the EAS instantiation to the ECSP management system for considerations to instantiate the target EAS by invoking an MnS API of the ECSP management system.

5.	The MnS of the ECSP management system responds to the EES with indicating that the requesting EAS instantiation will be considered.

6.	The ECSP management system determines if the requested EAS instantiation is acceptable by analyzing the deployment requirements and available resources; and proceeds the corresponding actions. (out the scope of this specification)

7. 	If the target EAS is instantiated successfully, the EES gets informed by an EAS registration procedure of the instantiated EAS with the EAS information (e.g., EAS profile).

8a-8b.	The EES notifies the EAS instantiation result with EAS discovery notification including the EAS profile to the relevant EECs which have already initiated the triggered input events for the target EAS instance.

### 7.32.3	Solution evaluation

This solution allows for an EES to dynamically trigger the EAS instantiation using 1) indication of a need for EAS instantiation to the ECSP management system; and 2) the EAS discovery notification for informing EEC of the result of the EAS instantiation with the EAS profile.

This solution addresses the key issue #9: enhancement of dynamic EAS instantiation triggering as specified in the clause 4.9.

This solution relies on the EDGEAPP architecture as specified in TS 23.558 [2] with extended capabilities of EES to determine if there is a need for EAS instantiation considering the requesting service characteristics by EEC or service load/capacity of EAS.

This solution also relies on an MnS API for indication of a need for EAS instantiation, which shall be provided by the ECSP management system by consulting with SA5.

END OF CHANGES