3GPP TSG-SA WG6 Meeting #50-e	S6-222337

e-meeting, 22nd – 31st Aug 2022

Title:	Reply LS to OPAG_34_Doc_07_OPAG_LS ETSI-3GPP-Network integration

Response to:	LS S6-222333

Release:	3GPP Rel-18

Work Item:	FS_eEDGEAPP / EDGEAPP_Ph2

Source:	3GPP SA6

To:	GSMA OPAG

Cc:	3GPP SA, SA2, SA3, SA5, ETSI ISG MEC, GSMA OPG

Contact person:	Basavaraj (Basu) Pattan

basavarajjp@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA6 would like to thank GSMA OPAG for the LS, and sharing the draft Operator Platform's (OP) Network Integration Support APIs definition document that includes SBI-NR and NEF NBI usage for SBI-NR.

SA6 thanks OPAG for taking into account the work of 3GPP SA6 (3GPP TS 23.558, and corresponding stage 3 work by 3GPP CT3 in 3GPP TS 29.558), namely the definition of Application Relocation API and Confirm User Location API. SA6 believes these APIs should be listed as NBI APIs (and not SBI-NR) in clause 1.1 (Table 1) and in clause 1.7, of the Operator Platform's (OP) Network Integration Support APIs definition document, since these APIs are not directly exposed by the core network, but rather at the Edge Enablement Layer (EEL) in reference to the EDGEAPP architecture, which is mapped to OP as specified in Annex D.2 of 3GPP TS 23.558.

Further, SA6 would like to bring to OPAG’s attention that:
1) Network capabilities (some of which are considered in your current document) are also available via EES, where EES may re-expose the network capabilities of the 3GPP core network to the EAS(s) via NB APIs as specified in clause 8.7.3 of 3GPP TS 23.558 with additional EEL abstraction 
2) Furthermore, you may like to consider the additional EES capabilities that are available to Edge Application Server (EAS) via NBI APIs as specified in clause 8.6 of 3GPP TS 23.558, in meeting the OP requirements.

SA6 agrees that collaboration among SDOs (3GPP, ETSI ISG MEC, and GSMA) is essential to avoid market fragmentation. As part of Release 18, SA6 is currently studying on enhancements to architecture for enabling Edge Applications in 3GPP TR 23.700-98, which includes additional NBI APIs in new R18 functionality e.g. to support federation and roaming.

Should you have any queries or further clarifications on these APIs, SA6 is happy to elaborate and provide more information.

# 2	Actions

To GSMA OPAG

ACTION: 	SA6 kindly requests GSMA OPAG to take the above information into consideration.

# 3	Dates of next TSG SA WG 6 meetings

SA6#51-e          10th Oct – 19th Oct 2022 	e-meeting

SA6#52             14th Nov – 18th Nov 2022 	Europe (TBC)