3GPP TSG-SA WG6 Meeting #50-e	S6-222373

e-meeting, 22nd – 31st August 2022	(revision of S6-222207)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0065 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | security credentials IE in Application Server Registration aligned with 33501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2022-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The security requirements including Application Server Registration/De-registration over MSGin5G-3 is specified in Annex Y.4 of TS33.501. The procedure is performed by both Application Server and MSGin5G Server. So the step 2 in figure 8.2.5.1 and step 3 in figure 8.2.6.1 is revised. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction on clause 8.2.5 and 8.2.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Application Server Registration procedure is wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5 and 8.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: The reference of TS33.501 is moved to NOTE |  |  |  |  |  |  |  |  |

*****************Change 1************************

### 8.2.5	Application Server Registration

The signalling flow for Application Server registration is illustrated in figure 8.2.5-1. Application Server may use the procedure in this clause to do registration. The procedure assumes that the Application Server is responsible for triggering registration to the MSGin5G Server in order to establish association with the MSGin5G Server to receive MSGin5G Service.

NOTE:	The procedure in this clause is optional for Application Server registration.

Pre-conditions:

1.	The Application Server has connected to the serving network successfully.

2.	An AS Service ID has been provisioned.

3.	The MSGin5G Server address has been provisioned on the Application Server.

4.	Both the Application Server and MSGin5G Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.2.5-1: Application Server registration

1.	The Application Server sends an Application Server registration request to the MSGin5G Server. The request includes security credentials required for the Application Server to register to the MSGin5G Server.  The request includes the AS Service ID and Application Server Profile information as detailed in Table 9.1.2.3-1.

2.	Upon receiving the request, the MSGin5G Server validates the Application Server registration request and verifies the security credentials.

NOTE: The authentication procedures in step 2 are built on top of the transport layer mechanism specified in Annex Y.4 of 3GPP TS 33.501 [16].

3.	The MSGin5G Server sends an Application Server registration response to the Application Server, the response includes the information elements as specified in Table 9.1.2.4-1. If the registration is successful, the MSGin5G Server stores the AS Profile information as detailed in Table 9.1.2.3-1.

### 8.2.6	Application Server De-registration

By de-registering, the Application Server informs the MSGin5G Server that it wishes to terminate its association with the MSGin5G Server.

The procedure assumes that the Application Server is responsible for triggering the de-registration from the MSGin5G Server. The signalling flow for Application Server de-registration is illustrated in figure 8.2.6-1.

Pre-conditions:

1.	The Application Server is registered to the MSGin5G Server.

Figure 8.2.6-1: Application Server de-registration

1.	The Application Server determines to de-register from the MSGin5G Server.

2.	The Application Server sends an Application Server de-registration request to the MSGin5G Server that includes the AS Service ID, as detailed in Table 9.1.2.5-1.

3.	The MSGin5G Server validates the Application Server de-registration request and verifies the security credentials. The MSGin5G Server deletes any applicable AS Profile information that it has stored.

NOTE: The authentication procedures in step 3 are built on top of the transport layer mechanism specified in Annex Y.4 of 3GPP TS 33.501 [16].

4.	The MSGin5G Server replies an Application Server de-registration response as detailed in Table 9.1.2.6-1.