3GPP TSG-SA WG6 Meeting #50-e	S6-222434

e-meeting, 22nd – 31st August 2022	(revision of S6-222155)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0090 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting MBS session ID types |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-08-22 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As described in clause 6.5.1 in 3GPP TS 23.247, the MC session ID may have two types: TMGI (for broadcast and multicast MBS sessions), and source specific IP multicast address (for multicast MBS sessions only). This has been correctly captured for example in Table 7.3.2.1-1 MBS session announcement. However, in other clauses, it is wrongly assumed that the MBS session ID is of TMGI type only. Therefore, this CR corrects this concept where needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the information element related to MBS session ID and its description in clauses 7.3.2.2, 7.3.2.10, 7.3.2.13, and 7.3.2.14. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is confusion related to the MBS session types throughout the document, and the content is not aligned with 3GPP TS 23.247. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.2, 7.3.2.10, 7.3.2.13, and 7.3.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.3.2.2	UE session join notification

Table 7.3.2.2-1 describes the information flow of UE session joining notification from the MC service client to the MC service server after successfully joining a certain multicast MBS session procedure as defined in 3GPP TS 23.247 [15].

Table 7.3.2.2-1: UE session join notification

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID(s) | M | The identity of the multicast MBS session(s) being joined. It is either TMGI or source specific IP multicast address. |
| MC service ID | M | Identity of the MC service user who is reporting the session status |
| MBS multicast joining status | M | The multicast listening status can be joined if successfully joined MBS session. |

# * * * * Second change * * * *

#### 7.3.2.10	MBS listening status report

Table 7.3.2.10-1 describes the information flow of MBS listening status report from the MC service client to the MC service server. The MBS listening status report is applicable to both broadcast and multicast MBS sessions.

Table 7.3.2.10-1: MBS listening status report

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID(s) | M | The identity of the MBS session(s) being monitored. |
| MC service ID | M | Identity of the MC service user who is reporting the session status. It may either be the MCPTT ID, MCVideo ID, or MCData ID. |
| MBS listening status | M | The listening status per MBS session ID |
| MBS reception quality level | O | The reception quality level |
| Unicast listening status | O | The unicast listening status associated with the unicast delivery. |
| NOTE: The set of quality levels helps service continuity in broadcast and multicast scenarios. A reception quality level may help to make an efficient switching decision to unicast delivery. How these levels are used is implementation specific. |  |  |

# * * * * Third change * * * *

#### 7.3.2.13	Media distribution request

Table 7.3.2.13-1 describes the information flow media distribution request from the MC Service server to an MC service server (5G MBS Session control role) that has a desired 5G MBS session.

The difference compared with the information flow defined in 3GPP TS 23.280 [3] is that in 5G MBS, the information element for QoS is 5QI.

Table 7.3.2.13-1: Media distribution request

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID | M | MBS session identifier |
| Bandwidth | M | Maximum bandwidth required |
| Separate floor control | M | Whether or not a separate session is required for floor control |
| SDP information | M | SDP with media and floor control information applicable to groups that can use this MBS session (e.g. codec, protocol id) |
| 5QI | O | Desired 5QI |
| MC Group ID | O | The MC group id for when the request is sent for a specific group call |

# * * * * Fourth change * * * *

#### 7.3.2.14	Media distribution response

Table 7.3.2.14-1 describes the information flow media distribution response from an MC Service server (5G MBS session control role) that has a desired 5G MBS session to the MC service server.

Table 7.3.2.14-1: Media distribution response

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID | M | MBS session identifier |
| Bandwidth | M | Maximum bandwidth required |
| SDP information | M | SDP with media and floor control information applicable to groups that can use this session (e.g. codec, protocol id) |
| 5QI | O | Actual 5QI |
| Media stream identifier | O | This element identifies the media stream of the SDP used for the group call |
| Media distribution indicator | O | Indicates to the MC service server whether the media in the ongoing group communication should be sent or not |

# * * * * End of changes * * * *