3GPP TSG-SA WG6 Meeting #50-e	S6-222446

E-meeting, 22th – 31th August 2022	  (revision of S6-222130)

Source:	CATT

Title:	Pseudo-CR on Solution#2 Update

Spec:	3GPP TR 23.700-96 v0.6.0

Agenda item:	9.7

Document for:	Approval

Contact:	Liping Wu, liping.wu@cictmobile.com

1. Introduction

This contribution updates solution#2 Support of both LCS and SUPL at Fused Location Server.

2. Reason for Change

According to the new architecture of KI#1, the Fused Location Function has became the part of Location Management Server and the reference point between the Fused Location Function and the 3rd Party Location Server has been changed, so the related contents in solution#2 should be updated accordingly.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-96 v0.6.0.

* * * First Change * * * *

7.2	Solution #2: Support of both LCS and SUPL at Fused Location Function

7.2.1	Solution description

This solution addresses key issue #1 in respect of application layer support of both LCS and SUPL.

7.2.1.1	Architectural models

According to OMA AD SUPL [10], Secure User Plane Location (SUPL) is an Enabler which utilizes existing standards where available and possible, to transfer assistance data and positioning data over a User Plane bearer, such as IP, to aid network and SUPL Enabled Terminal (SET) based positioning technologies in the calculation of a SET's position.

When MNO deploys both LCS and SUPL, the 3GPP TS 23.271 [3] defines the operation of SUPL in EPC, in which the location of SUPL can be retrived from GMLC. The potential interworking of control plane LCS and OMA SUPL both deployed by an MNO is outside the scope of that specification. For 5G LCS the support for SUPL is not defined in 3GPP TS 23.273 [4]. The 3GPP TS 38.305 [6] Annex A (informative) provides an architecture for interworking between LCS and OMA SUPL, in which the location of SUPL is directly exposed from SLP without the involvement of core network entities.

Considering the existing specifications regarding architecture aspects of SUPL, the potential architectural models of application layer consisting the location source of LCS and SUPL are shown in below figure .

Figure 7.2.1.1-1: Possible Fused Location Function architectural models consisting location source of LCS and SUPL

The potential architectural models includes:

a)	LCS and SUPL locations are exposed by 5GC (from GMLC) through the same interface (Le) to the LCS client which is within the MNO domain.

b)	LCS and SUPL locations are exposed by 5GC (from GMLC) and OMA SLP separately, the LCS client and SUPL agent are within the MNO domain.

NOTE:     The SLP can belong to the operators’ management and also can under the 3rd party location server’s control. The LMS retrieves SUPL from SLP within the 3rd party location server via LM-3P refence point if the SLP belongs to the 3rd party location server.

c)	LCS and SUPL locations are exposed by 5GC (from NEF) and OMA SLP separately to the AF and SUPL agent that are external to the MNO domain.

7.2.2	Solution evaluation

This solution is based on the architecture in the new solution of KI#1 and defines how to fuse the location information from different sources via different interfaces and the possible architecture models.

* * * End Change * * * *