3GPP TSG-SA WG6 Meeting #50-e	S6-222502

e-meeting, 22nd – 31st August 2022	(revision of S6-222316)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0121 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Edge Notification Service - solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_PH2 |  |  |  |  |  | Date: |  |  | 2022-08-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The EEC can be notified with the updated information for edge computing service by EES and ECS over EDGE-1 and EDGE-4, respectively. The notification services available to the EEC (i.e., service provisioning notification, EAS discovery notification, ACR information notification) are specified in release 17, but the details on the notification mechanism is not addressed. In this regard, additional ways (e.g. push notification mechanism and application triggering specified in 3GPP TS 23.501 [5], etc.) of providing the updates to the EEC in real time need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposal is to leverage SEAL notification management service mechanism for delivering the EEL Notifications to EEC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Solution for EEL Notifications to EEC will remain under specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3.3.4, 8.3.3.3.6, 9.1(new), 9.1.1(new), 9.1.1.1(new), 9.1.1.2(new), 9.1.1.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * OPTION 2 – First Change * * * *

##### 8.3.3.3.4	Service provisioning subscription request

Table 8.3.3.3.4-1 describes the information elements for service provisioning subscription request from the EEC to the ECS.

Table 8.3.3.3.4-1: Service provisioning subscription request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Notification Target Address (NOTE) | O | The Notification Target Address (e.g. URL) where the notifications destined for the EEC should be sent to. |
| AC profile(s) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity information | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| NOTE : This IE is same as Callback URL in SEAL notification management service. |  |  |

* * * OPTION 2 – Second Change * * * *

##### 8.3.3.3.6	Service provisioning notification

Table 8.3.3.3.6-1 describes the information elements for service provisioning notification from the ECS to the EEC.

Table 8.3.3.3.6-1: Service provisioning notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the ECS for the request |
| List of EDN configuration information | M | List of EDN configuration information as defined in Table 8.3.3.3.3-2. |

NOTE:	Service provisioning notification is the Notification data in SEAL Notification message of clause 17.3.2.4.

Editor’s note:	Similar updates to other Subscription and Notification messages is FFS.

* * * OPTION 2 – Third Change * * * *

## 9.1	Usage of SEAL services

### 9.1.1	Notification management service

#### 9.1.1.1	General

The notification management is a SEAL service that offers the notification functionality. This service enables EEC to subscribe and receive notifications from the EES and ECS, and thereby offloading the complexity of delivery and reception of notifications to the edge enabler layer.

#### 9.1.1.2	Information flows

The following information flows of notification management service of SEAL as specified in 3GPP TS 23.434 [13] are applicable for the EEL:

-	Create notification channel request specified in subclause 17.3.2.1;

-	Create notification channel response specified in subclause 17.3.2.2;

-	Open notification channel specified in subclause 17.3.2.3;

-	Notification message specified in subclause 17.3.2.4.

The usage of the above information flows are clarified as below:

-	The Callback URL is the address (e.g. Notification Target Address) where the notifications destined for the EEC;

-	VAL Application ID is EECID;

-	VAL Service ID is ACID;

-	VAL client is the EEC;

-	VAL server is the EES or ECS.

#### 9.1.1.3	Procedures

The following procedures of notification management service of SEAL as specified in 3GPP TS 23.434 [13] are applicable for the edge enabler layer:

-	Procedure for creating notification channel to receive notifications, specified in clause 17.3.3.

* * * OPTION 2 – End of changes * * * *