3GPP TSG-SA WG6 Meeting #50	S6-222507

e-meeting, 22nd– 31st August 2022	(revision of 2246)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0115 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on location management information flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Client-triggered or VAL server-triggered location reporting procedure, location reporting triggers configuration cancel procedure and event-trigger location information notification procedure allows both LM client and VAL server as requestor to obtain or cancel location information related to another LM client. However, the identity of the VAL server is not included as the requestor identity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding VAL server ID in identity information element of location reporting trigger, location information notification and location reporting configuration cancel request information flow. When VAL server initiates event-trigger location reporting procedure, it is considered as a location information subscription. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect information flow of location reporting trigger, location information notification and location reporting configuration cancel request. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.4, 9.2.3.7, 9.2.3.8, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.2.4	Location reporting trigger

Table 9.3.2.4-1 describes the information flow from the location management client or VAL server to the location management server for triggering a location reporting procedure.

Table 9.3.2.4-1: Location reporting trigger

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M (see NOTE 1) | Identity of the requesting authorized VAL user or VAL UE or VAL server |
| Identity | M (see NOTE 1) | Identity of the requested VAL user or VAL UE |
| VAL service ID | O | Identity of the VAL service for which the location reporting trigger is set. |
| Immediate Report Indicator | O (see NOTE 2) | Indicates whether an immediate location report is required |
| Requested location information | O (see NOTE 2) | Identifies what location information is requested |
| Triggering criteria | O (see NOTE 2) | Identifies when the requesting client/VAL server is expecting to receive the location report from the requested VAL user/ VAL UE |
| Minimum time between consecutive reports | O see (NOTE 2) | Defaults to 0 if absent otherwise indicates the interval time between consecutive reports |
| Endpoint information | O | Information of the endpoint of the requesting VAL server to which the location report notification has to be sent. It is provided if Immediate Report Indicator is set to required. |
| NOTE 1: The identity of the requesting VAL user/UE/VAL server and the requested VAL user/UE should belong to the same VAL service. NOTE 2: At least one of these rows shall be present. |  |  |

* * * Second Change * * * *

#### 9.3.2.7	Location information notification

Table 9.3.2.7-1 describes the information flow from the location management server to the VAL server or the location management client.

Table 9.3.2.7-1: Location information notification

| Information element | Status | Description |
| --- | --- | --- |
| Identities list | M | List of the VAL users or VAL UEs whose location information needs to be notified |
| Identity (see NOTE 2) | O | Identity of the VAL user or VAL UE subscribed to location of another VAL user or VAL UE (NOTE 1) |
| Subscription ID (see NOTE 2) | O | Subscription identity related to VAL server subscription with Location management server for location information notification. |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information | M | Location information |
| Timestamp | O | Timestamp of the location report |
| NOTE 1: This is only used for location management server sends location information notification to the VAL user or VAL UE who has subscribed the location. NOTE 2: Either Identity or Subscription ID shall be included. |  |  |

* * * Third Change * * * *

#### 9.3.2.8	Location reporting configuration cancel request

Table 9.3.2.8-1 describes the location reporting configuration cancel request information flow from the location management client or VAL server to the location management server.

Table 9.3.2.8-1: Location reporting configuration cancel request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting authorized VAL user or VAL UE or VAL server |
| Identity | M | Identity of the requested VAL user or VAL UE |
| VAL service ID | O | Identity of the VAL service for which the location reporting configuration is requested to be canceled. |

* * * Fourth Change * * * *

### 9.3.5	Client-triggered or VAL server-triggered location reporting procedure

Figure 9.3.5-1 illustrates the high level procedure of client-triggered or VAL server-triggered location reporting.

Figure 9.3.5-1: Client-triggered location reporting procedure

1.	Location management client 2 (authorized VAL user or VAL UE) or VAL server sends a location reporting trigger to the location management server to activate a location reporting procedure for obtaining the location information of location management client 1. If VAL server initiates an event-triggered location reporting procedure, then the location reporting trigger is considered as a location information subscription as specified in clause 9.3.7.

NOTE:	Step 1 can be performed when Location management client 2 or VAL server require to update the location reporting trigger.

2.	Location management server checks whether location management client 2 or VAL server is authorized to send a location reporting trigger. Depending on the information specified by the location reporting trigger, location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1.

3.	Once the location information of the location management client 1 is available in the location management server by the on-demand location reporting procedure, a location information report is sent to the location management client 2 or VAL server.

* * * End of Changes * * * *