3GPP TSG-SA WG6 Meeting #50-e	S6-222509

e-meeting, 22nd – 31st August 2022	(revision of S6-222249)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0118 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the NRM functional model to support 5MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S6 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022.08.08 |
| Category: | B |  |  |  |  |  | Release: |  |  | R18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | One-to-many transmission is an important requirements for many verticals, like V2X. In 5G, the MBS is designed and completed in R17 to support the one-to-many transmission at the network layer. However, the 5G MBS is not supported by the SEAL to facilitate the verticals to take advantage of this feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the NRM functional model and related descriptions to support 5G MBS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Verticals cannot use 5G MBS via the SEAL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.2.1, 14.2.5.aa (new), 14.2.5.bb (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /********************* First Change *********************/

#### 14.2.2.1	Generic on-network functional model for network resource management

Figure 14.2.2-1 illustrates the generic on-network functional model for network resource management.

Figure 14.2.2.1-1: On-network functional model for network resource management

The network resource management client communicates with the network resource management server over the NRM-UU reference point. The network resource management client provides the support for network resource management functions to the VAL client(s) over NRM-C reference point. The VAL server(s) communicate with the network resource management server over the NRM-S reference point.

The network resource management server obtains and controls the multicast/broadcast resource from the underlaying 3GPP network system by communicating with the BM-SC via MB2-C and xMB-C reference points towards LTE eMBMSand with the 5G MBS control plane functions via Nmb13, Nmb10 or N33 towards the 5GS. The network resource management server communicates with the PCRF via Rx reference point or communicates with the PCF via N5 reference point to control the unicast resources from the underlying 3GPP network system. The network resource management server communicates with the SCEF via T8 reference point or communicates with the NEF via N33 reference point to perform event monitoring procedures from the underlying 3GPP network system. The network resource management server interacts with NEF via N33 to obtain QoS monitoring information from the 5GS.

# /********************* Next Change *********************/

#### 14.2.5.aa	Nmb13

The reference point Nmb13 supports the interactions between the network resource management server and the MB-SMF is specified in 3GPP TS 23.247 [xx].

#### 14.2.5.bb	Nmb10

The reference point Nmb10 supports the interactions between the network resource management server and the MBSF is specified in 3GPP TS 23.247 [xx].

#### 14.2.5.dd	N6mb

The reference point N6mb supports the interactions between the VAL server and the MB-UPF is specified in 3GPP TS 23.247 [xx].

#### 14.2.5.ee	Nmb8

The reference point Nmb8 supports the interactions between the VAL server and the MBSTF is specified in 3GPP TS 23.247 [xx].

#### 14.2.5.ff	N6

The reference point N6 supports the interactions between the VAL server and the UPF is specified in 3GPP TS 23.501 [xx].

# /********************* End of Changes *********************/