3GPP TSG-SA WG6 Meeting #50-e	S6-222091

e-meeting, 22nd – 31st August 2022	(revision of S6-22xxxx)

Source:	Ericsson

Title:	ACR scenario selection in DN

Spec:	3GPP TR 23.700-98 v1.1.1

Agenda item:	9.8

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution provides a solution for KI#19.

2. Reason for Change

The selection of ACR scenarios made by EES was mentioned in sol#19 and selection of ACR scenario made by EEC was mentioned in sol#35. They are suitable in EEC triggered initial EAS discovery and EEC triggered T-EAS discovery during ACR.

There is no solution provided for the DN side triggered T-EAS discovery during ACR, where S-EAS and S-EES communicate with each other to determine T-EAS.

During the T-EAS discovery, the T-EAS service continuity support capability (e.g. supported ACR scenarios) is considered together with the service contitnuity support capability of AC, EEC and EES. Then ACR scenario selection can be further decided between S-EAS and S-EES and final result needs to be notified to the EEC and T-EAS.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes in TR 23.700-98.

Rev1: removed ACR scenario description and focus on interaction triggered by EAS.

Rev2: further simplify the diagram w/o T-EAS/EES.

Rev3: update baesd on HW’s suggestion.

* * * First Change * * * *

## 7.X	Solution #X: ACR scenario selection in DN

### 7.X.1	Architecture enhancements

None.

### 7.X.2	Solution description

#### 7.X.2.1	General

This solution provides options for the EAS or EES to make ACR scenario selection.

#### 7.X.2.2	EAS decided ACR scenario selection

Figure 7.X.2.2-1: EAS decided ACR scenario selection

In Figure 7.X.2.2-1, in step 1 the EAS decides the ACR scenario to be used in the ACR based on the supported ACR scenarios by the serving EAS, the serving EES and EEC and AC.

Editor’s note: How the EAS obtains the information about supported ACR scenarios by EES, EEC and AC is FFS.

The EAS provides the selected ACR scenario list in the request to the EES in step 2.

Editor’s note: it is FFS if this request can re-use selected target EAS declaration.

The EES responds the EAS in step 3 and further notifies EEC in step 4. The EEC stores the selected ACR scenarios to be used. The EES also stores the selected ACR scenarios to be used for the ACR.

#### 7.X.2.2	EES decided ACR scenario selection triggered by EAS

Figure 7.X.2.2.-1: EES decided ACR scenario selection

In Figure 7.X.2.2-1, in step 1 the EAS requests the EES to select the ACR scenario to be used in the ACR.

The EES, based on the supported ACR scenarios by the serving EAS (as indicated by the EAS in step 1), the supported ACR scenarios by the serving EES, the supported ACR scenarios by the AC and EEC, selects ACR scenarios in step 2 and responds the EAS in step 3.

The EES further notifies EEC in step 4. The EEC stores the selected ACR scenarios to be used.

### 7.X.3	Solution evaluation

### * * * End of Changes * * * *