3GPP TSG-SA WG6 Meeting #51-e	S6-222696

e-meeting, 10th – 19th October 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0072 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MSGin5G UE bulk de-registration over MSGin5G-6 reference point |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Only MSGin5G UE bulk registration over MSGin5G-6 reference point is specified in clause 8.2.6, the bulk de-registration procedure has not be specified. This CR proposes this procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MSGin5G UE bulk de-registration over MSGin5G-6 reference point |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MSGin5G UE bulk de-registration over MSGin5G-6 reference point is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.7, 8.2.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*****************Change 1************************

8.2.7	MSGin5G UE bulk registration over MSGin5G-6 reference point

When constrained devices UE-2a and UE-2b, which support an MSGin5G Client, perform registration via MSGin5G UE-1 to contact the MSGin5G Server, the MSGin5G UE-1 may decide to use bulk registration procedure specified in this clause based on the registration request sent from MSGin5G Clients in MSGin5G UE-2a and MSGin5G UE-2b, if allowed by service policy. The procedure for MSGin5G UE bulk registration is illustrated in figure 8.2.7-1.

Pre-conditions:

1.	The MSGin5G UE-1 has registered to the MSGin5G Server successfully.

2.	MSGin5G UE-2a and MSGin5G UE-2b have discovered and selected MSGin5G UE-1 as the MSGin5G Proxy UE as specified in clause 8.2.8 and connected to the serving network via MSGin5G UE-1 successfully.

3.	UE Service IDs and the MSGin5G Server addresses have been configured on the MSGin5G UE-1, MSGin5G UE-2a and MSGin5G UE-2b. All these MSGin5G UEs are served by a same MSGin5G Server.

4.	Both the MSGin5G UEs and MSGin5G Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.2.7-1: MSGin5G Client bulk registration

1.	The MSGin5G Client-2a sends an MSGin5G UE registration request to the MSGin5G Server as specified in clause 8.2.1, In addition to the Information Elements specified in table 8.2.1-1, the MSGin5G Client-2a may also add the Information Elements specified in table 8.2.7-1 in the MSGin5G UE registration request.

Table 8.2.7-1: MSGin5G UE registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration urgent degree | O | Indicates whether the registration request is urgent or not. If a MSGin5G UE registration request is indicated as urgent, the MSGin5G UE registration request cannot be bulk-handled. |
| Registration request expiration time | O | Indicates the maximum processing time of the registration request allowed by the MSGin5G Client-2. |

2.	Upon receiving the MSGin5G UE registration request, the MSGin5G Client-1 checks whether this request can be bulked based on the service policy and the information included in the MSGin5G UE registration request, e.g.

a)	whether the MSGin5G UE registration request is urgent; or

b)	whether bulk registration is supported by the MSGin5G service provider;

NOTE 1:	Whether bulk registration is supported can be configured in the MSGin5G Client-1.

NOTE 2:	The bulk registration conditions are specified by application provider or MSGin5G Service provider and are out of scope of this document.

3.	Based on step 2:

a)	if the MSGin5G UE registration request cannot be bulked, MSGin5G Client-1 forwards the registration request to MSGin5G Server; or

b)	if the MSGin5G UE registration request can be bulked, MSGin5G Client-1 sends a bulk registration notification to MSGin5G Client-2 and caches/stores the MSGin5G UE registration request until the bulk registration conditions can be fulfilled. The Information Elements specified in table 8.2.7-2 are included in the bulk registration notification. The steps 4 and 5 are skipped.

Table 8.2.7-2: bulk registration notification

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the requesting MSGin5G UE-2. |
| Expected registration time | O | The expected time when the bulk registration can be handled and the MSGin5G UE registration Response can be received. |

4.	The MSGin5G Server handles the MSGin5G UE registration request as specified in clause 8.2.1 and sends the corresponding MSGin5G UE registration response to MSGin5G Client-2a.

5.	The MSGin5G Client-1 sends the MSGin5G UE registration response to MSGin5G Client-2a. The MSGin5G Client-2a handles the MSGin5G UE registration response as specified in clause 8.2.1 and skips all the remaining steps in this clause.

6.	MSGin5G Client-2b sends an MSGin5G UE registration request and the MSGin5G UE registration request is handed as in steps 2-5.

7.	The MSGin5G Client-1 finds that the conditions of sending a bulk registration request can be fulfilled, e.g.

a)	the registration request expiration time in the MSGin5G UE registration request sent from MSGin5G Client-2a or MSGin5G Client-2b is reached; or

b)	the periodic bulk registration interval is reached; or

c)	the maximum MSGin5G UE registration request number in a MSGin5G UE bulk registration request is reached.

NOTE 3:	The maximum registration time, periodic bulk registration interval and maximum MSGin5G UE registration request number are implementation specific and out of the scope of the current specification.

8.	The MSGin5G Client-1 includes all cached/stored MSGin5G UE registration requests in an MSGin5G UE bulk registration request and sends it to the MSGin5G Server. The Information Elements specified in table 8.2.7-3 are included in the MSGin5G UE bulk registration request.

Table 8.2.7-3: MSGin5G UE bulk registration request

| Information element | Status | Description |
| --- | --- | --- |
| Bulk registration indicator | M | Indicates that this request is an MSGin5G UE bulk registration request. |
| Number of individual MSGin5G UE registration requests | M | Indicates total number of MSGin5G UE registration requests which are bulked in this MSGin5G UE bulk registration request |
| List of individual MSGin5G UE registration request | M | Each element in this list contains information as specified in Table 8.2.1-1. |

9.	Upon receiving the MSGin5G UE bulk registration request, the MSGin5G Server splits the MSGin5G UE bulk registration request, executes the authentication, authorization and registration procedures for each MSGin5G Client-2 individually as specified in clause 8.2.1.

Editor's note:	Security aspects of bulk registration depend on SA3 and is FFS

10.	The MSGin5G Server includes all corresponding MSGin5G UE registration responses in an MSGin5G UE bulk registration response and sends it to MSGin5G Client-1. The Information Elements specified in table 8.2.7-4 are included in the MSGin5G UE bulk registration response.

Table 8.2.7-4: MSGin5G UE bulk registration response

| Information element | Status | Description |
| --- | --- | --- |
| Bulk registration response indicator | M | Indicates that this response is an MSGin5G UE bulk registration response. |
| Number of individual MSGin5G UE registration responses | M | Indicates total number of MSGin5G UE registration responses which are bulked in this MSGin5G UE bulk registration response |
| List of individual MSGin5G UE registration response | M | Each element in this list contains information as specified in Table 8.2.1-4. |

11.	Upon receiving the MSGin5G UE bulk registration response, the MSGin5G Client-1 splits the MSGin5G UE bulk registration response into multiple individual MSGin5G UE registration response, each individual MSGin5G UE registration response contains information as specified in Table 8.2.1-4.

12.	MSGin5G Client-1 sends the MSGin5G UE registration Response 2a to MSGin5G Client-2a.

13.	MSGin5G Client-1 sends the MSGin5G UE registration Response 2b to MSGin5G Client-2b.

8.2.z	MSGin5G UE bulk de-registration over MSGin5G-6 reference point

The MSGin5G UE-1 may also decide to use bulk de-registration procedure specified in this clause based on the de-registration request sent from MSGin5G Clients in MSGin5G UE-2a and MSGin5G UE-2b, if allowed by service policy. The procedure for MSGin5G UE bulk registration is illustrated in figure 8.2.z-1.

Pre-conditions:

1.	The MSGin5G UE-1 has registered to the MSGin5G Server successfully.

2.	MSGin5G UE-2a and MSGin5G UE-2b have registered to the MSGin5G Server via MSGin5G UE-1 successfully and MSGin5G UE-1 acts as MSGin5G Proxy UE as specified in clause 8.2.8.

Figure 8.2.z-1: MSGin5G Client bulk de-registration

1.	The MSGin5G Client-2a sends an MSGin5G UE de-registration request to the MSGin5G Server as specified in clause 8.2.2, In addition to the Information Elements specified in table 8.2.2-1, the MSGin5G Client-2a may also add the Information Elements specified in table 8.2.z-1 in the MSGin5G UE de-registration request.

Table 8.2.z-1: MSGin5G UE registration request

| Information element | Status | Description |
| --- | --- | --- |
| De-registration urgent degree | O | Indicates whether the de-registration request is urgent or not. If a MSGin5G UE de-registration request is indicated as urgent, the MSGin5G UE de-registration request cannot be bulk-handled. |
| De-registration request expiration time | O | Indicates the maximum processing time of the registration request allowed by the MSGin5G Client-2. |

2.	Upon receiving the MSGin5G UE de-registration request, the MSGin5G Client-1 checks whether this request can be bulked based on the service policy and the information included in the MSGin5G UE registration request, e.g.

a)	whether the MSGin5G UE de-registration request is urgent; or

b)	whether bulk de-registration is supported by the MSGin5G service provider;

NOTE 4:	Whether bulk de-registration is supported can be configured in the MSGin5G Client-1.

NOTE 5:	The bulk de-registration conditions are specified by application provider or MSGin5G Service provider and are out of scope of this document.

3.	Based on step 2:

a)	if the MSGin5G UE de-registration request cannot be bulked, MSGin5G Client-1 forwards the de-registration request to MSGin5G Server; or

b)	if the MSGin5G UE de-registration request can be bulked, MSGin5G Client-1 sends a bulk de-registration notification to MSGin5G Client-2 and caches/stores the MSGin5G UE de-registration request until the bulk de-registration conditions can be fulfilled. The Information Elements specified in table 8.2.z-2 are included in the bulk registration notification. The steps 4 and 5 are skipped.

Table 8.2.z-2: bulk registration notification

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the requesting MSGin5G UE-2. |
| Expected de-registration time | O | The expected time when the bulk de-registration can be handled and the MSGin5G UE de-registration response can be received. |

4.	The MSGin5G Server handles the MSGin5G UE de-registration request as specified in clause 8.2.2 and sends the corresponding MSGin5G UE de-registration response to MSGin5G Client-2a.

5.	The MSGin5G Client-1 sends the MSGin5G UE de-registration response to MSGin5G Client-2a. The MSGin5G Client-2a handles the MSGin5G UE de-registration response as specified in clause 8.2.2 and skips all the remaining steps in this clause.

6.	MSGin5G Client-2b sends an MSGin5G UE de-registration request and the MSGin5G UE de-registration request is handed as in steps 2-5.

7.	The MSGin5G Client-1 finds that the conditions of sending a bulk de-registration request can be fulfilled, e.g.

a)	the de-registration request expiration time in the MSGin5G UE de-registration request sent from MSGin5G Client-2a or MSGin5G Client-2b is reached; or

b)	the periodic bulk de-registration interval is reached; or

c)	the maximum MSGin5G UE de-registration request number in a MSGin5G UE bulk de-registration request is reached.

NOTE 6:	The maximum de-registration time, periodic bulk de-registration interval and maximum MSGin5G UE de-registration request number are implementation specific and out of the scope of the current specification.

8.	The MSGin5G Client-1 includes all cached/stored MSGin5G UE de-registration requests in an MSGin5G UE bulk de-registration request and sends it to the MSGin5G Server. The Information Elements specified in table 8.2.z-3 are included in the MSGin5G UE bulk de-registration request.

Table 8.2.z-3: MSGin5G UE bulk registration request

| Information element | Status | Description |
| --- | --- | --- |
| Bulk de-registration indicator | M | Indicates that this request is a MSGin5G UE bulk de-registration request. |
| Number of individual MSGin5G UE de-registration requests | M | Indicates total number of MSGin5G UE de-registration requests which are bulked in this MSGin5G UE bulk de-registration request |
| List of individual MSGin5G UE de-registration request | M | Each element in this list contains information as specified in Table 8.2.2-1. |

9.	Upon receiving the MSGin5G UE bulk de-registration request, the MSGin5G Server splits the MSGin5G UE bulk de-registration request, executes the authentication procedures for each MSGin5G Client-2 individually as specified in clause 8.2.2.

Editor's note 2:	Security aspects of bulk de-registration depend on SA3 and is FFS

10.	The MSGin5G Server includes all corresponding MSGin5G UE de-registration responses in an MSGin5G UE bulk de-registration response and sends it to MSGin5G Client-1. The Information Elements specified in table 8.2.z-4 are included in the MSGin5G UE bulk de-registration response.

Table 8.2.z-4: MSGin5G UE bulk de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Bulk de-registration response indicator | M | Indicates that this response is an MSGin5G UE bulk de-registration response. |
| Number of individual MSGin5G UE de-registration responses | M | Indicates total number of MSGin5G UE de-registration responses which are bulked in this MSGin5G UE bulk de-registration response |
| List of individual MSGin5G UE de-registration response | M | Each element in this list contains information as specified in Table 8.2.2-2 |

11.	Upon receiving the MSGin5G UE bulk de-registration response, the MSGin5G Client-1 splits the MSGin5G UE bulk de-registration response into multiple individual MSGin5G UE de-registration response, each individual MSGin5G UE de-registration response contains information as specified in Table 8.2.2-2.

12.	MSGin5G Client-1 sends the MSGin5G UE de-registration Response 2a to MSGin5G Client-2a.

13.	MSGin5G Client-1 sends the MSGin5G UE de-registration Response 2b to MSGin5G Client-2b.