3GPP TSG-SA WG6 Meeting #51-e	S6-222854

e-meeting, 10th – 19th October 2022	(revision of S6-222469)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0350 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing EN's in CSC-22 and CSC-23 interfaces |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Below EN’s are addressed. Editor's note: How the SIP-2 and SIP-3 reference points are used for signalling messages is FFS. The SIP-2, and SIP-3 reference points are required for the functionalities such as subscribing to the location information in different MC systems and sharing of location information Editor's note: Whether SIP-2 and SIP-3 reference points are used for non-subscription/notification signalling messages is FFS. The CSC-23 reference point shouldn’t be restricted to use only subscription/notification related signalling for SIP-2 and SIP-3. It should allow non-subscription/notification related signalling. Additional clarification is provided on when to use the SIP-3 reference point which was not clear. The CSC-14 reference point shouldn’t be restricted to use only subscription/notification related signalling for SIP-1 and SIP-2. It should allow non-subscription/notification related signalling. The stage-3 spec is using non-subscription related signalling for location configuration and reporting. The proposed SIP-3 reference point additional clarification is in align with the desription provided in 7.5.2.4 of 23.379. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed both EN’s - Added SIP-2 and SIP-3 reference points for CSC-22 reference point and support for non-subscription/notification related signalling. - For CSC-23 reference, added support for non-subscription/notification related signalling with additional clarification on SIP-3 reference point used in case of a location management server and an MC gateway server, are served by different SIP cores. - In 7.5.2.14, updated to support both subscription/notification and non-subscription/notification related signalling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will have no SIP reference for the CSC-22 reference point and no support for non-subscription/notification related messaging for CSC-14 and CSC-23 reference points. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.14, 7.5.2.23 and 7.5.2.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev2: Revised the document Rev1: - Removed SIP-1 reference point which was proposed to be included in this CR. - Proposal to remove the SIP-3 reference point is reverted back and provided additional clarification on when to use SIP-3 reference point. - Cover page updated accordingly. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.5.2.14	Reference point CSC-14 (between the location management client and the location management server)

The CSC-14 reference point, which exists between the location management client and the location management server, is used by the location management server to receive location information report from location management client.

The CSC-14 reference point uses SIP-1 and SIP-2 reference points for transport and routing of (non-)subscription/notification related signalling. The CSC-14 reference point uses the HTTP-1 and HTTP-2 reference points for transport and routing of non-subscription/notification related signalling.

* * * Next Change * * * *

#### 7.5.2.23	Reference point CSC-22 (between location management servers in different MC systems)

The CSC-22 reference point, which exists between location management servers in different MC systems, is used to share location related information corresponding to the MC service usersbetween interconnected MC systems.

The CSC-22 reference point uses the HTTP-1, HTTP-2 and HTTP-3 reference points for transport and routing of non-subscription/notification related signalling. The CSC-22 reference point uses SIP-2 and SIP-3 reference points for transport and routing of (non-)subscription/notification related signalling.

#### 7.5.2.24	Reference point CSC-23 (between location management server and MC gateway server)

The CSC-23 reference point, which exists between location management server and MC gateway server, is used to handle location management related signalling.

The CSC-23 reference point uses SIP-2 and SIP-3 reference points for transport and routing of (non-)subscription/notification related signalling. The SIP-3 reference point is used when a location management server and an MC gateway server, are served by different SIP cores.

* * * End of Changes * * * *