3GPP TSG-SA6 Meeting #51-e	S6-222878

# Online, , 10th Oct 2022 - 19th Oct 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0131 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to architectural assumptions for EAS Service APIs enablement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, Uangel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-10-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In FS_eEDGEAPP study, the enablement of Service APIs exposed by EAS was studied as Key Issue #2 in TR 23.700-98; and concluded to support for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs within the EDGEAPP architecture by exploiting CAPIF as specified in clause 7.8 of TR 23.700-98. The details are specified in TR 23.700-98 as follows: - architectural requirements for EAS Service APIs as specified in the clause 5.2 of TR 23.700-98 - architectural assumptions, operations, and information flows as specified in the clause 7.8 of TR 23.700-98 - new service KPI IE for EAS Service API as specified in the clause 7.8.2.3 of TR 23.700-98 |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to support for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs within the EDGEAPP architecture, the following changes are proposed: - a new paragraph is added to clause 4.5 for details of the enablement of EAS Service APIs - a new architectural requirement is added to clause 5.2.5 - exposing EAS Service APIs is added as an optional functionality of EAS in clause 6.3.6 - a new note is added in clause 6.7.1 to include the exposure capability of EAS Service APIs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Exposing EAS Service APIs within the EDGEAPP architecture is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5, 5.2.5, 6.3.6, 6.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE

4.5	Capability exposure to EAS

The edge enabler layer exposes services towards the EASs. The exposed capabilities include the services of the Edge Enabler Layer and the re-exposed and enhanced services of the 3GPP core network. The capabilities exposed by the edge enabler layer are specified in clause 8.6 and the 3GPP network capability exposure is specified in clause 8.7. Other application layer capabilities like application enabler services and SEAL services may be exposed via edge enabler layer as per CAPIF as illustrated in Annex A.4.

The edge enabler layer also supports for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs via CAPIF as specified in 3GPP TS 23.222 [6].

2nd CHANGE

5.2.5	Capability exposure to EASs

5.2.5.1	General

This clause specifies the requirements for capability exposure to EAS.

5.2.5.2	Requirements

[AR-5.2.5.2-a]	The application layer architecture shall support exposure of 3GPP network's capabilities to the EASs.

[AR-5.2.5.2-b]	The application layer architecture shall support exposure of EES's capabilities to the EASs.

[AR-5.2.5.2-c]	The application layer architecture shall support exposure of EAS's capabilities to the other EASs.

3rd CHANGE

(VOID)

4th CHANGE

6.3.6	Edge Application Server (EAS)

EAS is the application server resident in the EDN, performing the server functions. The AC connects to the EAS in order to avail the services of the application with the benefits of Edge Computing.

It is possible that the server functions of an application are available only as an EAS.

However, it is also possible that certain server functions are available both at the edge and in the cloud, as an EAS and an Application Server resident in the cloud respectively. The server functions offered by an EAS and its cloud Application Server counterpart may be the same or may differ; if they differ, the Application Data Traffic exchanged with the AC may also be different.

The EAS can consume the 3GPP Core Network capabilities in the following ways, all of which are optional to support:

a)	invoking 3GPP Core Network capabilities via the edge enabler layer through the EES

b)	invoking 3GPP Core Network function (e.g. PCF) APIs directly, if it is an entity trusted by the 3GPP Core Network; and

c)	invoking the 3GPP Core Network capabilities through the capability exposure functions i.e. SCEF/NEF/SCEF+NEF.

The EAS can support processing ACR related operations (e.g. ACR status update, selected target EAS declaration).

The EAS can expose its Service API(s) (i.e., EAS Service API(s)) towards the other EASs by supporting API provider domain functions as specified in TS 23.222 [6]; and the EAS can consume the EAS Service API(s) exposed by the other EAS(s) by supporting API invoker functionalities as specified in TS 23.222 [6].

5th CHANGE

6.7.1	General

The Figure 6.7.1-1 shows the capability exposure for enabling edge applications.

Figure 6.7.1-1: Capability exposure for enabling edge applications

Capability exposure includes the 3GPP core network (i.e. 5GC, EPC), ECS and the EES capability exposure, to fulfil the needs of the edge service operations. The capability exposure functionality is utilized by the functional entities (i.e. EES, EAS and ECS) depicted in the architecture for enabling the edge applications.

NOTE: 	The edge enabling layer also supports the exposure of EAS Service APIs using CAPIF, which is not explicitly depicted in the Figure 6.7.1-1.

END OF CHANGES