3GPP TSG-SA WG6 Meeting #51-e	S6-222924

e-meeting, 10th – 19th October 2022	(revision of S6-222851)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0093 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Added additional subclause for switching from MBS session to unicast bearer for MCPTT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current specification, a generic procedure for switching and dedicated switching procedures are defined for the MCData and MCVideo. Dedicated clause is missing for describing the procedure for switching from MBS session to unicast bearer or eMBMS bearer for MCPTT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Dedicated clause is added for describing the procedure for switching from MBS session to unicast bearer or eMBMS bearer for MCPTT. In 7.3.3.11.3 and 7.3.3.12.3: The title and part of description is updated to cover the switching is not one one but is vice versa and across the access technology (LTE and 5G) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A dedicated procedure for MCPTT will not be available for switching from MBS session to unicast bearer or eMBMS bearer |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.10.5 (new), 7.3.3.11.3 and 7.3.3.12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Added In 7.3.3.11.3 and 7.3.3.12.3: The title and part of description is updated to cover the switching is not oneway only but is vice versa and across the access technologies (LTE and 5G) |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 7.3.3.10.5 	Switching between 5G MBS session, 5G unicast PDU session, LTE eMBMS bearer, and LTE unicast bearer for MCPTT

The MCPTT service shall support the procedure for switching between 5G MBS session, 5G unicast PDU session, LTE eMBMS bearer and LTE unicast bearer as specified in clause 7.3.3.8 and clause 7.3.3.9 with the following clarifications:

-	The MC service client is the MCPTT client;

-	The MC service server is the MCPTT server; and

-	The MC service ID is the MCPTT ID.

The MCPTT service shall use the MCPTT-1, MCPTT-4, MCPTT-7, MCPTT-8 and MCPTT-9 reference points for this procedure.

* * * Next Change * * * *

##### 7.3.3.11.3		Switching between 5G MBS session, 5G unicast PDU session, LTE eMBMS bearer, and LTE unicast bearer for MCVideo

The MCVideo service shall support the procedure for switching between 5G MBS session, 5G unicast PDU session, LTE eMBMS bearer and LTE unicast bearer as specified in clause 7.3.3.8 and clause 7.3.3.9 with the following clarifications:

-	The MC service client is the MCVideo client;

-	The MC service server is the MCVideo server; and

-	The MC service ID is the MCVideo ID.

The MCVideo service shall use the MCVideo-1, MCVideo-4, MCVideo-7, MCVideo-8 and MCVideo-9 reference points for this procedure.

* * * Next Change * * * *

##### 7.3.3.12.3 	Switching between 5G MBS session, 5G unicast PDU session, LTE eMBMS bearer, and LTE unicast bearer for MCData

The MCData service shall support the procedure for switching between 5G MBS session, 5G unicast PDU session, LTE eMBMS bearer, and LTE unicast bearer as specified in clause 7.3.3.8 and clause 7.3.3.9 with the following clarifications:

-	The MC service client is the MCData client;

-	The MC service server is the MCData server; and

-	The MC service ID is the MCData ID.

The MCData service shall use the MCData-SDS-1, MCData-SDS-2, MCData-FD-1, MCData-FD-3, MCData-DS-1 and MCData-DS-3 reference points for this procedure.

* * * End of Changes * * * *