3GPP TSG-SA WG6 Meeting #51-e	S6-222926

e-meeting, 10th – 19th October 2022	(revision of S6-222695 rev1)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0071 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | bulk de-registration of Non-MSGin5G UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR proposes bulk de-registration procedure of Non-MSGin5G UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | bulk de-registration procedure of Non-MSGin5G UEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | bulk de-registration procedure of Non-MSGin5G UEs is not supported by MSGin5G service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: modify some description of the steps. |  |  |  |  |  |  |  |  |

*****************Change 1************************

8.2.Y	Non-MSGin5G UE bulk de-registration

The procedure assumes that the Message Gateway is responsible for initiating the bulk de-registration from the MSGin5G Server on behalf of the Non-MSGin5G UEs. The signaling flow for Non-MSGin5G UE bulk de-registration is illustrated in figure 8.2.y-1.

Pre-conditions:

1.	The Message Gateway successfully performed registration with the MSGin5G Server on behalf of a set of Non-MSGin5G UEs.

Figure 8.2.y-1: Non-MSGin5G UE bulk de-registration

1.	The Message Gateway determines to perform Non-MSGin5G UE bulk de-registration.

NOTE:	The conditions of whether the Non-MSGin5G UEs can be bulked de-registered are implementation specific and out of the scope of the current specification.

2.	The Message Gateway sends a Non-MSGin5G UE bulk de-registration request to the MSGin5G Server. The Information Elements specified in table 8.2.y-1 are included in the Non-MSGin5G UE bulk de-registration request.

Table 8.2.y-1: Non-MSGin5G UE bulk de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE de-registration requests | M | Indicates total number of Non-MSGin5G UE de-registration requests which are bulked in this Non-MSGin5G UE bulk de-registration request |
| List of individual UE Service ID | M | Each element in this list contains a UE service identifier assigned to the Non-MSGin5G UE. |

3.	Upon receiving the request, the MSGin5G Server processes each of the UE registration from the list individually as specified in clause 8.2.4. The MSGin5G Server deletes any applicable UE Service ID in the List of individual UE Service ID and associated MSGin5G Client Profile information that it has stored.

Editor's note:	Security aspect of Non-MSGin5G UE bulk de-registration is the responsibility of SA3.

4.	The MSGin5G Server replies with a Non-MSGin5G UE bulk de-registration response as shown in table 8.2.y-2.

Table 8.2.y-2: Non-MSGin5G UE bulk registration response

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual Non-MSGin5G UE de-registration responses | M | Indicates total number of Non-MSGin5G UE de-registration responses which are bulked in this Non-MSGin5G UE bulk de-registration response |
| List of individual Non-MSGin5G UE de-registration response | M | Each element in this list contains information as specified in Table 8.2.4-2. |