3GPP TSG-SA WG6 Meeting #51-e	S6-222962

e-meeting, 10th – 19th October 2022	(revision of S6-222724)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0129 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support simu-EAS connectivity in ACR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-09-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on KI#21 conclusion, sol#22 adds impact in ACR request procedure triggered by EEC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add simu-EAS connectivity info in ACR request and AC profile. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2, 8.8.3.4, 8.8.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.2.2	AC Profile

An AC Profile includes information about AC used to determine services and service characteristics required.

Table 8.2.2-1: AC Profile

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | Identity of the AC. |
| AC Type | O | The category or type of AC (e.g. V2X). This is an implementation specific value. |
| Preferred ECSP list | O | When used in a service provisioning request, this IE indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |
| AC Schedule | O | The expected operation schedule of the AC (e.g. time windows) |
| Expected AC Geographical Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| AC Service Continuity Support | O | Indicates if service continuity support is required or not for the application. The IE also indicates which ACR scenarios are supported by the AC and which of these are preferred by the AC. |
| Simultaneous EAS connectivity information in service continuity | O | Indicates if simultaneous EAS connectivity is needed and the inactive time guidance for keeping connectivity towards the S-EAS. |
| List of EASs | O | List of EAS that serve the AC along with the service KPIs required by the AC |
| > EASID | M | Identifier of the EAS |
| > Expected AC Service KPIs | O | KPIs expected in order for ACs to receive currently required services from the EAS, as described in Table 8.2.3-1 |
| > Minimum required AC Service KPIs | O | Minimum KPIs required in order for ACs to receive meaningful services from the EAS, as described in Table 8.2.3-1 |

* * * Next Change * * * *

#### 8.8.3.4	ACR launching procedure

Figure 8.8.3.4-1 illustrates the ACR launching procedure by the EEC or the S-EAS.

If this procedure is triggered by the EEC, depending on the ACR action indicated in the ACR request, the procedure is used for either ACR initiation or ACR determination. This procedure of the ACR initiation can be re-sent as described in clause 8.8.1.3.

If this procedure is triggered by the S-EAS, the procedure is used for ACR determination.

Pre-condition:

1.	The EEC has been authorized to communicate with the EES as specified in clause 8.11, if the procedure is triggered by the EEC; and

2.	Information related to the S-EES is available with the S-EAS, if the procedure is triggered by the S-EAS.

Figure 8.8.3.4-1: ACR launching procedure

1.	The EEC or the S-EAS sends an ACR request message to the EES in order to start ACR. The ACR request message includes ACR action to indicate either ACR initiation request or ACR determination request. If the procedure is triggered by the S-EAS, the ACR request message is only for ACR determination.

An ACR request for ACR initiation sent by the EEC:

-	includes an indication of whether the EEC requests the EES to perform EAS notification; and

-	provides information used by EES to perform AF traffic influence as in 3GPP TS 23 501 [2].

The ACR request for ACR initiation may include the simultaneous EAS connectivity information in service continuity (see table 8.8.4.4-1) which was previously received as part of the AC profile.

An ACR request for ACR determination sent either by the EEC or the EAS informs the EES that the need for ACR has been detected by the requestor.

2.	The EES checks if the requestor is authorized for this operation. If authorized, the EES processes the request and performs the required operations.

If the request in step 1 is for ACR initiation:

-	the EES may use information provided in the request to apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable), as described in 3GPP TS 23.501 [2], clause 5.6.7.1; and

NOTE:	The simultanenous EAS connectivity information sent by EES is used to maintain both S-PSA and T-PSA in supporting simultaneous connectivity with both S-EAS and T-EAS during the service continuity as described in clause 6.3.4 of 3GPP TS 23.548 [20].

Editor's note:       Since the 3GPP CN only supports simultaneous PSA connectivity in SSC mode 3 or session breakout, it is FFS whether EES should firstly know PDU session capability before invoking AF traffic influence API.

-	if the EAS notification indication in ACR initiation data is provided in the step 1 request and the EAS has subscribed to receive such notification, the EES shall notify the EAS indicated in the ACR initiation data about the need to start ACR by sending an ACR management notification for the "ACT start" event, as described in clause 8.6.3.

If the request in step 1 is for ACR determination, the EES decides to execute ACR as described in clause 8.8.2.5.

If the request in step 1 includes Previous T-EAS Endpoint:

-	if the previous EAS notification indication is provided in the step 1 request and the EAS has subscribed to receive such notification, the EES shall notify the EAS about the cancellation of the ACR with the previous T-EAS by sending an ACR management notification for the "ACT stop" event, as described in clause 8.6.3.

-	The EAS will inform the remote EAS about application context cancellation, which is outside the scope of this specification. The T-EAS sends the ACR status update message to the T-EES which will include failed result with an appropriate cause indicating the reason for the failure.

3.	The EES responds to the requestor's request with an ACR response message.

In case of re-sending ACR initiation, if serving EES was changed and EEC context was relocated, the T-EES can clean up any relocated EEC context either indicated in the re-sent ACR request for scenario described in clause 8.8.2.6 or upon reception of the ACR status update with failed result from T-EAS for other scenarios.

* * * Next Change * * * *

#### 8.8.4.4	ACR request

Table 8.8.4.4-1 describes information elements for the ACR request sent either from the EEC to the S-EES or T-EES, or by the S-EAS to the S-EES.

Table 8.8.4.4-1: ACR request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. EECID or EASID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EASID | O | Identifier of the EAS |
| UE identifier | M | The identifier of the UE (i.e. GPSI). |
| ACID | O | The identifier of the AC. |
| ACR action (NOTE 3) | M | Indicates the ACR action (ACR initiation or ACR determination) |
| ACR initiation data (NOTE 2) | O | ACR initiation IEs to be included in an ACR request message when ACR action indicates it is ACR initiation request. |
| > T-EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS. |
| > Previous T-EAS Endpoint (NOTE 7) | O | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS of the previous ACR. |
| > DNAI of the T-EAS | O | DNAI information associated with the T-EAS. |
| > N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to the T-EAS DNAI. |
| > Simultaneous EAS connectivity information | O | Indicates if simultaneous EAS connectivity is needed and the inactive time guidance for keeping connectivity towards the S-EAS. |
| > EAS notification indication | M | Indicates whether to notify the EAS about the need of ACR. |
| > Previous EAS notification indication (NOTE 7) | O | Indicates whether to notify the EAS about the cancellation of a previous ACR. |
| > S-EAS endpoint (NOTE 1) | O | Endpoint information of the S-EAS |
| > EEC context relocation details | O | Information required for EEC context relocation using the EEC context push or EEC context pull mechanisms. |
| >> EEC Context ID (NOTE 5) | O | Identifier of the EEC Context |
| >> S-EES ID (NOTE 5) | O | Identifier of the EES that provided EEC context ID. |
| >> S-EES endpoint (NOTE 5) | O | The endpoint address (e.g. URI, IP address) of the EES that provided EEC context ID. |
| >> T-EES ID (NOTE 6) | O | Identifier of the T-EES. |
| >> T-EES endpoint (NOTE 6) | O | The endpoint address (e.g. URI, IP address) of the T-EES. |
| ACR determination data (NOTE 2) | O | ACR determination IEs to be included in an ACR request message when ACR action indicates it is ACR determination request. |
| > S-EAS endpoint | M | Endpoint information of the S-EAS |
| NOTE 1: This IE shall be present if the EAS notification indication or previous EAS notification indication indicates that the EAS needs to be informed. NOTE 2: Either ACR initiation or ACR determination shall be included corresponding to the ACR action. NOTE 3: This IE shall indicate ACR determination if the request originates from the S-EAS. NOTE 4: Void. NOTE 5: This IE may be present only if the request originates from the EEC towards the T-EES. NOTE 6: This IE may be present only if the request originates from the EEC towards the S-EES. NOTE 7: These IEs shall be present when the EEC re-sends the ACR request as described in clause 8.8.1.3 to indicate a previous ACR is to be cancelled. |  |  |

* * * End of Changes * * * *