3GPP TSG-SA WG6 Meeting #51-e	S6-222992

e-meeting, 10th – 19th October 2022	(revision of S6-222800)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0134 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EES monitoring the UE mobility for service continuity planning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the case where the EAS cannot monitoring the UE mobility or EAS does not want to monitor the UE mobility, the EES can perform the UE mobility monitoring for the service continuity planning case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The EES identify the ACR type when EES detect ACR event, if ACT is complete, the EAS will notify the result of ACT immediately to the EES, once EES detects that UE moves to the predicted/expected location, the EES will notify EEC ACR complete, then the EEC can comfirm the compeletion of ACR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Solution for UE monitoring the UE mobility for service continuity planning will remain under specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2.5, 8.8.2.3, 8.8.3.4, 8.8.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.8.2.5	S-EES executed ACR

Figure 8.8.2.5-1 illustrates the S-EES detecting, deciding and executing ACR from the S-EAS to the T-EAS. This may include EELManagedACR by S-EES when initiated by S-EAS as per clause 8.8.3.6.

Pre-condition:

1.	The AC at the UE already has a connection to the S-EAS;

2.	The EEC is able to communicate with the S-EES;

3.	The EEC has subscribed to receive ACR information notifications for target information notification events and ACR complete events from the S-EES, as described in clause 8.8.3.5.2;

4.	The S-EAS optionally subscribed to receive ACR management notifications for "ACR facilitation" events to the S-EES, in order to enable detection at S-EAS.

5.	In case of EELManagedACR, the T-EAS has subscribed to receive ACT status notifications as described in clause 8.8.3.6.2.3.

6. The EES will monitor the UE mobitliy to Predicted/Expected UE location.

Editor's Note:	how the EES can determine which entity (EAS or EES) will perform the detection of UE mobility to the predicted/expected location is FFS.

Figure 8.8.2.5-1: S-EES executed ACR

1.	The S-EAS may initiate EELManagedACR with S-EES as specified in clause 8.8.3.6. In this step, the S-EAS and S-EES negotiate an address of the Application Context storage to S-EES. The S-EAS puts the Application Context at this address which can be further accessed by the S-EES when the ACT is required.

In this case, the S-EES executes steps 2 (i.e., S-EES detection), 4, 5, 6, 7, 8, 9, 10, 11, 13 and 14. Rest of steps are skipped.

Phase I: ACR Detection

2.	Detection entities (S-EAS, S-EES, EEC) detect that ACR may be required and identify the ACID and Predicted/Expected UE location or EAS service area as described in clause 8.8.1.1. The detection by the S-EES may be triggered by the User Plane path change notification received from the 3GPP Core Network due to S-EAS request for "ACR facilitation" event (see clause 8.6.3) or due to step 1.

The detection entity may detect that ACR may be required for an expected or predicted UE location in the future as described in clause 8.8.1.1.

Phase II: ACR Decision

3.	The detection entity performs ACR launching procedure (as described in clause 8.8.3.4) with the ACR action indicating ACR determination and the corresponding ACR determination data. If the EEC or S-EAS detect the ACR event, the EEC or S-EAS may inform S-EES with ACID, and predicted/expected UE location or EAS service area in the ACR launching procedure.

4.	The S-EES authorises the message if received. The S-EES decides to execute ACR based on the information received or local detection, and the information of EEC context or EAS profile, and then proceed the below steps.

Phase III:	ACR Execution

5.	The S-EES determines T-EES and T-EAS via the Discover T-EAS procedure in clause 8.8.3.2 of the present document. When in step 2 the ACR has been triggered for service continuity planning, then UE Location and Target DNAI values provided in the Retrieve T-EES procedure contain the expected UE Location and expected Target DNAI. The S-EES may decide not to perform ACR if T-EAS is not available.

6.	If the T-EES is different than the S-EES and the EEC Context at the S-EES is not stale, the S-EES initiates EEC Context Push relocation with the T-EES as described in clause 8.9.2.3. Otherwise, if the T-EES is the same as the S-EES, EEC Context Push relocation is skipped.

7.	The S-EES sends the target information notification to the EEC as described in clause 8.8.3.5.3.

8.	The S-EES may apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable).

9.	The S-EES sends the ACR management notification (e.g. as notification for "ACR facilitation" event or "ACT start" event as described in clause 8.6.3 or due to step 1) to the S-EAS to initiate ACT between the S-EAS and the T-EAS.

10.	The Application Context is transferred from S-EAS to the T-EAS at implementation specific time. In the case of EELManagedACR, the S-EES accesses the Application Context from the address as per step 1 and the S-EES and T-EES engage in the ACT from S-EAS to the T-EAS (obtained as per step 5) in a secure way. Further the T-EAS accesses the Application Context made available by the T-EES. If S-EAS performs the ACT directly with T-EAS, the specification of such process is out of scope of the present document.

NOTE 1:	The Application Context is encrypted and protected by the application layer. The S-EES and the T-EES engage in the packet level transport of the Application Context and they have no visibility to the content of the Application Context.

When in step 2 the ACR has been triggered for service continuity planning, if the UE does not move to the predicted location, the EEC does not connect to T-EES, the AC does not connect to the T-EAS. Post-ACR Clean up is skipped.

NOTE 2:	The S-EAS or T-EAS can further decide to terminate the ACR, and the T-EAS can discard the application context based on information received from EEL and/or other methods (e.g. monitoring the location of the UE). It is up to the implementation of the S-EAS and T-EAS whether and how to make such a decision.

NOTE 3:	When in step 2 the ACR has been triggered for service continuity planning, Post-ACR Clean up is performed after the UE moves to the expected location.

Phase IV:	Post-ACR Clean up

11.	In case of EELManagedACR, once the ACT is successful, the T-EES sends an ACT status notification to the T-EAS as described in clause 8.8.3.6.2.4, indicating that the Application Context is available.

12.	The S-EAS sends the ACT status update message to the S-EES as specified in clause 8.8.3.8 when S-EAS detects application context transmission is complete.

13.	The T-EAS sends the ACR status update message to the T-EES as specified in clause 8.8.3.8. If the status indicates a successful ACT, and that the EEC Context relocation procedure was attempted but failed, then the T-EES indicates the failure to the T-EAS with the ACR status update response.

NOTE 4:	If the EDGE-3 subscription initialization result indicates failure, then the EAS can perform the required EDGE-3 subscriptions at the T-EES.

NOTE 5:	Steps 12 and 13 can occur in any order.

14.	If the status in step 12 indicates a successful ACT, the S-EES sends the ACR information notification (ACR complete) message to the EEC to confirm that the ACR has completed as specified in clause 8.8.3.5.3. Once S-EES detects the UE has moved to the predicted/expected UE location or EAS service area and the status in step 12 indicates a successful ACT, then the S-EES sends ACR complete notify message to the EEC when the ACR type is service continuity planning.  If the EEC Context relocation procedure was attempted, then the notification includes EEC context relocation status IE, indicating the result of the EEC context relocation procedure. If the EEC context relocation status indicates that the EEC context relocation was not successful, then the EEC may perform the required EDGE-1 operations such as create subscriptions at the T-EES.

NOTE 6:	The Application Client mechanism to support switchover of the application traffic to T-EAS is out of scope of the specification.

* * * NEXT Change * * * *

#### 8.8.2.3	EEC executed ACR via S-EES

In this scenario, the EEC is triggered as a result of the UE's movement as described in 8.8.1.1. Figure 8.8.2.3-1 illustrates the EEC executing ACR via the S-EES.

Pre-condition:

1.	The AC at the UE already has a connection to the S-EAS; and

2.	The EEC is able to communicate with the S-EES.

3. The EES will monitor the UE mobitliy to Predicted/Expected UE location.

Editor's Note:	how the EES can determine which entity (EAS or EES) will perform the detection of UE mobility to the predicted/expected location is FFS.

Figure 8.8.2.3-1: EEC executed ACR

Phase I: ACR Detection

1.	The EEC detects that ACR may be required as described in clause 8.8.1.1. The EEC may detect that ACR may be required for an expected or predicted UE location in the future as described in clause 8.8.1.1.

Phase II: ACR Decision

2.	The EEC decides to proceed required procedures for triggering ACR.

Phase III:	ACR Execution

3.	The EEC determines the T-EES by using the provisioned information or performing service provisioning procedure per clause 8.3 of the present document. When in step 1 the ACR for service continuity planning is triggered, then the Connectivity information and UE Location in the Service Provisioning (as specified in clause 8.3) procedure contains the expected Connectivity information and expected UE Location. If the UE is within the service area of the T-EES, upon selecting T-EES the UE may need to establish a new PDU connection to the target EDN. If EEC registration configuration for the T-EES indicates that EEC registration is required, the EEC performs EEC registration with the selected T-EES as specified in clause 8.4.2.2.2. The EEC can then discover and select T-EAS by performing EAS Discovery with the T-EES per clause 8.5.2 of the present document.

NOTE 1:	Several EEC registrations with different EESs may result from T-EAS discovery process during a single ACR operation.

4.	The EEC performs ACR launching procedure (as described in clause 8.8.3.4) to the S-EES with indicating S-EES with predicted/expected UE location or EAS service area, the ACR action indicating ACR initiation and the corresponding ACR initiation data (with the need to notify the EAS). The S-EES authorises the request from the EEC. The S-EES decides to execute ACR based on the information received from the EEC, EEC context and/or EAS profile. The S-EES may apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable) and sends the ACR management notification for the "ACT start" event to the S-EAS, as described in clause 8.6.3, to initiate ACT between the S-EAS and the T-EAS. If the EEC has not subscribed to receive ACR information notifications for ACR complete events from the S-EES, the EEC subscribes for the notifications as described in clause 8.8.3.5.2.

5.	If the T-EES is different than the S-EES and the EEC Context at the S-EES is not stale, the S-EES initiates EEC Context Push relocation with the T-EES as described in clause 8.9.2.3. Otherwise, if the T-EES is the same as the S-EES, EEC Context Push relocation is skipped.

6.	The S-EAS transfers the application context to the T-EAS at implementation specific time. This process is out of scope of the present specification.

When in step 1 the ACR has been triggered for service continuity planning, if the UE does not move to the predicted location, the EEC does not connect to T-EES, the AC does not connect to the T-EAS. Post-ACR Clean up is skipped.

NOTE 2:	The S-EAS or T-EAS can further decide to terminate the ACR, and the T-EAS can discard the application context based on information received from EEL and/or other methods (e.g. monitoring the location of the UE). It is up to the implementation of the S-EAS and T-EAS whether and how to make such a decision.

NOTE 3:	When in step 1 the ACR for service continuity planning is triggered, Post-ACR Clean up is performed after the UE moves to the predicted location.

Phase IV:	Post-ACR Clean up

7. When S-EAS detects application context transmission is complete, the S-EAS sends the ACR status update message to the S-EES as specified in clause 8.8.3.8.

8.	The T-EAS sends the ACR status update message to the T-EES as specified in clause 8.8.3.8. If the status indicates a successful ACT, and that the EEC Context relocation procedure was attempted but failed, then the T-EES indicates the failure to the T-EAS with the ACR status update response.

NOTE 4:	If the EDGE-3 subscription initialization result indicates failure, then the EAS can perform the required EDGE-3 subscriptions at the T-EES.

NOTE 5:	Steps 7 and 8 can occur in any order.

9. If the status in step 7 indicates a successful ACT, the S-EES sends the ACR information notification (ACR complete) message to the EEC to confirm that the ACR has completed as specified in clause 8.8.3.5.3. Once S-EES detects the UE has moved to the predicted/expected UE location or EAS service area, then the S-EES sends ACR complete notify message to the EEC indicating that UE has moved to the predicted location when the ACR type is service continuity planning. If the EEC Context relocation procedure was attempted, then the notification includes EEC context relocation status IE, indicating the result of the EEC context relocation procedure. If the EEC context relocation status indicates that the EEC context relocation was not successful, then the EEC may perform the required EDGE-1 operations such as create subscriptions at the T-EES.

* * * NEXT Change * * * *

#### 8.8.3.4	ACR launching procedure

Figure 8.8.3.4-1 illustrates the ACR launching procedure by the EEC or the S-EAS.

If this procedure is triggered by the EEC, depending on the ACR action indicated in the ACR request, the procedure is used for either ACR initiation or ACR determination. This procedure of the ACR initiation can be re-sent as described in clause 8.8.1.3.

If this procedure is triggered by the S-EAS, the procedure is used for ACR determination.

Pre-condition:

1.	The EEC has been authorized to communicate with the EES as specified in clause 8.11, if the procedure is triggered by the EEC; and

2.	Information related to the S-EES is available with the S-EAS, if the procedure is triggered by the S-EAS.

Figure 8.8.3.4-1: ACR launching procedure

1.	The EEC or the S-EAS sends an ACR request message to the EES in order to start ACR. The ACR request message includes Predicted/Expected UE location or EAS service area to indicate that the EES should detect whether the UE has moves to the Predicted/Expected UE location or EAS service area or not in ACR clean-up phase. The ACR request message includes ACR action to indicate either ACR initiation request or ACR determination request. If the procedure is triggered by the S-EAS, the ACR request message is only for ACR determination.

An ACR request for ACR initiation sent by the EEC:

-	includes an indication of whether the EEC requests the EES to perform EAS notification; and

-	provides information used by EES to perform AF traffic influence as in 3GPP TS 23 501 [2].

An ACR request for ACR determination sent either by the EEC or the EAS informs the EES that the need for ACR has been detected by the requestor.

2.	The EES checks if the requestor is authorized for this operation. If authorized, the EES processes the request and performs the required operations.

If the request in step 1 is for ACR initiation:

-	the EES may use information provided in the request to apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable), as described in 3GPP TS 23.501 [2], clause 5.6.7.1; and

-	if the EAS notification indication in ACR initiation data is provided in the step 1 request and the EAS has subscribed to receive such notification, the EES shall notify the EAS indicated in the ACR initiation data about the need to start ACR by sending an ACR management notification for the "ACT start" event, as described in clause 8.6.3.

If the request in step 1 is for ACR determination, the EES decides to execute ACR as described in clause 8.8.2.5.

If the request in step 1 includes Previous T-EAS Endpoint:

-	if the previous EAS notification indication is provided in the step 1 request and the EAS has subscribed to receive such notification, the EES shall notify the EAS about the cancellation of the ACR with the previous T-EAS by sending an ACR management notification for the "ACT stop" event, as described in clause 8.6.3.

-	The EAS will inform the remote EAS about application context cancellation, which is outside the scope of this specification. The T-EAS sends the ACR status update message to the T-EES which will include failed result with an appropriate cause indicating the reason for the failure.

3.	The EES responds to the requestor's request with an ACR response message.

In case of re-sending ACR initiation, if serving EES was changed and EEC context was relocated, the T-EES can clean up any relocated EEC context either indicated in the re-sent ACR request for scenario described in clause 8.8.2.6 or upon reception of the ACR status update with failed result from T-EAS for other scenarios.

* * * NEXT Change * * * *

#### 8.8.4.4	ACR request

Table 8.8.4.4-1 describes information elements for the ACR request sent either from the EEC to the S-EES or T-EES, or by the S-EAS to the S-EES.

Table 8.8.4.4-1: ACR request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. EECID or EASID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EASID | O | Identifier of the EAS |
| UE identifier | M | The identifier of the UE (i.e. GPSI). |
| Predicted/Expected UE location or EAS service area (NOTE8) | O | The predicted/expected location information of the UE. The UE location is described in clause 7.3.2 or the predicted/expected EAS service area as described in clause 7.3.3.3 |
| ACID | O | The identifier of the AC. |
| ACR action (NOTE 3) | M | Indicates the ACR action (ACR initiation or ACR determination) |
| ACR initiation data (NOTE 2) | O | ACR initiation IEs to be included in an ACR request message when ACR action indicates it is ACR initiation request. |
| > T-EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS. |
| > Previous T-EAS Endpoint (NOTE 7) | O | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS of the previous ACR. |
| > DNAI of the T-EAS | O | DNAI information associated with the T-EAS. |
| > N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to the T-EAS DNAI. |
| > EAS notification indication | M | Indicates whether to notify the EAS about the need of ACR. |
| > Previous EAS notification indication (NOTE 7) | O | Indicates whether to notify the EAS about the cancellation of a previous ACR. |
| > S-EAS endpoint (NOTE 1) | O | Endpoint information of the S-EAS |
| > EEC context relocation details | O | Information required for EEC context relocation using the EEC context push or EEC context pull mechanisms. |
| >> EEC Context ID (NOTE 5) | O | Identifier of the EEC Context |
| >> S-EES ID (NOTE 5) | O | Identifier of the EES that provided EEC context ID. |
| >> S-EES endpoint (NOTE 5) | O | The endpoint address (e.g. URI, IP address) of the EES that provided EEC context ID. |
| >> T-EES ID (NOTE 6) | O | Identifier of the T-EES. |
| >> T-EES endpoint (NOTE 6) | O | The endpoint address (e.g. URI, IP address) of the T-EES. |
| ACR determination data (NOTE 2) | O | ACR determination IEs to be included in an ACR request message when ACR action indicates it is ACR determination request. |
| > S-EAS endpoint | M | Endpoint information of the S-EAS |
| NOTE 1: This IE shall be present if the EAS notification indication or previous EAS notification indication indicates that the EAS needs to be informed. NOTE 2: Either ACR initiation or ACR determination shall be included corresponding to the ACR action. NOTE 3: This IE shall indicate ACR determination if the request originates from the S-EAS. NOTE 4: Void. NOTE 5: This IE may be present only if the request originates from the EEC towards the T-EES. NOTE 6: This IE may be present only if the request originates from the EEC towards the S-EES. NOTE 7: These IEs shall be present when the EEC re-sends the ACR request as described in clause 8.8.1.3 to indicate a previous ACR is to be cancelled. NOTE 8: This IE shall be present if the ACR procedure is for service continuity planning. |  |  |

* * * END of Change * * * *